APT REPORT

on

COMMON FREQUENCY BANDS FOR OPERATION OF SHORT RANGE RADIOCOMMUNICATION DEVICES

No. APT/AWF/REP-07 (Rev.1)

Edition: September 2009

Adopted by

The 5th APT Wireless Forum Meeting

26 – 29 August 2008

Macao, China

and

Revised at

The 7th APT Wireless Forum Meeting

23 – 26 September 2009

Phuket, Thailand

|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The APT Wireless Forum |  | Report APT/AWF/REP-07 (Rev.1) |  |
|  |  |  | # September 2009 |  |

Source: AWF-7/OUT-20, APT/AWF/REP-07

Revision to the APT Report on

COMMON FREQUENCY BANDS FOR OPERATION OF SHORT RANGE RADIOCOMMUNICATION DEVICES

I. Introduction

The Sub-Working Group 5 (Sub-WG5) at the 4th AWF meeting had proposed to develop recommendations on common frequency bands below 1 GHz for operation of low power devices. The output document AWF-4/OUT-09 containing the proposal was confirmed at the same meeting as part of its future work item. Also, the Administrations considered that it is necessary to share the regulatory information regarding operation of SRD among APT member countries as early as possible, in order to facilitate the market access by equipment vendors.

To better understand the frequency bands that have been opened up for SRD operations, the SWG5 has undertaken a survey on the technical and procedural regulations of Short Range Devices (SRD) on 19 March 2008. The objective of the survey on the technical regulations is to determine, among other technical requirements, the operating specifications of SRD across different types of applications. For procedural regulations, the key focus is on the type approval process, MRA arrangement, licensing requirements, operating parameters as well as future policies. Details of the survey questionnaire can be found in Annex 1.

At the closing date of submission, the Administrations from Brunei Darussalam, Hong Kong, Japan, Korea, Malaysia, Philippines, New Zealand and Singapore responded to the survey. Technical regulations stipulated in respective member countries are appended in Annex 2. As stated in the circular to APT members, the ITU Radio Assembly has approved Resolution ITU-R54 on the ‘Studies to achieve harmonization for short-range radiocommunication devices (SRDs)’ in October 2007, Geneva. The survey compilation of this document may be helpful for APT member countries in preparing their national contributions to the ITU-R Working Party 1B. In addition, the APT member countries wishing to formulate their national regulations for SRD devices could consider this compilation as a guide.

II. Survey Results

Q1. State the radio equipment type approval process and related certification and verification bodies

a. Brunei Darussalam

b. China

Type Approval is obligatory for all the SRDs. SRDs should be tested by qualified test body for conformity with technical regulations or requirements. Ministry of Industry and Information Technology is the only certification authority for Type Approval. Only the type-approved short range devices can be used legally in China.

c. Hong Kong

Nil

d. Japan

Please refer to http://www.tele.soumu.go.jp/e/equ/tech/index.htm.

e. Republic of Korea

All the SRDs are subject of Type registration. They should be tested by designated test body for conformity with technical regulations or requirements. The certification body is Radio Research Laboratory. The Korea Communications Commission Notification (KCC Notification) “Regulations on type approval and type registration for broadcasting and communication equipments”.

f. Malaysia

Radio equipment must be certified by SIRIM which is at the moment the only certifying agency appointed by SKMM. Certification is divided into Type Approval & Special Approval. In general all equipment for sale in this country must be type approved. However if equipments are imported for exclusive use by the applicant for temporary period or for private usage, the equipment can be considered for Special Approval certification. This is normally applicable for equipment to be used for trial, exhibition, R&D and training.

g. New Zealand

New Zealand and Australia have established similar self-declaration processes for the certification of SRD products. The supplier of the SRD product to the market (New Zealand or Australia) is registered with the regulator and provided with an identification code.  In New Zealand the SRD product must meet a prescribed Standard, with the supplier maintaining a compliance folder with a description of the product and a declaration of compliance with an appropriate Standard. For products designated as moderate spectrum risk (level of conformity 2) the compliance folder must include a test report or other documented performance specifications, and for high spectrum risk (level of conformity 3) there must be a test report from an accredited testing body. The product must be labeled with the New Zealand supplier identification code when placed on the New Zealand market, except for some 2.4GHz and 5.8GHz SRD products declared and labeled for the Australian market which are deemed acceptable in New Zealand.

h. Philippines

Only type-approved and/or accepted Short Range Devices (SRDs) shall be allowed for use in the country.

i. Singapore

Equipment registration shall be based on the submission of the Supplier’s Declaration of Conformity (SDoC) to signify that the supplier has carried out conformity assessment on the equipment to IDA’s Technical Specifications. All registration requests can be made on-line via http://tls.ida.gov.sg under Equipment Registration group, using one of the following two equipment registration selections:

- Equipment Registration for ISM-band Short Range Devices/Lowe Power Devices;

- Telecommunication Equipment Registration

For Telecommunication Equipment Registration, the supplier may select one of the following two schemes to register:

- Simplified Equipment Registration, SER (Self-declaration);

- General Equipment Registration, GER (Declaration certified by an IDA recognized body or evaluated by IDA)

j. Vietnam

Only type-approved and/or accepted SRDs shall be allowed for use in the Vietnam. For more information, please refer to Decision 45/2006/QD-BBCVT of Minister of Post and Telematics on promulgating "The list of products subject to specialized management by the Ministry of Posts and Telematics and compulsory to be type-approved"

Q2. State the mutual Recognition Arrangement with other countries/regions

a. Brunei Darussalam

Currently AITI has no mutual agreement with other country. However AITI is working with Singapore to finalise the MRA between the two countries.

b. China

Currently, MIIT has no mutual Recognition Arrangement with other countries/regions.

c. Hong Kong

Nil

d. Japan

(1) EC

(2) Singapore

(3) USA

e. Republic of Korea

Korea has entered into MRA with Canada, United States and Vietnam. The test reports from designated laboratories of those countries should be recognized.

f. Malaysia

On 28 January 2005, Malaysia has sealed a bilateral agreement on MRA for telecommunications equipment with Singapore. The signing means that both countries to recognise each other’s equipment conformity testing processes by removing duplicative testing procedures and reducing cost and time required to bring telecommunications equipment into both markets. This is the Phase I of the bilateral MRA where both countries will first accept equipment test reports issued by accredited test laboratories recognised by each country.

The MRA is under ATRC(ASEAN Telecommunications Regulators’ Council) JSC (Joint Sectoral Committee) MRA.

g. New Zealand

New Zealand has entered into mutual recognition agreements for accredited testing bodies with Australia, members of  the Asia-Pacific Economic Cooperation Group, China, members of the European Union, Singapore and Taiwan

h. Philippines

SRDs shall be subject to one time registration prior to use and shall bear National Telecommunications Commission (NTC) registration number issued by the Commission for proper identification. The Commission shall device a standard numbering scheme for the registration of SRDs.

i. Singapore

Equipment that has been certified by the following MRA partners can enter directly into Singapore market without the need for re-certification: Australia, Brunei, Canada, Chinese Taipei, Hong Kong, India, Indonesia, Japan, Malaysia and United States.

j. Vietnam

Vietnam has entered into MRA with Korea, USA.

Q3. State the licensing requirements; whether subject to general licence or individual licence.

a. Brunei Darussalam

No license is required for the usage of the SRDs except for certain frequency band that uses high power. However, “dealer license” is required for companies who sells and distribute these devices. In addition, the short range devices need to be type approval (based on the above technical conditions) before it can be sold and used.

b. China

Individual license is not required for SRDs in China.

c. Hong Kong

Nil

d. Japan

Radio stations with antenna power of 0.01 watts or less which operate without causing interference with or disturbance to the operation of other radio stations under certain conditions,

and which exclusively use Radio Equipment with Conformity Mark.

e. Republic of Korea

All the SRDs are exempted from individual license in Korea.

f. Malaysia

Generally, the licensing of SRD are via Class Assignment (please refer to http://www.skmm.gov.my/what_we_do/spectrum/assign.asp) subject to the specified technical requirements stipulated in the Notification Of Issuance Of Class Assignments

g. New Zealand

In addition to standards conformance the product must also meet the appropriate general license requirements (see http://www.rsm.govt.nz/cms/licensing/types-of-licence/general-user-licences/short-range-devices/ ).

h. Philippines

Only duly accredited radio dealers/manufacturers shall buy, sell and carry on stocks SRDs that are legally imported, type-approved/accepted, and registered with the Commission, and the same shall be included in their sales and stocks report. However, any individual/entity intending to use, purchase, import SRDs may be allowed provided that the equipment are type-approved/accepted and shall be registered with the Commission.

i. Singapore

Generally, all SRDs operating within IDA’s technical specifications are licence-exempted.

j. Vietnam

All the SRDs operating within technical specifications of Ministry of Information and Communications (MIC) are exempted from license in Vietnam.

Q4. State the special operating parameters or conditions for SRDs.

a. Brunei Darussalam

Only subject to the technical conditions stated above.

b. China

Only the type-approved short range devices can be used legally in China.

c. Hong Kong

Nil

d. Japan

Required parameters or conditions varies from system to system.

e. Republic of Korea

SRDs for relaying communications and broadcasting services should be installed by the related service provider or the identities in the contract with the related service provider.

f. Malaysia

Special operating parameters or conditions for SRDs

Please refer to http://www.skmm.gov.my/registers/cma/ClassAssignment/pdf/Class%20Assign-BI-register.pdf

g. New Zealand

The operating conditions for SRDs are contained within the general license mentioned above e.g. shall not cause interference, and shall not claim interference from other licensed services, indoor vs indoor/outdoor use, particular bands may be used only for particular applications.

h. Philippines

Duly accredited radio dealers/manufacturers shall register with the Commission all imported SRDs not later than 5 days upon release from the Bureau of Customs.

i. Singapore

Short range devices are intended for communications in confined areas of buildings as well as for localized on-site operations. The operation of SRD shall not cause interference with other authorized radio-communication services, and be able to tolerate any interference caused by other radio-communication services, electrical or electronic equipment. Also, the SRD shall be marked with the supplier/manufacturer’s model or type reference. The markings shall be legible, indelible and readily visible.

j. Vietnam

All the SRDs operating within MIC’s technical specifications shall not cause interference, and shall not claim interference from other licensed devices. The current MIC’s technical specifications regulation is Decision 47/2006/QD-BBCVT of 29/11/2006.

Q5. State the future policy regarding licensing framework for SRDs (if any)

a. Brunei Darussalam

Nil

b. China

Nil

c. Hong Kong

Nil

d. Japan

Nothing particular

e. Republic of Korea

Nothing available

f. Malaysia

Nil

g. New Zealand

The current SRD licensing and certification arrangement is regularly reviewed, with particular emphasis on harmonization with Australia.

h. Philippines

Nil

i. Singapore

SRD licensing framework will be reviewed periodically, with the aim to streamline licensing processes and relax technical conditions where appropriate.

j. Vietnam

The SRDs regulations in Vietnam is now being revised. New frequency bands may be open for SRDs.

Conclusions

Overall, the survey results have provided useful information on the regulatory and technical regulations established in the APT member countries, which now serve as a good reference for other administrations intending to open up more frequency bands for SRD. As technical regulations differ among the countries, this has created much difficulty in harmonising certain frequency bands in this region. It is also noted that even when the same band is allowed for SRD operation, the type of application designated for SRD may not be the same; resulting in different categories of SRD operating under various output power levels.

The compilation of survey results in Table 1 shows the frequency bands already harmonized across Brunei Darussalam, Hong Kong, Japan, Korea, Malaysia, Philippines, New Zealand and Singapore,  and is summarised as follows:

| Typical Applications | Frequencies / Frequency band (MHz) | Range of the maximum power level |
| --- | --- | --- |
| Cordless Phones / Telemetry | 315 | 25uW to 10mW e.r.p |
| Medical Implant | 402-405 | 25uW e.r.p |
| RFID | 433.92 | 1mW to 25m W e.r.p |
| WLAN | 2400-2483.5 | 10mW to 1000mW e.i.r.p |
| Vehicle Radar | 76000-77000 | 10mW to 100W e.i.r.p |

Table 1

Table 2 (see footnote 2) shows the frequency bands already harmonized across Hong Kong, Korea, Philippines, New Zealand and Singapore:

| Typical Applications | Frequencies / Frequency band (MHz) | Range of maximum power level |
| --- | --- | --- |
| RFID | 13.553-13.567 | 100 mW (e.i.r.p) / 42 dBµA/m at 10m |
|  | 26.96-27.28 | 0.5W to 3W e.r.p / 42 dBµA/m at 10m |
| Model Control | 40.66-40.70 | 100 mW to 1000mW e.r.p |
|  | 72-72.25 | 10 mW to 750 mW e.r.p |
| Cordless Phones | 864.8-865 | 10 mW to 1000 mW e.r.p |
| WLAN | 5725-5850 | 10 mW to 4W e.i.r.p |

Table 2

As market developments of SRD are primarily dependent on economies of scale, there is a need to work towards harmonized spectrum bands for SRD, to provide regulatory certainty for manufactures and end-users, as well as to catalyst the developments of new SRD applications.  To achieve the above, the possible approach is to simplify the conditions on the use of SRD, to the largest extent possible, and to adopt frequency bands that are already established in Europe or US. For example, ERC Recommendation 70-03 could be a key reference for the development of regulations for SRD, the ITU Recommendation SM. 1538-2 established by the ITU or other relevant bodies. This however should take into account potential interference from SRD and ensuring that existing services remain protected.

In considering the complexity of identifying common frequencies and technical conditions, the proposed follow-up action is to seek consensus among APT members in designating common bands for SRD in the above-mentioned table and timely development of an APT Recommendation or Decision in future AWF meetings.

ANNEX 1

QUESTIONNAIRE on

COMMON FREQUENCY BANDS FOR OPERATION OF SHORT RANGE RADIOCOMMUNICATION DEVICES

by

SWG Sub-WG5

A. Introduction

Short Range Radiocommunication Devices (SRDs) is a term which applies to radio transmitters operating over short distance and at low power. Generally, these devices have little potential to cause interference to other radio equipments. They are usually exempted from licensing if operating within approved technical standards.

ITU Radio Assembly held in Geneva in October 2007 approved Resolution ITU-R54 on the ‘Studies to achieve harmonization for short-range radiocommunication devices (SRDs).’

In response to this Resolution, ITU-R Study Group is expected to start its work from June 2008 when the first meeting is held in Geneva. It is necessary to share the regulatory information regarding operation of SRDs among APT member countries as early as possible.  Spectrum Sub- Working Group 5 proposes to conduct a survey on the technical and procedural regulations of SRDs.

It is also helpful for APT member countries to prepare their national contributions to ITU-R study and to facilitate market access by equipment vendors in Asia Pacific.

The survey information solicited may form the basis for drafting of APT report or recommendation, to be used as a guide for APT member countries wishing to formulate their national regulations for SRDs devices.

B. About Your Administration

Name of Administration	:

Contact Person		:

Telephone			:

Postal Address			:

Email Address 		:

C. Survey Questions

In addition to the technical regulations or requirements (with examples given) as shown in Section D, you are kindly requested to also provide information on the administrative or procedural regulations covering the following:

- The radio equipment type approval process and related certification and verification bodies

- Mutual Recognition Arrangement with other countries/regions

- Licensing requirements; whether subject to general licence or individual licence

- Special operating parameters or conditions for SRDs

- Future policy regarding licensing framework for SRDs (if any)

The provision of the above information is optional.

ANNEX 2

Technical Regulations in Brunei Darussalam

| Technical Regulations for Short Range Radiocommunication Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No | Typical Application Types | Authorised Frequency Bands / Frequencies | Maximum Field Strength / RF Output power | Transmitter Spurious Emissions | Applicable Radio Standards | Remarks |
| 1 | Induction loop system / RFID | 16 – 150 kHz | ≤ 66 dBμA/m @ 3m | ≥ 32 dB below carrier at 3 m or EN 300 224-1 | EN 300 224-1 |  |
|  |  | 150 – 5000 kHz | ≤ 13.5 dBμA/m @ 10m |  |  |  |
|  |  | 6765 – 6795 kHz | ≤ 42 dBμA/m @ 10m |  |  |  |
|  |  | 7400 – 8800 kHz | ≤ 9 dBμA/m @ 10m |  |  |  |
|  |  | 13.553 – 13.567 MHz | ≤ 94 dBμV/m @ 10m |  |  |  |
| 2 | Radio detection, alarm system | 0.016 – 0.150 MHz | ≤ 100 dBμV/m @ 3m | ≥ 32 dB below carrier at 3 m or EN 300 330-1 | FCC Part 15 or EN 300 330-1 |  |
| 3 |  | 13.553 – 13.567 MHz | ≤ 94 dBμV/m @ 10m |  |  |  |
| 4 |  | 240.15 – 240.30 MHz 300.00 – 300.30 MHz 312.00 – 316.00 MHz 444.40 – 444.80 MHz | ≤ 100 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m or EN 300 220-1 | FCC Part 15 or EN 300 220-1 |  |
| 5 | Wireless microphone | 0.51 – 1.60 MHz | ≤ 57 dBμV/m @ 3m |  |  |  |
| 6 |  | 88.00 – 108.00 MHz | ≤ 60 dBμV/m @ 10m |  |  |  |
| 7 |  | 470.00 – 742.00 MHz | ≤ 10 mW (e.r.p.) |  |  |  |
| 8 | Remote controls of garage door, cameras, toys and miscellaneous devices | 26.96 – 27.28 MHz | ≤ 100 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m or EN 300 220-1 | FCC Part 15 or EN 300 220-1 |  |
|  |  | 40.665 – 40.695 MHz | ≤ 100 mW (e.r.p.) |  |  |  |
|  |  | 72.13 – 72.21 MHz |  |  |  |  |
| 9 | Remote controls of aircraft and glider models, telemetry, detection and alarm systems | 26.96 – 27.28 MHz 29.70 – 30.00 MHz | ≤ 100 mW (e.r.p.) |  |  |  |
| 10 | Medical and Biological telemetry | 40.50 – 41.00 MHz | ≤ 0.01 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m or EN 300 220-1 | FCC Part 15 or EN 300 220-1 |  |
|  |  | 216.00 – 217.00 MHz | > 25 μW to ≤ 100 mW (e.r.p.) |  |  |  |
|  |  | 454.00 – 454.50 MHz | ≤ 2 mW (e.r.p.) |  |  |  |
| 11 | Wireless modem, data communication system | 72.080 MHz 72.200 MHz 72.400 MHz 72.600 MHz | ≤ 100 mW (e.r.p.) | ≥ 43 dB below carrier over 100 kHz to 2000 MHz; EN 300 390-1 or EN 300 113-1 | EN 300 390-1 or EN 300 113-1 |  |
| 12 | Short range radar systems such as automatic cruise control and collision warning systems for vehicle | 76 – 77 GHz | ≤ 37 dBm (e.r.p.) when vehicle is in motion ≤ 23.5 dBm (e.r.p.) when vehicle is stationary | FCC Part 15 § 15.253 (c) or EN 301 091 | FCC Part 15 or EN 301 091 |  |
| 13 | Radio telemetry, telecommand system | 433.05 – 434.79 MHz | ≤ 10 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m or EN 300 220-1 | FCC Part 15 or EN 300 220-1 |  |
| 14 | Radio Telemetry, Telecommand, RFID system | 866 – 869 MHz 923 – 925 MHz | ≤ 500 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m; EN 300 220-1 or EN 302 208 | FCC Part 15 ; EN 300 220-1 or EN 302 208 |  |
| 15 | Radio Frequency Identification (RFID) systems | 923 – 925 MHz | > 500 mW (e.r.p.) ≤ 2000 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m; EN 300 220-1 or EN 302 208 | FCC Part 15 ; EN 300 220-1 or EN 302 208 | Only RFID systems operating in the 923 -925 MHz frequency band shall be allowed to transmit between 500 mW and 2000 mW (e.r.p.), and approved on an exceptional basis. |
| 16 | Wireless video transmitter and other SRD applications | 2.4000 – 2.4835 GHz | ≤ 100 mW (e.i.r.p.) | FCC Part 15 § 15.209; § 15.249 (d) or EN 300 440-1 | FCC Part 15 or EN 300 440-1 |  |
| 17 |  | 10.50 – 10.55 GHz | ≤ 117 dBμV/m @ 10m |  |  |  |
| 18 |  | 24.00 – 24.25 GHz | ≤ 100 mW (e.i.r.p.) |  |  | Radar gun devices are not allowed to operate under this provision. |
| 19 | Bluetooth | 2.4000 – 2.4835 GHz | ≤ 100 mW (e.i.r.p.) | FCC Part 15 § 15.209; or EN 300 328 | FCC Part 15 § 15.247 or EN 300 328 |  |
| 20 | Wireless LAN only | 2.4000 – 2.4835 GHz | ≤ 200 mW (e.i.r.p.) |  |  | WLAN for non-localised operations shall be approved on an exceptional basis. |
| 21 | SRD applications | 5.725 – 5.850 GHz | ≤ 100 mW (e.i.r.p.) | FCC Part 15 § 15.209 | FCC Part 15 § 15.247 or 15.407 |  |
| 22 | Wireless LAN | 5.725 – 5.850 GHz | ≤ 1000 mW (e.i.r.p.) |  |  | Non-localised operations shall be approved on an exceptional basis. |
| 23 |  | 5.725 – 5.850 GHz | > 1000 mW (e.i.r.p.) ≤ 4000 mW (e.i.r.p.) |  |  | Operating under this provision shall be approved on an exceptional basis. |
| 24 | Wireless LAN | 5.150 – 5.350 GHz | > 100 mW (e.i.r.p.) ≤ 200 mW (e.i.r.p.) | FCC Part 15 § 15.407 (b) or EN 301 893 | FCC Part 15 § 15.407 or EN 301 893 | WLAN operating in 5.250 – 5.350 GHz under this provision shall employ Dynamic Frequency Selection (DFS) mechanism and implement Transmit Power Control (TPC). Non-localised operations shall be approved on an exceptional basis. |
| 25 | Wireless LAN | 5.150 – 5.350 GHz | ≤ 100 mW (e.i.r.p.) | FCC Part 15 § 15.407 (b) or EN 301 893 | FCC Part 15 § 15.407 or EN 301 893 | WLAN operating under this provision shall implement DFS function in the frequency range 5.250 – 5.350 GHz. Non-localised operations shall be approved on an exceptional basis. |

Technical Regulations in China

2	Technical parameters requirements

2.1	Analogue cordless telephone

Transmit frequencies used for base set (MHz):	45.000, 45.025, 45.050, ..., 45.475

Transmit frequencies used for hand set (MHz):	48.000, 48.025, 48.050, ..., 48.475

Total channel number:	             20

Radiated power limit:	                         20 mW (e.r.p.)

Maximum occupied bandwidth:	16 kHz

Frequency tolerance:	                        1.8 kHz

2.2	Wireless audio transmitters and measuring devices for civilian purposes

–	Operating frequency band (MHz):	87 to 108

Radiated power limit:	                        3 mW (e.r.p.)

Maximum occupied bandwidth:	200 kHz

Frequency tolerance:	                        100  10-6

–	Operating frequency band (MHz):	75.4 to 76.0, 84 to 87

Radiated power limit:	                        10 mW (e.r.p.)

Maximum occupied bandwidth:	200 kHz

Frequency tolerance:	                        100  10-6

–	Operating frequency band (MHz):	189.9 to 223.0

Radiated power limit:	                        10 mW (e.r.p.)

Maximum occupied bandwidth:	200 kHz

Frequency tolerance:	                        100 10-6

–	Operating frequency bands (MHz):	470 to 510, 630 to 787

Radiated power limit:	                        50 mW (e.r.p.)

Maximum occupied bandwidth:	200 kHz

Frequency tolerance:	                        100  10-6

2.3	Model and toy remote-control devices

–          Operating frequencies (MHz):	26.975, 26.995, 27.025, 27.045,

27.075, 27.095, 27.125, 27.145,

27.175, 27.195, 27.225, 27.255

Radiated power limit:	                        750 mW (e.r.p.)

Maximum occupied bandwidth:	8 kHz

Frequency tolerance:	                       100  10-6

–         Operating frequencies (MHz):  	40.61, 40.63, 40.65, 40.67, 40.69,

40.71, 40.73, 40.75, 40.77, 40.79,

40.81, 40.83, 40.85

Radiated power limit:	                        750 mW (e.r.p.)

Maximum occupied bandwidth:	20 kHz

Frequency tolerance:	                        30  10-6

–          Operating frequencies (MHz):	72.13, 72.15, 72.17, 72.19, 72.21,

72.79, 72.81, 72.83, 72.85, 72.87

Radiated power limit:	                        750 mW (e.r.p.)

Maximum occupied bandwidth:	20 kHz

Frequency tolerance:	                        30  10-6

2.4	Citizen band private mobile radio equipment

–	Operating frequencies (MHz):	409.7500, 409.7625, 409.7750, 409.7875,

409.8000, 409.8125, 409.8250, 409.8375,

409.8500, 409.8625, 409.8750, 409.8875,

409.9000, 409.9125, 409.9250, 409.9375,

409.9500, 409.9625, 409.9750, 409.9875

Radiated power limit:	                        500 mW (e.r.p.)

Modulation type:	                        F3E

Channel spacing:	                       12.5 kHz

Frequency tolerance:	                        5  10-6

2.5	General radio remote-control devices

–	Operating frequency bands (MHz):	470 to 566, 614 to 787

Radiated power limit:	                        5 mW (e.r.p.)

Maximum occupied bandwidth:	1 MHz

2.6	Biomedical telemetry transmitters

–          Operating frequency bands (MHz):	174 to 216, 407 to 425, 608 to 630

Radiated power limit:	                        10 mW (e.r.p.)

Frequency tolerance:	                        100  10-6

–         Operating frequency bands (MHz):	402 to 405

Radiated power limit:	                        25μW (e.r.p.)

Maximum occupied bandwidth:         300kHz

Frequency tolerance:	                        100  10-6

2.7	Equipment for lifting

–	Operating frequencies (MHz):	223.100, 223.700, 223.975, 224.600,

225.025, 225.325, 230.100, 230.700,

230.975, 231.600, 232.025, 232.325

Radiated power limit:	                        20 mW (e.r.p.)

Maximum occupied bandwidth:	16 kHz

Frequency tolerance:	                        4  10-6

2.8	Equipment for weighing

–	Operating frequencies (MHz):	223.300, 224.900, 230.050, 233.050, 234.050

Maximum occupied bandwidth:	50 kHz

Radiated power limit:	                        50 mW (e.r.p.)

Frequency tolerance:	                        4  10-6

–	Operating frequencies (MHz):	450.0125, 450.0625, 450.1125, 450.1625, 450.2125

Maximum occupied bandwidth:	20 kHz

Radiated power limit:	                        50 mW (e.r.p.)

Frequency tolerance:	                        4  10-6

2.9	Radio remote-control equipment used in industry

–	Operating frequencies (MHz):	418.950, 418.975, 419.000, 419.025,

419.050, 419.075, 419.100, 419.125,

419.150, 419.175, 419.200, 419.250, 419.275

Radiated power limit:	                        20 mW (e.r.p.)

Maximum occupied bandwidth:	16 kHz

Frequency tolerance:	                         4 10-6

2.10	Equipment for transporting data

–	Operating frequencies (MHz):	223.150, 223.250, 223.275, 223.350,

224.050, 224.250, 228.050, 228.100,

228.200, 228.275, 228.425, 228.575,

228.600, 228.800, 230.150, 230.250,

230.275, 230.350, 231.050, 231.250

Radiated power limit:	                        10 mW (e.r.p.)

Maximum occupied bandwidth:	16 kHz

Frequency tolerance:	                        4 10-6

2.11	Radio control devices for civilian purposes

–	Operating frequency bands (MHz):	314 to 316, 430 to 432, 433 to 434.79

Radiated power limit:	                        10 mW (e.r.p.)

Maximum occupied bandwidth:	400 kHz

–	Operating frequency bands (MHz):	779 to 787

Radiated power limit:	                        10 mW (e.r.p.)

–          Operating frequency bandwidth (MHz):                 868 to 868.6

Radiated power limit:                                              5mW (e.r.p)

Maximum duty cycle:                                              1%

Frequency tolerance:                                             100 10-6

2.12	Other SRDs

–	Equipment A:

Operating frequency band (kHz):	9 to 190

Magnetic field strength limit:	72 dB A/m at 10 m (in 9 to 50 kHz, quasi-peak detector)

72 dB A/m at 10m (in 50 to 190 kHz descending 3 dB/octave, quasi-peak detector)

–	Equipment B:

Operating frequency bands (MHz): 	1.7 to 2.1, 2.2 to 3.0, 3.1 to 4.1, 4.2 to 5.6, 5.7 to 6.2, 7.3 to 8.3, 8.4 to 9.9

Magnetic field strength limit:     	     9 dB A/m at 10 m (quasi-peak detector)

Maximum 6 dB bandwidth:	                 200 kHz

Frequency tolerance:	                             100  10-6

–	Equipment C:

Operating frequency bands (MHz):	6.765 to 6.795, 13.553 to 13.567, 26.957 to 27.283

Magnetic field strength limit:   	      42 dB  A/m at 10 m (quasi-peak detector)

Frequency tolerance:	100  10-6

Spurious emission limit:	9 dB A/m at 10 m (in 13.553 to 13.567 MHz, any emission removed by less than 140 kHz from the band edges, quasi-peak detector)

–	Equipment D:

Operating frequency band: 	315 kHz to 30 MHz (excluding Equipment A, B, C)

Magnetic field strength limit:  	-5 dB A/m at 10 m (in 315 kHz to 1 MHz, quasi-peak detector)

–15 dB A/m at 10 m (in 1 to 30 MHz, quasi-peak detector)

–	Equipment E:

Operating frequency band (MHz): 	40.66 to 40.70

Radiated power limit:	10 mW (e.r.p.)

Frequency tolerance: 	100  10-6

–       Equipment F (excluding digital cordless telephone, Bluetooth devices and WLAN devices):

Operating frequency band (MHz): 	2400 to 2483.5

Radiated power limit:	10 mW (e.i.r.p.)

Frequency tolerance: 	75 kHz

–	Equipment G:

Operating frequency band (GHz): 	24.00 to 24.25

Radiated power limit:	20 mW (e.i.r.p.)

–           Equipment H:

Operating frequency band (GHz):       	59 to 64

Conducted power limit: 	10 dBm

Radiated power limit (Peak):               	47dBm (e.i.r.p.)

Radiated power limit (RMS):              	44dBm (e.i.r.p.)

Out-of-band emission limit:                	57-59GHz and 64-66GHz

-5dBm/MHz (RMS)

2.13	Digital cordless telephone

–	Operating frequency band (MHz): 	2400 to 2483.5

Radiated power limit:     	25 mW (average EIRP)

Frequency tolerance:	20  10-6

2.14	Automotive radars (Collision avoidance radars)

–	Operating frequency band (GHz):	76 to 77

Radiated power limit:	55 dBm (peak EIRP)

2.15     RFID systems at 800/900 MHz bands

–           Operating frequency band (MHz):  	840~845/ 920~925

Radiated power limit:

| Frequency band (MHz) | Limit (e.r.p) |
| --- | --- |
| 840.5-844.5 920.5-924.5 | 2W |
| 840-845 920-925 | 100mW |

Central Frequency:     fc(MHz)=840.125+N

fc(MHz)=920.125+M

M, N: integer 1~19

Maximum occupied bandwidth:		16 kHz

Frequency tolerance: 		20 10-6

Adjacent channel leakage power ratio limit: 	40 dB (1st adjacent channel)

60 dB (2nd adjacent channel)

Transmitter spurious emissions limit:

| Operating Mode | Frequency range | Emission limit (dBm) | Detector |
| --- | --- | --- | --- |
|  | 30MHz-1GHz | -36 | RMS |
|  | 1-12.75GHz | -30 |  |
|  | 806-821MHz 825-835MHz 851-866MHz 870-880MHz 885-915MHz 930-960MHz | -52 |  |
|  | 1.7-2.2GHz | -47 |  |
| Idle Mode | 30MHz-1GHz | -57 |  |
|  | 1-12.75GHz | -47 |  |

2.16	Car alarm

–	Operating frequencies (MHz):	409.7500,  409.7625, 409.7750, 409.7875,

409.8000,  409.8125,  409.8250, 409.8375,

409.8500, 409.8625

Radiated power limit:    	0.5 W (e.r.p.)

Modulation type:	F2D

Frequency tolerance:            	510-6

Transmitter Spurious emissions limit: 50μW

3	Operating parameters requirements

3.1	The use of SRDs is forbidden when it causes harmful interference to other legal radio stations. If it causes harmful interference, the operation must be stopped. It can be put into operation again only after special measures are taken to eliminate such interference.

3.2	The use of SRDs must avoid or bear the interference from other legal radio stations or radiation interference from ISM devices. There is no legal protection for SRDs when it encounters interference. But the user can make an appeal to the local radio regulatory office.

3.3	Its use is forbidden near airports or airplanes.

3.4	The use of SRDs need not be licensed, but the necessary examination or test from the radio regulatory office is required so as to ensure that the SRDs perform within the acceptance range.

3.5	In order to develop, produce or import SRDs, they must go through the relevant formalities according to the relevant rules issued by the State Radio Office.

3.6	SRDs, without type approval by the State Radio Office, cannot be produced, sold and used in China.

3.7	For SRDs having passed the type approval of the State Radio Office, manufacturers and users cannot change the operating frequency or increase the transmitting power arbitrarily (including the addition of an extra RF amplifier). They cannot install any external antenna or replace the original one by another transmitting antenna, and cannot change the original design specification and function arbitrarily.

3.8	SRDs must be installed inside an integrated cabinet. Its external adjustment and control are only used within the range of the technical specifications of the approved type.

3.9	When using the SRDs listed below the followed stipulations must be applied:

3.9.1	Wireless audio transmitters:

They cannot be used locally when the used frequency is the same as that of the local radio or TV stations.

Their operation must be stopped if they interfere with local stations. They can be reused only after eliminating the interference and adjusting the frequency to a free one.

To avoid interference to biomedical telemetry equipment wireless audio transmitters cannot be used in the hospital. Manufacturers of wireless audio transmitters have to demonstrate this stipulation in their product manuals.

3.9.2	Biomedical telemetry transmitters:

Radio devices for transmitting measurement signals of either human or animal biomedical phenomena are allowed to be used by hospitals or medical institute and forbidden to cause interference to the radio astronomy service.

Radio devices operating in the 402-405 MHz band should use “Search before transmitting” protocol.

3.9.3	Equipment for lifting, equipment for weighing:

Before installation, the EMC environment must be tested so as to avoid interference to other equipment which can cause unnecessary production accidents.

Their operation must be stopped immediately when they cause harmful interference. They can be reused only after removing the interference by adjusting the frequency to a free one.

In order to protect the radio astronomy service, devices operating at the following frequencies are forbidden to be used in Beijing and Pingtang, Guizhou Province.

223.100 MHz, 223.700 MHz, 223.975 MHz, 224.600 MHz, 225.025 MHz, 225.325 MHz, 230.100 MHz, 230.700 MHz, 230.975 MHz, 231.600 MHz, 232.025 MHz, 232.325 MHz.

3.9.4	Radio remote-control equipment used in industry:

It must be used inside the industrial workshop (or inside the building).

3.9.5	Equipment for transporting data:

It must be used inside the building.

In order to protect the radio astronomy service, devices operating at the following frequencies  are forbidden to be used in Beijing and Pingtang, Guizhou Province.

223.150 MHz, 223.250 MHz, 223.275 MHz, 223.350 MHz, 224.050 MHz, 224.250 MHz, 228.050 MHz, 228.100 MHz, 228.200 MHz, 228.275 MHz, 228.425 MHz, 228.575 MHz, 228.600 MHz, 228.800 MHz, 230.150 MHz, 230.250 MHz, 230.275 MHz, 230.350 MHz, 231.050 MHz, 231.250 MHz.

3.9.6	Radio control devices for civilian purposes:

They cannot be used for radio remote-control toys and models.

3.9.7	General radio remote-control devices:

They cannot be used for radio remote control toys.

They cannot be used locally when the used frequency is the same as that of local radio or TV stations.

Their operation must be stopped if they cause harmful interference to local radio or TV stations. They can be reused only after removing the interference by adjusting the frequency to a free one.

Equipment H operating in 59-64 GHz should not interfere with legal radio equipments for  space research, monitoring Earth's resources, radio astronomy and other radio services , and they would not be protected from interferences.

3.9.8	Model and toy remote-control devices:

Remote-control devices for unmanned models and toys, such as plane models in the air, ship models over the water surface and automobile models on land, cannot be used for other types of radio equipment.

They are limited to one-way control.

They cannot be used for transmitting audio signals.They are required to stop use during radio control period and within radio control area. To meet requirements of electromagnetic environment, all kinds of model and toy remote-control devices are forbidden to use within a radius of 5 000 metres. The centre of a circle for this forbidden area is the middle of the airport runway.

Radio transmitters are forbidden to set up in models.

3.9.9	Digital cordless telephone:

Digital cordless telephone operating in the 2 400-2 483.5 MHz band should use at least 75 hopping frequencies.

The average time of occupancy on any channel shall not be greater than 0.4 seconds within a period of 60 seconds.

3.9.10    RFID systems at 800/900 MHz bands

RFID equipments should operate in frequency hopping mode, and the maximum dell time for each channel should be 2s.

3.9.11    Car alarm

Duration of car alarm signal should be less than 15s.

4	General requirements

4.1	Frequency ranges of measurement for radiated spurious emissions

TABLE 1

| Operating frequency range | Lower frequency of measurement range | Upper frequency of measurement range |
| --- | --- | --- |
| 9 kHz - 100 MHz | 9 kHz | 1 GHz |
| 100-600 MHz | 30 MHz | 10th harmonic |
| 600 MHz - 2.5 GHz | 30 MHz | 12.75 GHz |
| 2.5-13 GHz | 30 MHz | 26 GHz |
| Above 13 GHz | 30 MHz | 2nd harmonic |

4.2	Radiated spurious emission limits

4.2.1	Radiated spurious emission limits are showed in the following table when a transmitter is in the state of maximum emission power.

TABLE 2

| Frequency range | Testing bandwidth | Emission limit | Detector |
| --- | --- | --- | --- |
| 9-150 kHz | 200 kHz (6 dB) | 27 dB A/m at 10 m (descending 3 dB/octave) | Quasi-peak |
| 150 kHz - 10 MHz | 9 kHz (6 dB) |  |  |
| 10-30 MHz | 9 kHz (6 dB) | –3.5 dB A/m at 10 m | Quasi-peak |
| 30 MHz - 1 GHz | 100 kHz (3 dB) | –36 dBm | RMS |
| 1-40 GHz | 1 MHz (3 dB) | –30 dBm | RMS |
| Above 40 GHz | 1 MHz (3 dB) | –20 dBm | RMS |

(1)	Magnetic field strength measurement should be made on an open field site. Radiated power measurement should be performed in a fully anechoic chamber.

(2)	The state of a transmitter operating at frequencies below 30 MHz can be set up in the state single carrier transmission.

(3)	If the concrete technical parameter does not comply with the general requirements, the former should be adopted.

4.2.2	Radiated spurious emission limits are showed in the following table when a transmitter is in idle or standby state.

TABLE 3

| Frequency range | Testing bandwidth | Emission limit | Detector |
| --- | --- | --- | --- |
| 9-150 kHz | 200 kHz (6 dB) | 6 dB A/m at 10 m (descending 3 dB/octave) | Quasi-peak |
| 150 kHz - 10 MHz | 9 kHz (6 dB) |  |  |
| 10-30 MHz | 9 kHz (6 dB) | –24.5 dB A/m at 10 m | Quasi-peak |
| 30 MHz - 1 GHz | 100 kHz (3 dB) | –47 dBm | RMS |
| Above 1 GHz | 1 MHz (3 dB) |  |  |

4.3	Radiated spurious emission should not exceed –54 dBm in 48.5-72.5 MHz, 76-108 MHz, 167-223 MHz, 470-566 MHz, and 606-798 MHz bands.

4.4	Conducted disturbance emissions at power ports, signal ports and telecommunication ports should comply with GB9254-1998: “Information technology equipment – Radio disturbance characteristics – Limits and methods of measurement”. This technical standard was issued by former State Administration of Quality and Technology Supervision of China in 1998.

4.5	For the bands above 30 MHz within operating frequency ranges mentioned above, radiated power can not exceed –80 dBm/Hz (e.i.r.p.) at the band edges. For the bands below 30 MHz, the edges of the occupied frequency bandwidth on any operating channel (99% of energy) can not exceed operating frequency ranges mentioned above.

Manufacturers of SRDs should announce the condition extremes of operating environment for normal use. Emission power and frequency tolerance under the condition extremes should meet requirements mentioned above.

Technical Regulations in Hong Kong

| Technical Regulations for Short Range Radiocommunication Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Num | Typical Application Type | Authorised Frequency Bands / Frequencies | Maximum Field Strength / RF Output power | Remarks |
|  |  | 3–195 kHz | electric field strength not to exceed 40 dBμV/m and magnetic field strength not to exceed 48.4 dBnA/m at 100 m from the apparatus |  |
|  | Cordless phone | 1627.5-1796.5 kHz | electric field strength not to exceed 88 dBμV/m at 30 m from the apparatus |  |
|  | RFID | 13.553-13.567 MHz | (a) electric field strength not to exceed 80 dBμV/m at 30 m from the apparatus; or (b) magnetic field strength not to exceed 42 dBμA/m at 10 m from the apparatus |  |
|  |  | 26.96-27.28 MHz | mean power not to exceed 0.5 W |  |
|  | Wireless mic | 33-33.28 MHz | e.r.p. not to exceed 10 mW |  |
|  | Model control | 35.145 – 35.225 MHz | e.r.p. not to exceed 100 mW |  |
|  | Wireless mic | 36.26-36.54 MHz | e.r.p. not to exceed 10 mW |  |
|  | Wireless mic | 36.41-36.69 MHz | e.r.p. not to exceed 10 mW |  |
|  | Wireless mic | 36.71-36.99 MHz | e.r.p. not to exceed 10 mW |  |
|  | Wireless mic | 36.96-37.24 MHz | e.r.p. not to exceed 10 mW |  |
|  | Model control | 40.66-40.70 MHz | e.r.p. not to exceed 100 mW |  |
|  |  | 42.75-43.03 MHz | e.r.p. not to exceed 10 mW |  |
|  | Cordless phone | 43.71-44.49 MHz | electric field strength not to exceed 10 mV/m at 3 m from the apparatus |  |
|  |  | 44.73-45.01 MHz | e.r.p. not to exceed 10 mW |  |
|  | Cordless phone | 46.6-46.98 MHz | electric field strength not to exceed 10 mV/m at 3 m from the apparatus |  |
|  |  | 47.13-47.41 MHz | e.r.p. not to exceed 10 mW |  |
|  | Cordless phone | 47.43-47.56 MHz | e.r.p. not to exceed 10 mW |  |
|  | Cordless phone | 48.75-50 MHz | electric field strength not to exceed 10 mV/m at 3 m from the apparatus |  |
|  | Model control | 72.00–72.02 MHz | carrier power not to exceed 750 mW |  |
|  |  | 72.12–72.14 MHz |  |  |
|  |  | 72.16–72.22 MHz |  |  |
|  |  | 72.26–72.28 MHz |  |  |
|  | Wireless mic | 173.96-174.24 MHz | e.r.p. not to exceed 20 mW |  |
|  | Wireless mic | 187.5-188.0 MHz | e.r.p. not to exceed 10 mW |  |
|  | Cordless phone | 253.85-255 MHz | e.r.p. not to exceed 12 mW |  |
|  |  | 266.75-267.25 MHz | e.r.p. not to exceed 10 mW |  |
|  |  | 313.75-314.25 MHz | e.r.p. not to exceed 10 mW |  |
|  |  | 314.75-315.25 MHz | e.r.p. not to exceed 10 mW |  |
|  | Cordless phone | 380.2-381.325 MHz | e.r.p. not to exceed 12 mW |  |
|  | Medical implant | 402 – 405 MHz | e.i.r.p. not to exceed 25 W |  |
|  | Portable radios | 409.74-410 MHz | e.r.p. not to exceed 0.5 W |  |
|  | RFID | 433.92 MHz centred frequency and 500 kHz occupied bandwidth | e.r.p. not to exceed 2.2 mW |  |
|  |  | 819.1-823.1 MHz | (a) e.r.p. not to exceed 100 mW; and (b) power spectral density not to exceed 10 mW per 25 kHz |  |
|  | Cordless phone | 864.1-868.1 MHz | carrier power or e.r.p. not to exceed 10 mW |  |
|  | RFID | 865–868 MHz | e.r.p. not to exceed 100 mW |  |
|  | RFID | 865.6–867.6 MHz | e.r.p. not to exceed 2 W |  |
|  | RFID | 865.6–868 MHz | e.r.p. not to exceed 500 mW |  |
|  |  | 919.5-920.0 MHz | e.r.p. not to exceed 10 mW |  |
|  | RFID | 920–925 MHz | e.i.r.p. not to exceed 4 W |  |
|  | Cordless phone | 1880-1900 MHz | (a) peak power not to exceed 250 mW for apparatus with antenna output terminal; or (b) peak e.i.r.p. not to exceed 250 mW for apparatus with integral antenna |  |
|  | Cordless phone | 1895-1906.1 MHz | (a) carrier power not to exceed 10 mW for apparatus with antenna output terminal; or (b) e.r.p. not to exceed 10 mW for apparatus with integral antenna |  |
|  | WLAN, RFID | 2400-2483.5 MHz | (a) peak e.i.r.p. not to exceed 4 W for frequency hopping spread spectrum modulation or digital modulation systems; or (b) aggregate e.r.p. not to exceed 100 mW for any modulation |  |
|  | WLAN | 5150-5350 MHz | e.i.r.p. not to exceed 200 mW using only digital modulation |  |
|  | WLAN | 5470–5725 MHz | e.i.r.p. not to exceed 1 W |  |
|  | WLAN | 5725-5850 MHz | (a) peak e.i.r.p. not to exceed 4 W for frequency hopping spread spectrum modulation or digital modulation systems; or (b) aggregate e.r.p. not to exceed 100 mW for any modulation |  |
|  |  | 18.82-18.87 GHz | (a) e.r.p. not to exceed 100 mW; and (b) power spectral density not to exceed 3 mW per 100 kHz |  |
|  | Vehicle radar | 76–77 GHz | carrier power not to exceed 10 mW |  |

Technical Regulations in Japan

|  | Technical Regulations for Short Range Radiocommunication Devices |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type of emission |  | Frequency band (MHz) | Occupied bandwidth (kHz) | Power level or spectral density (e.i.r.p.) | Antenna power and Antenna gain | Carrier sense |  |
| Telemeter, telecontrol and data transmission |  |  |  |  |  |  |  |
| - |  | 312-315.25 |  |  250W (–6 dBm) | - | Not required |  |
|  |  | 312-315.05 |  |  25W (–16 dBm) |  |  |  |
|  |  | 426.025-426.1375 (12.5 kHz spacing) |  8.5 |  1.6 mW (2.14 dBm)  |  1 mW 2.14 dBi | Not required |  |
|  |  | 426.0375-426.1125 (25 kHz spacing) |  8.5  16 |  1.6 mW (2.14 dBm) |  1 mW 2.14 dBi | Not required |  |
|  |  | 429.175-429.7375 (12.5 kHz spacing) |  8.5 |  |  | 7 V |  |
|  |  | 429.8125-429.9250 (12.5 kHz spacing) |  |  |  |  |  |
|  |  | 449.7125-449.8250 (12.5 kHz spacing) |  |  |  |  |  |
| F1D, F1F, F2D, F2F, F7D, F7F, G1D, G1F, G2D, G2F, G7D, G7F, D1D, D1F, D2D, D2F, D7D or D7F |  | 449.8375-449.8875 (12.5 kHz spacing) |  |  |  |  |  |
|  |  | 469.4375-469.4875 (12.5 kHz spacing) |  |  |  |  |  |
|  |  | 1 216-1 216.5 (50 kHz spacing) |  16  32 |  |  | 4.47 V |  |
|  |  | 1 252-1 252.5 (50 kHz spacing) |  |  16 mW (12.14 dBm) |  10 mW 2.14 dBi |  |  |
|  |  | 1 216.55-1 217 (50 kHz spacing) |  |  |  |  |  |
|  |  | 1 252.5-1 253 (50 kHz spacing) |  |  |  |  |  |
|  |  | 1 216.0125-1 216.5125 (25 kHz spacing) |  16 |  |  |  |  |
|  |  | 1 252.0125-1 252.5125 (25 kHz spacing) |  |  |  |  |  |
|  |  | 1 216.5375-1 216.9875 (25 kHz spacing) |  |  |  |  |  |
|  |  | 1 252.5375-1 252.9875 (25 kHz spacing) |  |  |  |  |  |
| Wireless telephone |  |  |  |  |  |  |  |
| F1D, F1E, F2D, F2E, F3E, F7W, G1D, G1E, G2D, G2E, G7E, G7W, D1D, D1E, D2D, D2E, D3E, D7E or D7W |  | 422.2-422.3 (12.5 kHz spacing) |  8.5 |  16 mW (12.14 dBm) |  10 mW 2.14 dBi | 7 V |  |
|  |  | 421.8125-421.9125 (12.5 kHz spacing) |  |  |  |  |  |
|  |  | 440.2625-440.3625 (12.5 kHz spacing) |  |  |  |  |  |
|  |  | 422.05-422.1875 (12.5 kHz spacing) |  |  |  |  |  |
|  |  | 421.575-421.8 (12.5 kHz spacing) |  |  |  |  |  |
|  |  | 440.025-440.25 (12.5 kHz spacing) |  |  |  |  |  |
| Radio pager |  |  |  |  |  |  |  |
| F1B, F2B, F3E, G1B or G2B |  | 429.75 429.7625 |  8.5 |  16 mW (12.14 dBm) |  10 mW 2.14 dBi | 7 V |  |
|  |  | 429.775 429.7875 429.8 |  |  |  |  |  |
| Radio microphone |  |  |  |  |  |  |  |
| F1D,F1E,F2D,F3E,F7D,F7E,F7W,F8E,F8W,F9W,D1D,D1E,D7D,D7E,D7W,G1D,G1E,G7D,G7E,G7W or N0N |  | 806.125-809.75 (125 kHz spacing) | Frequency modulation (except for Frequency shift keying)  110 Frequency modulation (limited to Frequency shift keying), Phase modulation or Quadrature amplitude modulation  192 |  16 mW (12.14 dBm) |  10 mW 2.14 dBi | Not required |  |
| F3E, F8W, F2D or F9W |  | 322.025-322.15 (25 kHz spacing) |  30 |  1.6 mW (2.14 dBm) |  1 mW 2.14 dBi | Not required |  |
|  |  | 322.25-322.4 (25 kHz spacing) |  |  |  |  |  |
| Medical telemeter |  |  |  |  |  |  |  |
| F1D, F2D, F3D, F7D, F8D or F9D |  | 420.05-421.0375, 424.4875-425.975, 429.25-429.7375, 440.5625-441.55, 444.5125-445.5 and 448.675-449.6625 (12.5 kHz spacing) |  8.5 |  1.6 mW (2.14 dBm) |  1 mW 2.14 dBi | Not required |  |
| F7D, F8D or F9D |  | 420.0625-421.0125, 424.5-425.95, 429.2625-429.7125, 440.575-441.525, 444.525-445.475, 448.6875-449.6375 (25 kHz spacing) |  8.5  16 |  |  |  |  |
| F7D, F8D, F9D or G7D |  | 420.075-420.975, 424.5125-425.9125, 429.275-429.675, 440.5875-441.4875, 444.5375-445.4375, 448.7-449.6 (50 kHz spacing) |  16  32 |  |  |  |  |
| F7D, F8D, F9D or G7D |  | 420.1-420.9, 424.5375-425.8375, 429.3-429.6, 440.6125-441.4125, 444.5625-445.3625, 448.725-449.525, (100 kHz spacing) |  32  64 |  |  |  |  |
| F7D, F8D, F9D or G7D |  | 420.3, 420.8, 424.7375, 425.2375, 425.7375, 429.5, 440.8125, 441.3125, 444.7625, 445.2625, 448.925, 449.425 |  64  320 |  16 mW (12.14 dBm) |  10 mW 2.14 dBi |  |  |
| Hearing aid |  |  |  |  |  |  |  |
| F3E or F8W |  | 75.2125-75.5875 (12.5 kHz spacing) |  20 |  16 mW (12.14 dBm) |  10 mW 2.14 dBi | Not required |  |
| F3E or F8W |  | 75.225-75.575 (25 kHz spacing) |  20  30 |  |  |  |  |
| F3E or F8W |  | 75.2625-75.5125 (62.5 kHz spacing) |  30  80 |  |  |  |  |
| PHS (land mobile station) |  |  |  |  |  |  |  |
| G1C, G1D, G1E, G1F, G1X, G1W, G7C, G7D, G7E, G7F, G7X or G7W |  | 1 884.65-1 919.45 | 1 884.65-1 893.5 MHz 288 1 893.5-1 919.45 MHz: 884 |  25 mW (14 dBm) |  10 mW 4 dBi | Not required |  |
| Wireless LAN |  |  |  |  |  |  |  |
| SS (spread spectrum) (DS (direct sequence), FH (frequency hopping), FH/DS), OFDM or others |  | 2 400-2 483.5 | FH or FH/DS:  85.5 MHz Others:  26 MHz | FH or FH/DS:  4.9 mW/MHz (6.9 dBm/MHz) DS or OFDM:  16 mW/MHz (12.14 dBm/MHz) Others:  16 mW (12.14 dBm/MHz) | FH or FH/DS:  3 mW/MHz DS or OFDM:  10 mW/MHz Others:  10 mW 4 dBi  | Not required |  |
| SS (DS, FH or FH/DS) |  | 2 471-2 497 |  26 MHz |  16 mW (12.14 dBm/MHz) |  10 mW/MHz  2.14 dBi | Not required |  |
| SS (DS), OFDM or others |  | 5 150-5 350 (indoor use) | DS or Others  18 MHz OFDM:  19 MHz |  10 mW/MHz | DS or OFDM:  10 mW/MHz Others:  10 mW | 100 mV/m DFS and TPC are required in 5 250- 5 350 MHz |  |
|  |  |  | OFDM:  38 MHz | OFDM:  5 mW/MHz | OFDM:  5 mW/MHz |  |  |
| SS (DS), OFDM or others |  | 5 470-5 725 |  19.7 MHz |  50 mW/MHz | DS or OFDM:  10 mW/MHz Others:  10 mW | 100 mV/m DFS and TPC are required |  |
|  |  |  | OFDM:  38 MHz | OFDM:  25 mW/MHz | OFDM:  5 mW/MHz |  |  |
| Millimetre-wave radar |  |  |  |  |  |  |  |
| – |  | 60.5 GHz 76.5 GHz |  500 MHz | 100 W 50 dBm |  10 mW  40 dBi | Not required |  |
| Radio stations for cordless phones |  |  |  |  |  |  |  |
| F1D, F2A, F2B, F2C, F2D, F2N, F2X or F3E |  | 253.8625-254.9625 (12.5 kHz spacing) 380.2125-381.3125 (12.5 kHz spacing) |  8.5 |  10 mW (10 dBm) | – | 2 V |  |
| Mobile station identification |  |  |  |  |  |  |  |
| N0N, A1D, AXN, F1D, F2D or G1D |  | 2 440 (2 427-2 453) 2 450 (2 434.25-2 465.75) 2 455 (2 439.25-2 470.75) |  5.5 |  1 W (30 dBm) |  10 mW  20 dBi | Not required |  |
| Radio stations for low-power security systems |  |  |  |  |  |  |  |
| F1D, F2D or G1D |  | 426.25-426.8375 (12.5 kHz spacing) |  8.5 |  10 mW (10 dBm) | – | Not required |  |
|  |  | 426.2625-426.8375 (25 kHz spacing) |  8.5  16 |  |  |  |  |
| Radio stations for digital cordless phones |  |  |  |  |  |  |  |
| G1C, G1D, G1E, G1F, G1X, G1W, G7C, G7D, G7E, G7F, G1X or G7W |  | 1 893.65-1 905.95 (300 kHz spacing) |  288 |  25 mW (14 dBm) |  10 mW  4 dBi | 159 V |  |
| Mobile land stations for toll-road automatic toll collection systems |  |  |  |  |  |  |  |
| A1D |  | 5.835 GHz 5.845 GHz |  8 MHz |  100 mW (20 dBm) |  10 mW  10 dBi | Not required |  |
| RF identification (RFID) systems |  |  |  |  |  |  |  |
| － |  | 433.67-434.17(1) | 500 kHz (Interrogator) 200 kHz (Active tag) |  0.4 mW (–4 dBm) (2) (Interrogator)  1 mW (0 dBm) (Active tag) | － | Not required |  |
| N0N, A1D, AXN, H1D, R1D, J1D, F1D, F2D or G1D |  | 952-954 | *n kHz(3) (n = 1-9) | W (36 dBm) |  1 W (4)  6 dBi | –74 dBm |  |
|  |  | 952-955 | *n kHz(3) (n = 1-3) |  mW (13 dBm) |  10 mW  3 dBi | –64 dBm |  |
|  |  | 950.9-955.7 | *n kHz(3) (n = 1-3) |  mW (3 dBm) |  1 mW  3 dBi | –75 dBm(5) |  |
|  |  | 954.1-954.9 | *n kHz(3) (n = 1-3) |  mW (13dBm) |  10 mW  3 dBi | –75 dBm |  |
| N0N, A1D, AXN, F1D, F2D or G1D |  | 2 427-2 470.75 | FH:   43.75 MHz DS:  5.5 MHz |  (44.77 dBμ) |  300 mW(4)  20 dBi | Not required |  |
|  |  |  | 5.5 MHz |  (30 dBμ) |  10 mW  20 dBi |  |  |
| Medical implant communication systems |  |  |  |  |  |  |  |
| A1D, F1D or G1D |  | 402-405 |  300 kHz |  25 W (–16 dBm) |  | 10LogB –150 + GdB (with 1 mW regarded as 0 dB) (6) |  |
|  |  | 403.5-403.8 |  | mW (–40 dBm) |  | Not required |  |
| Sensors for detecting or measuring mobile objects |  |  |  |  |  |  |  |
| － |  | 10.525 GHz |  40 MHz |  2.5 W (34 dBm) |  10 mW  24 dBi |  |  |
|  |  | 24.15 GHz |  76 MHz |  |  |  |  |
| Quasi-millimeter-wave communication systems |  |  |  |  |  |  |  |
| OFDM or others |  | 24.77-25.23 GHz 27.02-27.46 GHz |  18 MHz |  100 mW/MHz (20 dBm/MHz) |  10 mW/MHz  10 dBi | 460 mW/m |  |
| OFDM: orthogonal frequency division multiplexing PSK: phase shift keying (1) International logistics only. (2) Power level (e.i.r.p.) from interrogators is limited in less than 0.1 mW (–10 dBm) when sending a signal for the start of switching active tags on. (3) n: n represents the number of unit radio channels which are simultaneously used. (4) A registration is required for establishment of this radio station though a license is not required. (5) Not required when transmission time is limited within 3.6 seconds per hour. (6) B is the maximum radiation bandwidth in the communication state (which refers to the bandwidth in which the radio equipment in a living body or the radio control equipment outside the living body radiates and is the larger of either of the upper limit and the lower limit frequency width (Hz) at which the attenuation from the maximum value of the radiation power during the maximum modulation becomes 20 dB). G is the absolute gain of the receiving antenna. |  |  |  |  |  |  |  |

Technical Regulations in Republic of Korea

| Technical Regulations for Short Range Radiocommunication Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Num | Typical Application Type | Authorised Frequency Bands / Frequencies | Maximum Field Strength / RF Output power | Remarks |
|  | Citizen band transceiver | 26.965, 26.975, 26.985, 27.005, 27.015, 27.025, 27.035, 27.055, 27.065, 27.075, 27.085, 27.105, 27.115, 27.125, 27.135, 27.155, 27.165, 27.175, 27.185, 27.205, 27.215, 27.225, 27.235, 27.245, 27.255, 27.265, 27.275, 27.285, 27.295, 27.305, 27.315, 27.325, 27.335, 27.345, 27.355, 27.365, 27.375, 27.385, 27.395 and 27.405 MHz | 3 W | The maximum OBW is 6 kHz for double side band and 3 kHz for single side band emission. The antenna should be whip type, and the limit of antenna length is 1 m for portable type, 3 m for built-in vehicle type (total height should not be higher than 4.5 m) and 6 m for fixed type. The channel 27.065 MHz is designated for emergency communication (such as fire alarm). The channel 27.065 MHz is designated for meteorological, medical, traffic guide. |
|  | Citizen band transceiver | 448.7375, 448.7500,…, 448.9250 MHz and 449.1500, 449.1625,…, 449.2625 MHz | 500 mW (e.r.p.) | Total number of channel is 26. The channel 448.7375 MHz is designated for channel control. The maximum OBW is 8.5 kHz. |
|  | Citizen band transceiver | 424.1375 (449.1375) 424.1500 (449.1500) 424.1625 (449.1625) 424.1750 (449.1750) 424.1875 (449.1875) 424.2000 (449.2000) 424.2125 (449.2125) 424.2250 (449.2250) 424.2375 (449.2375) 424.2500 (449.2500) and 424.2625 (449.2625) MHz | 500 mW (e.r.p.) | The channels 424.1375 (449.1375) MHz are designated for channel control. The maximum OBW is 8.5 kHz. |
| - 1 | Low power devices (LPD) | 0 - 322 MHz 322 MHz - 10 GHz 10 GHz - 150 GHz Above 150 GHz | 500 uV/m @ 3m 35 uV/m @ 3m 3.5f uV/m @ 3m (1) 500 uV/m @ 3m | The measured value for the frequency of less than 15 MHz should be multiplied by the near field measurement compensation factor (6π/λ), where λ is wavelength in meter. (1) f : Frequency (GHz). |
|  | Radio Controller for model automobile and model shipcraft | 26.995, 27.045,…, 27.195 MHz (5 channels with 50 kHz space), 40.255, 40.275,…, 40.495 MHz (13 channels with 20 kHz space), 75.630, 76.650, ..., 75.790 MHz (9 channels with 20 kHz space) | 10 mV/m @10m |  |
|  | Radio controller for model aerocraft | 40.715, 40.735,..., 40.995 MHz (15 channels with 20 kHz space), 72.630, 72.650,…, 72.990 MHz (19 channels with 20 kHz space) | 10 mV/m @10m |  |
|  | Radio controller for toy, security alarm or telecommand | 13.552 - 13.568 MHz 26.958 - 27.282 MHz 40.656 - 40.704 MHz | 10 mV/m @10m |  |
|  | LPD for data transmission | 173.0250, 173.0375,…, 173.2750 MHz (21 channels with 12.5kHz space) | 5 mW (e.r.p.) | The maximum OBW is 8.5 kHz. |
|  | LPD for data transmission | 173.6250, 173.6375,…173.7875 MHz (14 channels with 12.5kHz space) | 10 mW (e.r.p.) | The maximum OBW is 8.5 kHz. |
|  | LPD for data transmission | 219.000 (224.000), 219.025 (224.025) 219.050 (224.050), 219.075 (224.075) 219.100 (224.100), 219.125 (224.125) (6 pair channels with 25kHz space) | 10 mW (e.r.p.) | The frequencies of 219.000 (224.000) MHz are for channel control. The maximum OBW is 16 kHz. Frequencies in ( ) are for duplex communication. |
|  | LPD for data transmission | 311.0125, 311.0250,…, 311.1250 MHz (10 channels with 12.5kHz space) | 5 mW (e.r.p.) | The maximum OBW is 8.5 kHz. |
|  | LPD for data transmission | 424.7000, 424.7125,…, 424.9500 MHz (21 channels with 12.5 kHz space) | 10 mW (e.r.p.) | The channel 424.7 MHz is for channel control. The maximum OBW is 8.5 kHz. |
|  | LPD for tire pressure monitoring system (TPMS) | 433.795 ∼ 434.045 MHz | 3 mW (e.r.p.) |  |
|  | LPD for data transmission | 447.6000, 447.6125,…, 447.8500 MHz (21 channels with 12.5 kHz space) | 5 mW (e.r.p.) | The maximum OBW is 8.5 kHz. |
|  | SRD for Data transmission | 447.8625, 447.8750,…, 447.9875 MHz (11 channels with 12.5 kHz space) | 10 mW (e.r.p.) | The maximum OBW is 8.5 kHz. |
|  | LPD for inducement of the visually handicapped | 235.3000, 235.3125, 235.3250, 235.3375 MHz for base station and 358.5000, 358.5125, 358.5250, 358.5375 MHz for mobile station | 10 mW (e.r.p.) | The maximum OBW is 8.5 kHz. |
|  | LPD for security application | 447.2625, 447.2750,…, 447.5625 MHz (25 channels with 12.5 kHz space) | 10 mW (e.r.p.) | The maximum OBW is 8.5 kHz. |
|  | LPD for data transmission or voice radio paging | 219.150, 219.175, 219.200 and 219.225 MHz (4 channels with 25 kHz space) | 10 mW (e.r.p.) | The maximum OBW is 16 kHz. |
|  | LPD for wireless microphone or audio transmission | 72.610 - 73.910 MHz 74.000 - 74.800 MHz 75.620 - 75.790 MHz | 10 mW (e.r.p.) | The maximum OBW is 60 kHz. |
|  | LPD for wireless microphone or audio transmission | 173.020 - 173.280 MHz 217.250 - 220.110 MHz 223.000 - 225.000 MHz 740.000 - 752.000 MHz 928.000 - 930.000 MHz 950.000 - 952.000 MHz | 10 mW (e.r.p.) | The maximum OBW is 200 kHz. |
|  | LPD for Wireless access system including wireless LAN | 5 150 - 5 250 MHz 5 250 - 5 350 MHz 5 470 - 5 650 MHz | 2.5 mW/MHz 10 mW/MHz 10mW/MHz | Nominal antenna gain is 7 dBi. |
|  | LPD for Wireless access system including wireless LAN | 17 705 - 17 715 MHz 17 725 - 17 735 MHz 19 265 - 19 275 MHz 19 285 - 19 295 MHz | 10 mW (e.r.p.) | The maximum OBW is 10 MHz. |
|  | LPD for data communication | 2 400 - 2 483.5 MHz 5 725 - 5 825 MHz | 3 mW/MHz 1) (for FHSS type) 10 mW/MHz 2) (for other spread spectrum type) 10 mW (other type3)) | The nominal antenna gain is 6 dBi (20 dBi for point-to-point application) 1) The peak power of a hopping channel divided by whole hopping frequency band (MHz). 2) 5 mW/MHz in case of OBW 26-40 MHz and 0.1 mW/MHz in case of OBW 40-60 MHz. 3) The maximum OBW is 26 MHz for 2.4 GHz band and 70 MHz for 5.8 GHz band. |
|  | LPD for analog video transmission | 2 410, 2 430, 2 450 and 2 470 MHz | 10 mW | The nominal antenna gain is 6 dBi (20 dBi for point-to-point application) The maximum OBW is 16 MHz. |
|  | LPD for dedicated short range communication | 5 800 and 5 810 MHz | 10 mW (e.r.p.) | The maximum OBW is 8 MHz. Nominal antenna gain is 22 dBi for road side unit and 8 dBi for on-board unit. |
|  | Vehicle identification system | 2.440 (2.427-2.453) 2.445 (2.434-2.465) 2.455 (2.439-2.470) | 300 mW | Nominal antenna gain is 20 dBi. |
|  | RFID | 13.552 - 13.568 | 93.5 dBuV/m @ 10m |  |
|  | RFID | 433.670 - 434.170 MHz | 3.6 mW (e.i.r.p.) |  |
|  | RFID | 908.5 - 914.0 MHz | 1 W | Nominal antenna gain is 6 dBi. |
|  | Car radar | 76 - 77 GHz | 10 mW | 50 dBm (e.i.r.p.) |
|  | Cordless phone | 46.510 (49.695), 46.530 (49.710), 46.550 (49.725), 46.570 (49.740), 46.590 (49.755), 46.610 (49.670), 46.630 (49.845), 46.670 (49.860), 46.710 (49.770), 46.730 (49.875), 46.770 (49.830), 46.830 (49.890), 46.870 (49.930), 46.930 (49.990) and 46.970 (49.970) MHz | 3 mW | The maximum OBW is 16 kHz. |
|  | Cordless phone | 959.0125 (914.0125), 959.0375 (914.0375),…, 959.9875 (914.9875) MHz (40 pair channels with 25kHz space) | 10 mW | Type registration would be granted until 31st December 2008, and manufacturing/ sale would be permitted until 31st December 2013. |
|  | Cordless phone (Digital) | 1786.336 - 1791.520 MHz | 100 mW (e.i.r.p.) | The maximum OBW is 1.728 kHz. |
|  | Cordless phone (Digital) | 2400 – 2483.5 MHz | 3 mW/MHz 1) (for FHSS type) 10 mW/MHz 2) (for other spread spectrum type) 10 mW (other type3)) | The nominal antenna gain is 6 dBi 1) The peak power of a hopping channel divided by whole hopping frequency band (MHz). 2) 5 mW/MHz in case of OBW 26-40 MHz and 0.1 mW/MHz in case of OBW 40-60 MHz. 3) The maximum OBW is 26 MHz. |
|  | UWB device | 3.1 - 4.8 GHz 7.2 - 10.2 GHz | -41.3 dBm/MHz (e.i.r.p) | The minimum 10-dB bandwidth is 450 MHz. Interference mitigation function (DAA, LDC, etc) should be adopted in the band of 3.1 – 4.8 GHz. |
|  | Non-specific SRD | 57 – 64 GHz | 10 mW | Nominal antenna gain is 17 dBi (47 dBi for point-to-point application) |
|  | Medical implant communication system (MICS) | 402∼405 MHz | 25 uW (e.i.r.p.) | Maximum OBW is 300 kHz. |

Technical Regulations in Malaysia

| Technical Regulations for Short Range Radiocommunication Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Num | Typical Application Type | Authorised Frequency Bands / Frequencies | Maximum Field Strength / RF Output power (mW) | Remarks |
|  | Short range communication device | 6.7650 to 6.7950 13.5530 to 13.5670 26.9570 to 27.2830 40.6600 to 40.7000 433.0000 to 435.0000 | ≤100 (EIRP) |  |
|  |  | 2400.0000 to 2500.0000 | ≤500 (EIRP) |  |
|  |  | 5150.0000 to 5250.0000 5250.0000 to 5350.0000 5725.0000 to 5875.0000 24.0000 GHz to 24.2500 GHz 61.0000 GHz to 61.5000 GHz 122.0000 GHz to 123.0000 GHz 244.0000 GHz to 246.0000 GHz | ≤1000 (EIRP) |  |
|  | Personal Radio Service Device | 477.5250 to 477.9875 | ≤500 |  |
|  | Cordless telephone | 46.6100 to 46.9700 49.6100 to 49.9700 | ≤50 (EIRP) |  |
|  |  | 866.0000 to 871.0000 CT2/CT3 freq. Band* | ≤50 (EIRP) |  |
|  |  | 1880.0000 to 1900.0000 2400.0000 to 2483.5000 | ≤100 (EIRP) |  |
|  | Two-way radio pager access device | 279.0000 to 281.0000/ 919.0000 to 923.0000 | ≤1000 |  |
|  | Radio telemetry access device | 162.9750 to 163.1500 | ≤1000 |  |
|  | Infra red device | 187.5000 THz to 420.0000 THz | ≤125 |  |
|  | Remote controlled consumer device - boat, car model/garage door/camera/toy robot, crane, etc | 26.9650 to 27.2750 40.0000 47.0000 49.0000 303.0000 to 320.0000 433.0000 to 435.0000 | ≤50 (EIRP) |  |
|  | Security device - radio detection and alarm | 3.0000 kHz to 195.0000 kHz 228.0063 to 228.9937 303.0000 to 320.0000 400.0000 to 402.0000 433.0000 to 435.0000 868.1000 76.0000 GHz to 77.000GHz | <50 (EIRP) |  |
|  | Wireless microphone system | 26.95728 to 27.28272 40.4350 to 40.9250 87.5000 to 108.000 182.0250 to 182.9750 183.0250 to 183.4750 217.0250 to 217.9750 218.0250 to 218.4750 510.0000 to 798.0000 | <50 (EIRP) |  |
|  | Free space optics device | 193.5484 THz (wavelength of 1550nm) 352.9412 THz (wavelength of 850nm) | ≤650 |  |
|  | Industrial, Scientific and Medical (ISM) device | 6765.0000 kHz to 6795.0000 kHz 13.5530 to 13.5670 26.9570 to 27.2830 40.6600 to 40.7000 2400.0000 to 2500.0000 5725.0000 to 5875.0000 24.0000 GHz to 24.2500 GHz 61.0000 GHz to 61.5000 GHz 122.0000 GHz to 123.0000 GHz 244.0000 GHz to 246.0000 GHz | <500 (EIRP) |  |
| - M | Active Medical Implant | 402.0000 MHz to 405.0000 MHz 9.0000 kHz to 315.0000 kHz | 25 microWatts 30 dB microA/m at 10 m | * planned |
|  | RFID | 13.5530 MHz to 13.5670 MHz 433.0000 MHz to 435.0000 MHz 869.0000 MHz to 870.3750 MHz 919.0000 MHz to 923.0000 MHz 2400.0000 MHz to 2500.0000 MHz | 100 milliWatts 100 milliWatts 500 milliWatts 2 Watts ERP 500 milliWatts | *planned |

Technical Regulations in New Zealand

| Technical Regulations for Short Range Radiocommunication Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Num | Typical Application Type | Authorised Frequency Bands / Frequencies | Maximum Field Strength / RF Output power | Remarks |
|  | Telemetry/Telecommand | 0.009 - 0.03 MHz | maximum permitted field strength is 2400/f(kHz) µV/m measured using an average detector at 300 metres – where f is the centre frequency. |  |
|  | Telemetry/Telecommand | 0.03 - 0.19 MHz | 10 mW eirp |  |
|  | Telemetry/Telecommand | 6.765 - 6.795 MHz | 10 mW eirp |  |
|  | Telemetry/Telecommand | 13.55 - 13.57 MHz | 100 mW eirp |  |
|  | Unrestricted | 26.95 - 27.3 MHz | 1000 mW eirp |  |
|  | Unrestricted | 29.7 – 30 MHz | 100 mW eirp |  |
|  | Unrestricted | 35.5 37.2 MHz | 100 |  |
|  | Unrestricted | 40.66 - 40.7 MHz | 1000 mW eirp |  |
|  | Unrestricted | 40.8 - 41.0 MHz | 100 mW eirp |  |
|  | Auditory Aids | 72 - 72.25 MHz | 100 mW eirp |  |
|  | Unrestricted | 72.25 - 72.50 MHz | 100 mW eirp |  |
|  | Audio senders | 88 - 108 MHz | 0.00002 mW eirp |  |
|  | Unrestricted | 107 - 108 MHz | 25 mW eirp |  |
|  | Unrestricted | 160.1 - 160.6 MHz | 500 mW eirp |  |
|  | Unrestricted | 173 - 174 MHz | 100 mW eirp |  |
|  | Telemetry/Telecommand | 235 - 300 MHz | 1 mW eirp |  |
|  | Telemetry/Telecommand | 300 - 322 MHz | 10 mW eirp |  |
|  | Biomedical Telemetry | 402 - 406 MHz | 0.025 mW eirp | the maximum permitted duty cycle is 0.1% |
|  | Telemetry/Telecommand | 433.05 - 434.79 MHz | 25 mW eirp |  |
|  | Biomedical Telemetry | 444 - 444.925 MHz | 25 mW eirp |  |
|  | Unrestricted | 458.54 - 458.61 MHz | 500 mW eirp |  |
|  | Unrestricted | 466.80 - 466.85 MHz | 500 mW eirp |  |
|  | Biomedical Telemetry | 470 - 470.5 MHz | 100 mW eirp |  |
|  | Unrestricted | 471 - 471.5 MHz | 100 mW eirp |  |
|  | Audio/Video Senders | 614 - 646 MHz | 25 mW eirp |  |
|  | Unrestricted | 819 - 824 MHz | 100 mW eirp |  |
|  | Unrestricted | 864 - 868 MHz | 1000 mW eirp | may operate with gain antennas provided the peak power does not exceed 4 watts e.i.r.p. |
|  | Telemetry/Telecommand (refer Note 3) | 869.2 - 869.25 MHz | 10 mW eirp |  |
|  | Telemetry/Telecommand | 915 - 921 MHz | 3 mW eirp |  |
|  | Unrestricted | 921 -929 MHz | 1000 mW eirp |  |
|  | Unrestricted | 2.4 - 2.4835 GHz | 1000 mW eirp | may operate with gain antennas provided the peak power does not exceed 4 watts e.i.r.p. |
|  | Radiolocation | 2.9 - 3.4 GHz | 100 mW eirp |  |
|  | Wireless LAN - | 5.15 - 5.25 GHz | 200 mW eirp | indoor use - the maximum permitted power density is 10 mW/MHz e.i.r.p. or equivalently 0.25 mW/25 kHz e.i.r.p. |
|  | Wireless LAN | 5.25 - 5.35 GHz | 1000 mW eirp | Indoor-Only Systems: In the band 5250 to 5350 MHz the maximum permitted mean power is 200 mW e.i.r.p. and the maximum permitted mean power density is 10 mW/MHz e.i.r.p., provided Dynamic Frequency Selection and Transmitter Power Control are implemented. If transmitter power control is not in use, then the e.i.r.p. values shall be reduced by 3 dB. Indoor and Outdoor Systems: In the band 5250 to 5350 MHz, the maximum permitted mean power is 1watt e.i.r.p. and the maximum permitted mean power density is 50 mW/MHz, provided Dynamic Frequency Selection and Transmitter Power Control are implemented in conjunction with the following vertical radiation angle mask where q is the angle above the local horizontal plane (of the Earth): Maximum permitted mean power density Elevation angle above horizontal –13 dB(W/MHz) for 0° <= θ < 8° -13 - 0.716(θ-8) dB(W/MHz) for 8° <= θ < 40° –35.9 – 1.22(θ-40) dB(W/MHz) for 40° <= θ <= 45° -42 dB(W/MHz) for 45° < θ |
|  | Wireless LAN | 5.47 - 5.725 GHz | 1000 mW eirp | the maximum transmitter power is 250 mW with a maximum permitted mean power of 1 watt e.i.r.p. and a maximum permitted mean power density of 50 mW/MHz e.i.r.p., provided Dynamic Frequency Selection and Transmitter Power Control are implemented. If transmitter power control is not in use, then the maximum permitted mean power shall be reduced by 3 dB. |
|  | Radiolocation | 5.47 - 5.725 GHz | 100 mW eirp |  |
|  | Unrestricted (refer Note 2) | 5.725 - 5.875 GHz | 1000 mW eirp |  |
|  | Road Transport and Traffic Telematics | 5.725 - 5.875 GHz | 2000 mW eirp |  |
|  | Radiolocation | 8.5 - 10 GHz | 100 mW eirp |  |
|  | Radiolocation - radar systems only | 10 - 10.6 GHz | 25 mW eirp |  |
|  | Radiolocation | 15.7 - 17.3 GHz | 100 mW eirp |  |
|  | Unrestricted | 24 - 24.25 GHz | 1000 mW eirp |  |
|  | Radiolocation | 33.4 - 36 GHz | 100 mW eirp |  |
|  | Field Disturbance Sensors | 46.7 - 46.9 GHz | 100 mW eirp |  |
|  | Fixed point-to-point links | 57 - 64 GHz | 20000 mW eirp | the average power density of any emission, measured during the transmit interval shall not exceed 9 µW/cm2 at a distance of 3 metres and the peak power density of any emission shall not exceed 18 µW/cm2 at a distance of 3 metres. In the band 57 – 64 GHz, the peak total transmitter power shall not exceed 500 mW. In the band 57 – 64 GHz, for emissions of bandwidths less than 100 MHz the transmitter peak power must be limited to 500 mW x (bandwidth (MHz) / 100 (MHz)). |
|  | Radiolocation | 59 - 64 GHz | 100 mW eirp |  |
|  | Field Disturbance Sensors | 76 - 77 GHz | 1000 mW eirp |  |
|  | Unrestricted | 122 - 123 GHz | 1000 mW eirp |  |
|  | Unrestricted | 244 - 246 GHz | 1000 mW eirp |  |

Technical Regulations in Philippines

| Technical Regulations for Short Range Radiocommunication Devices |  |  |  |  |
| --- | --- | --- | --- | --- |
| Num | Typical Application Type | Authorised Frequency Bands / Frequencies | Maximum Field Strength / RF Output power | Remarks |
|  | Ultra Low Power Active MICS | 9-315 KHz | 30 dBuA/m @ 10m | *individual transmitters may combine adjacent channels for increased bandwidth up to 300 KHz. |
|  |  | 4042-405 MHz * | 25 uW (e.r.p.) |  |
|  | Biomedical Devices | 40.66-40.70 MHz | 1000 uV/m @ 3m |  |
|  | Alarms | 868.6-868.7 MHz | 10 mW (e.r.p.) |  |
|  |  | 869.2-869.25 MHz | 10 mW (e.r.p.) |  |
|  |  | 869.25-869.3 MHz | 10 mW (e.r.p.) |  |
|  |  | 869.65-869.7 MHz | 25 mW (e.r.p.) |  |
|  | Equipment For Detecting Movement and Alert Alarms | 2400-2483.5 MHz | 25 mW (e.i.r.p.) |  |
|  |  | 9200-9500 MHz | 25 mW (e.i.r.p.) |  |
|  |  | 9500-9975 MHz | 25 mW (e.i.r.p.) |  |
|  |  | 13.4-14.0 GHz | 25 mW (e.i.r.p.) |  |
|  |  | 24.05-24.25 GHz | 100 mW (e.i.r.p.) |  |
|  | Equipment For Detecting Movement and Alert Alarms | 2400-2483.5 MHz | 25 mW (e.i.r.p.) |  |
|  |  | 9200-9500 MHz | 25 mW (e.i.r.p.) |  |
|  |  | 9500-9975 MHz | 25 mW (e.i.r.p.) |  |
|  |  | 13.4-14.0 GHz | 25 mW (e.i.r.p.) |  |
|  |  | 24.05-24.25 GHz | 100 mW (e.i.r.p.) |  |
|  | Inductive Applications | 9-59.750 KHz | 72 dBuA/m @ 10m |  |
|  |  | 59.750-60.250 KHz | 42 dBuA/m @ 10m |  |
|  |  | 60.250-70 KHz | 69 dBuA/m @ 10m |  |
|  |  | 70-119 KHz | 42 dBuA/m @ 10m |  |
|  |  | 119-135 KHz | 66 dBuA/m @ 10m |  |
|  |  | 135-140 KHz | 42 dBuA/m @ 10m |  |
|  |  | 140-148.5 KHz | 37.7 dBuA/m @ 10m |  |
|  |  | 3155-3400 KHz | 13.5 dBuA/m @ 10m |  |
|  |  | 6765-6795 KHz | 42 dBuA/m @ 10m |  |
|  |  | 7400-8800 KHz | 9 dBuA/m @ 10m |  |
|  |  | 13.553-13.567 MHz | 42 dBuA/m @ 10m |  |
|  |  | 26.957-27.283 MHz | 42 dBuA/m @ 10m |  |
|  |  | 10.2-11 MHz | 9 dBuA/m @ 10m |  |
|  | Non-specific Short Range Devices, Telemetry, Telecommand, Alarms, Data in General and Other Similar Applications | 6765-6795 KHz | 42 dBuA/m @ 10m |  |
|  |  | 13.553-13.567 MHz | 42 dBuA/m @ 10m |  |
|  |  | 26.957-27.283 MHz | 10 mW erp / 42 dBuA/m @ 10m |  |
|  |  | 40.660-40.700 MHz | 10 mW (e.r.p.) |  |
|  |  | 138.2-138.45 MHz | 10 mW (e.r.p.) |  |
|  |  | 315 MHz | 10 mW (e.r.p.) |  |
|  |  | 433.050-434.790 MHz | 10 mW (e.r.p.) |  |
|  |  | 868.000-868.600 MHz | 25 mW (e.r.p.) |  |
|  |  | 868.700-869.200 MHz | 25 mW (e.r.p.) |  |
|  |  | 869.3-869.4 MHz | 25 mW (e.r.p.) |  |
|  |  | 869.700-870.000 MHz | 5 mW (e.r.p.) |  |
|  |  | 2400-2483.5 MHz | 10 mW (e.i.r.p.) |  |
|  |  | 5725-5875 MHz | 25 mW (e.i.r.p.) |  |
|  |  | 24.00-24.25 GHz | 100 mW (e.i.r.p.) |  |
|  |  | 61.0-61.5 GHz | 100 mW (e.i.r.p.) |  |
|  |  | 122-123 GHz | 100 mW (e.i.r.p.) |  |
|  |  | 244-246 GHz | 100 mW (e.i.r.p.) |  |
|  | Road Transport and Traffic Telematics | 5795-5805 MHz* | 2W (e.i.r.p.) | * Individual license required |
|  |  | 63-64 GHz | 8W (e.i.r.p.) |  |
|  |  | 76-77 GHz | 55 dBm peak |  |
|  | Wireless Audio Applications | 72.0-73.0 MHz * | 80 mV/m at 3m (field strength) | * for auditory assistance device only. In case of analogue systems, the maximum occupied bandwidth should not exceed 300 KHz |
|  |  | 75.4-76.0 MHz * | 80 mV/m at 3m (field strength) |  |
|  |  | 863-865 MHz | 10 mW (e.r.p.) |  |
|  |  | 864.8-865.0 MHz | 10 mW (e.r.p.) |  |
|  | Wireless Microphones | 29.7-47.0 MHz | 2 mW (e.r.p.) | 50 mW restricted to for body worn microphones |
|  |  | 173.965-174.015 MHz | 10 mW (e.r.p.) |  |
|  |  | 174-216 MHz | 10 mW (e.r.p.)/50 mW (e.r.p) |  |
|  |  | 470-862 MHz | 10 mW (e.r.p.)/50 mW (e.r.p) |  |
|  |  | 863-865 MHz | 10 mW (e.r.p.) |  |
|  |  | 1785-1800 MHz | 10 mW (e.i.r.p.)/50 mW (e.i.r.p) |  |
|  | Wireless Video Transmitter | 630-710 MHz | 76 dBuV/m at 3m 5-8 MHz |  |
|  |  | 2400-2483.5 MHz (Narrowband) | 100 mW (e.i.r.p.) |  |

Technical Regulations in Singapore

Technical Regulations for Short Range Radiocommunication Devices

| Num | Typical Application Types | Authorised Frequency Bands / Frequencies | Maximum Field Strength / RF Output power | Transmitter Spurious Emissions | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Induction loop system / RFID | 16 – 150 kHz | ≤ 66 dBμA/m @ 3m | ≥ 32 dB below carrier at 3 m or EN 300 224-1 |  |
|  |  | 150 – 5000 kHz | ≤ 13.5 dBμA/m @ 10m |  |  |
|  |  | 6765 – 6795 kHz | ≤ 42 dBμA/m @ 10m |  |  |
|  |  | 7400 – 8800 kHz | ≤ 9 dBμA/m @ 10m |  |  |
| 2 | Radio detection, alarm system | 0.016 – 0.150 MHz | ≤ 100 dBμV/m @ 3m | ≥ 32 dB below carrier at 3 m or EN 300 330-1 |  |
| 3 |  | 13.553 – 13.567 MHz | ≤ 94 dBμV/m @ 10m |  |  |
| 4 |  | 146.35 – 146.50 MHz 240.15 – 240.30 MHz 300.00 – 300.30 MHz 312.00 – 316.00 MHz 444.40 – 444.80 MHz | ≤ 100 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m or EN 300 220-1 |  |
| 5 | Wireless microphone | 0.51 – 1.60 MHz | ≤ 57 dBμV/m @ 3m |  |  |
| 6 |  | 40.66 – 40.70 MHz | ≤ 65 dBμV/m @ 10m |  |  |
| 7 |  | 88.00 – 108.00 MHz | ≤ 60 dBμV/m @ 10m |  |  |
| 8 |  | 470.00 – 806.00 MHz | ≤ 10 mW (e.r.p.) |  |  |
| 9 | Wireless microphone, Hearing/Audio assistance aids | 169.40 – 175.00 MHz | ≤ 500 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m or EN 300 220-1 |  |
|  |  | 180.00 – 200.00 MHz 487.00 – 507.00 MHz | ≤ 112 dBμV/m @ 10m |  |  |
| 10 | Remote controls of garage door, cameras, toys and miscellaneous devices | 26.96 – 27.28 MHz | ≤ 100 mW (e.r.p.) Note 1 | ≥ 32 dB below carrier at 3 m or EN 300 220-1 |  |
|  |  | 34.995 – 35.225 MHz | ≤ 100 mW (e.r.p.) |  |  |
|  |  | 40.665 – 40.695 MHz | ≤ 500 mW (e.r.p.) |  |  |
|  |  | 40.77 – 40.83 MHz |  |  |  |
|  |  | 72.13 – 72.21 MHz |  |  |  |
| 11 | Remote controls of aircraft and glider models, telemetry, detection and alarm systems | 26.96 – 27.28 MHz 29.70 – 30.00 MHz | ≤ 500 mW (e.r.p.) |  |  |
| 12 | Remote control of cranes and loading arms | 170.275 MHz 170.375 MHz 173.575 MHz 173.675 MHz 451.750 MHz 452.000 MHz 452.050 MHz 452.325 MHz | ≤ 1000 mW (e.r.p.) |  | Operating under these provisions shall be approved on an exceptional basis. |
| 13 | On-site radio paging system | 26.96 – 27.28 MHz 40.66 – 40.70 MHz | ≤ 3000 mW (e.r.p.) Note 1 | ≥ 32 dB below carrier at 3 m; EN 300 135-1; EN 300 433-1 or EN 300 224-1 | Operating under these provisions shall be approved on an exceptional basis. |
| 14 |  | 151.125 MHz 151.150 MHz | ≤ 3000 mW (e.r.p.) | ≥ 60 dB below carrier over 100 kHz to 2000 MHz or EN 300 224-1 |  |
| 15 | Medical and Biological telemetry | 40.50 – 41.00 MHz | ≤ 0.01 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m or EN 300 220-1 |  |
|  |  | 216.00 – 217.00 MHz | > 25 μW to ≤ 100 mW (e.r.p.) |  |  |
|  |  | 454.00 – 454.50 MHz | ≤ 2 mW (e.r.p.) |  |  |
| 16 |  | 1427.00 – 1432.00 MHz | > 25 μW to ≤ 100 mW (e.r.p.) | FCC Part 15 or EN 300 440-1 |  |
| 17 |  | All frequencies | ≤ 25 μW (e.r.p.) | FCC Part 15; EN 300 220-1; EN 300 330-1; or EN 300 440-1 |  |
| 18 | Wireless modem, data communication system | 72.080 MHz 72.200 MHz 72.400 MHz 72.600 MHz 158.275/162.875 MHz 158.325/162.925 MHz 453.7250/458.7250 MHz 453.7375/458.7375 MHz 453.7500/458.7500 MHz 453.7625/458.7625 MHz | ≤ 1000 mW (e.r.p.) Note 1 | ≥ 43 dB below carrier over 100 kHz to 2000 MHz; EN 300 390-1 or EN 300 113-1 |  |
| 19 | Short range radar systems such as automatic cruise control and collision warning systems for vehicle | 76 – 77 GHz | ≤ 37 dBm (e.r.p.) when vehicle is in motion ≤ 23.5 dBm (e.r.p.) when vehicle is stationary | FCC Part 15 § 15.253 (c) or EN 301 091 |  |
| 20 | Radio telemetry, telecommand system | 433.05 – 434.79 MHz | ≤ 10 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m or EN 300 220-1 |  |
| 21 | Radio Telemetry, Telecommand, RFID system | 866 – 869 MHz 920 – 925 MHz | ≤ 500 mW (e.r.p.) Note 1 | ≥ 32 dB below carrier at 3 m; EN 300 220-1 or EN 302 208 |  |
| 22 | Radio Frequency Identification (RFID) systems | 920 – 925 MHz | > 500 mW (e.r.p.) ≤ 2000 mW (e.r.p.) | ≥ 32 dB below carrier at 3 m; EN 300 220-1 or EN 302 208 | Only RFID systems operating in the 920 -925 MHz frequency band shall be allowed to transmit between 500 mW and 2000 mW (e.r.p.), and approved on an exceptional basis. |
| 23 | Wireless video transmitter and other SRD applications | 2.4000 – 2.4835 GHz | ≤ 100 mW (e.i.r.p.) Note 2 | FCC Part 15 § 15.209; § 15.249 (d) or EN 300 440-1 |  |
| 24 |  | 10.50 – 10.55 GHz | ≤ 117 dBμV/m @ 10m |  |  |
| 25 |  | 24.00 – 24.25 GHz | ≤ 100 mW (e.i.r.p.) |  | Radar gun devices are not allowed to operate. |

| Num | Typical Application Types | Authorised Frequency Bands / Frequencies | Maximum Field Strength / RF Output power | Transmitter Spurious Emissions | Remarks |
| --- | --- | --- | --- | --- | --- |
| 26 | Bluetooth | 2.4000 – 2.4835 GHz | ≤ 100 mW (e.i.r.p.) Note 2 | FCC Part 15 § 15.209; or EN 300 328 |  |
| 27 | Wireless LAN only | 2.4000 – 2.4835 GHz | ≤ 200 mW (e.i.r.p.) |  | WLAN for non-localised operations shall be approved on an exceptional basis. |
| 28 | SRD applications | 5.725 – 5.850 GHz | ≤ 100 mW (e.i.r.p.) | FCC Part 15 § 15.209 |  |
| 29 | Wireless LAN and Broad Band Access (WBA) only | 5.725 – 5.850 GHz | ≤ 1000 mW (e.i.r.p.) |  | Non-localised operations shall be approved on an exceptional basis. |
| 30 |  | 5.725 – 5.850 GHz | > 1000 mW (e.i.r.p.) ≤ 4000 mW (e.i.r.p.) |  | Operating under this provision shall be approved on an exceptional basis. |
| 31 | Wireless LAN | 5.150 – 5.350 GHz | > 100 mW (e.i.r.p.) Note 2 ≤ 200 mW (e.i.r.p.) | FCC Part 15 § 15.407 (b) or EN 301 893 | WLAN operating in 5.250 – 5.350 GHz under this provision shall employ Dynamic Frequency Selection (DFS) mechanism and implement Transmit Power Control (TPC). Non-localised operations shall be approved on an exceptional basis. |
| 32 | Wireless LAN | 5.150 – 5.350 GHz | ≤ 100 mW (e.i.r.p.) | FCC Part 15 § 15.407 (b) or EN 301 893 | WLAN operating under this provision shall implement DFS function in the frequency range 5.250 – 5.350 GHz. Non-localised operations shall be approved on an exceptional basis. |

Technical Regulations in Vietnam

The MIC’s Decision 47/2006/QD-BBCVT of 29/11/2006 includes individual technical requirement for each type of SRDs. The common requirements are presented in the table below:

| TECHNICAL REQUIREMENTS FOR SHORT RANGE RADIOCOMMUNICATION DEVICES |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Frequency band (MHz) | Emission (Max Power) | Spurious Emission (Max power or min deterioration) | Type of devices or applications |
|  | A | B | C | D |
|  | 13.553 ÷ 13.567 | ≤ 4,5 mW ERP | Detail in relevant annex | Radio alarm and detection systems |
|  |  |  |  | RFID |
|  |  |  |  | Other applications |
|  | 26.957 ÷ 27.283 | ≤ 100 mW ERP | ≥ 40 dBc at output of the transmiter | Radio remote control |
|  |  |  |  | Other applications |
|  | 29.70 ÷ 30.00 | ≤ 100 mW ERP | ≥ 40 dBc at output of the transmiter | Radio remote control |
|  |  |  |  | Radio alarm and detection systems |
|  | 40.66 ÷ 40.70 | ≤ 100 mW ERP | ≥ 40 dBc at output of the transmiter | Radio Microphne and Headphone |
|  |  |  |  | Radio remote control |
|  |  |  |  | Other applications |
|  | 43.71 ÷ 44.00 46.60 ÷ 46.98 48.75 ÷ 49.51 49.66 ÷ 50.00 | ≤ 183 W ERP | ≥ 32 dBc at 3m | Cordless Telephone |
|  | 72.00 ÷ 72.99 | ≤ 1 W ERP | ≥ 40 dBc at output of the transmiter | Radio remote control for aircraft model |
|  | 88 ÷ 108 | ≤ 3 W ERP | ≥ 32 dBc at 3m | Radio Microphone and Headphone |
|  | 146.35 ÷ 146.50 | ≤ 100 mW ERP | ≥ 40 dBc at output of the transmiter | Radio alarm and detection systems |
|  | 182.025 ÷ 182.975 | ≤ 30 mW ERP | ≥ 40 dBc at output of the transmiter | Radio Microphone and Headphone |
|  | 217.025 ÷ 217.975 | ≤ 30 mW ERP | ≥ 40 dBc at output of the transmiter | Radio Microphone and Headphone |
|  | 218.025 ÷ 218.475 | ≤ 30 mW ERP | ≥ 40 dBc at output of the transmiter | Radio Microphone and Headphone |
|  | 240.15 ÷ 240.30 | ≤ 100 mW ERP | ≥ 40 dBc at output of the transmiter | Radio alarm and detection systems |
|  | 300.00 ÷ 300.33 | ≤ 100 mW ERP | ≥ 40 dBc at output of the transmiter | Radio alarm and detection systems |
|  | 314.7 ÷ 315.0 | ≤ 100 mW ERP | ≥ 40 dBc at output of the transmiter | Radio alarm and detection systems |
|  | 402 ÷ 405 | ≤ 25 W ERP | Detail in relevant annex | MICS |
|  | 403.5 ÷ 403.8 | ≤ 100 nW ERP |  | MITS |
|  | 433.05 ÷ 434.79 | ≤ 10 mW ERP | ≥ 40 dBc at output of the transmiter | RFID |
|  |  |  |  | Radio remote control |
|  | 444.40 ÷ 444.80 | ≤ 100 mW ERP | ≥ 40 dBc at output of the transmiter | Radio alarm and detection systems |
|  | 482.19 ÷ 488.00 | ≤ 30 mW ERP | ≥ 40 dBc at output of the transmiter | Radio Microphne and Headphone |
|  | 866 ÷ 868 | ≤ 500 mW ERP | ≥ 40 dBc at output of the transmiter | RFID |
|  | 920 ÷ 925 | ≤ 500 mW ERP | ≥ 40 dBc at output of the transmiter | RFID |
|  | 821 ÷ 822 924 ÷ 925 | ≤ 183 W ERP | ≥ 32 dBc at 3m | Cordless Telephone |
|  | 2400 ÷ 2483 | ≤ 100 mW EIRP and ≤ 100 mW/100KHz EIRP for devices using FHSS modulation ≤ 10 mW/1MHz EIRP for devices using other modulations. | Detail in relevant annex | WLAN |
|  |  |  |  | Other spread spectrum applications |
|  | 5150 ÷ 5250 | ≤ 200 mW EIRP and ≤ 10 mW/MHz | Detail in relevant annex | WLAN |
|  | 5250 ÷ 5350 | ≤ 200 mW EIRP and ≤ 10 mW/MHz | Detail in relevant annex | WLAN |

____________

http://www.itu.int/dms_pub/itu-r/opb/res/R-RES-R.54-2007-PDF-E.pdf

http://www.mic.gov.vn/VBQPPL/details_law_e.asp?LawDoc_ID=4560206

http://www.mic.gov.vn/details_law.asp?LawDoc_ID=2357281

Frequency and power shown in Table 1 and 2 are indicative ranges only.  Actual transmit frequency or power may be subject to revision and updates in future.

http://www.erodocdb.dk/docs/doc98/official/pdf/REC7003E.PDF

See report ITU-R SM 2057, “Studies related to the impact of devices using Ultra Wideband (UWB) technologies on radiocommunication services”

http://www.itu.int/dms_pub/itu-r/opb/res/R-RES-R.54-2007-PDF-E.pdf

Administrations may indicate additional information on channel spacing, necessary bandwidth and interference mitigation requirement.

Administrations may indicate additional information on channel spacing, necessary bandwidth, interference mitigation requirement, unwanted emission limit and applicable radio standards.

Administrations may indicate additional information on channel spacing, necessary bandwidth, interference mitigation requirement, unwanted emission limit and applicable radio standards.

Administrations may indicate additional information on channel spacing, necessary bandwidth, interference mitigation requirement, unwanted emission limit and applicable radio standards.

Administrations may indicate additional information on channel spacing, necessary bandwidth, interference mitigation requirement, unwanted emission limit and applicable radio standards.

N	ote 1     Effective Radiated Power (e.r.p.) refers to radiation of a half wave tuned dipole, which is used for frequencies below 1 GHz.

N	ote 1     Effective Radiated Power (e.r.p.) refers to radiation of a half wave tuned dipole, which is used for frequencies below 1 GHz.

N	ote 1     Effective Radiated Power (e.r.p.) refers to radiation of a half wave tuned dipole, which is used for frequencies below 1 GHz.

N	ote 1     Effective Radiated Power (e.r.p.) refers to radiation of a half wave tuned dipole, which is used for frequencies below 1 GHz.

N	ote 2     Equivalent Isotropic Radiated Power (e.i.r.p.) is a product of the power supplied to the antenna and the maximum antenna gain, relative to an isotropic antenna, and is used for frequencies above 1 GHz. There is a constant difference of 2.15 dB between e.i.r.p. and e.r.p. [e.i.r.p. (dBm) = e.r.p. (dBm) + 2.15]

N	ote 2     Equivalent Isotropic Radiated Power (e.i.r.p.) is a product of the power supplied to the antenna and the maximum antenna gain, relative to an isotropic antenna, and is used for frequencies above 1 GHz. There is a constant difference of 2.15 dB between e.i.r.p. and e.r.p. [e.i.r.p. (dBm) = e.r.p. (dBm) + 2.15]

N	ote 2     Equivalent Isotropic Radiated Power (e.i.r.p.) is a product of the power supplied to the antenna and the maximum antenna gain, relative to an isotropic antenna, and is used for frequencies above 1 GHz. There is a constant difference of 2.15 dB between e.i.r.p. and e.r.p. [e.i.r.p. (dBm) = e.r.p. (dBm) + 2.15]