APT REPORT ON

Current status and future plan of implementation and deployment of IMT-2020 (5G) in Asia-Pacific region

No. APT/AWG/REP-111
Edition: September 2021

Adopted by

28th Meeting of APT Wireless Group
6 – 14 September 2021, Virtual/Online Meeting

(Source: AWG-28/OUT-08 (Rev.1))

APT REPORT ON

Current status and future plan of implementation and deployment of IMT-2020 (5G) in Asia-Pacific region

Introduction

IMT-2020 systems are mobile systems that include the new capabilities of IMT that go beyond those of IMT-Advanced as described in Recommendation ITU-R M.2083, called IMT-2020 Vision. IMT-2020 systems support low to high mobility applications and a wide range of data rates in accordance with user and service demands in multiple user environments. IMT-2020 also has capabilities for high quality multimedia applications within a wide range of services and platforms, providing a significant improvement in performance and quality of service.

ITU-R WP 5D finally completed IMT-2020 specification as Recommendation ITU-R M.2150 issued in February 2021 through the long-years process which consist of developing requirements, inviting proposals and evaluation. Regarding spectrum aspects for IMT-2020, the World Radiocommunication Conference 2019 decided additional IMT identification on some frequency bands above 24.25 GHz.

3GPP developed the first version of 5G (Release 15) specification in June 2018 and the first commercial 5G service started in April 2019. The updated version (Release 16), which meet all requirement of IMT-2020, has been released in July 2020 and incorporated in M.2150.

As of September 2021, some APT countries already launched their IMT-2020(5G) service and others are expected to launch in near future. The timings of first implementation are different in each country but it is common challenge for all APT members to deploy IMT-2020(5G) nationwide smoothly. It is beneficial to share information on current status and future plan of implementation and deployment of IMT-2020(5G) in Asia Pacific region.

Scope

The objective of this report is to collect and share information below among APT members to facilitate implementation on IMT-2020(5G) in Asia Pacific region.

- Current status and future plan of implementation and deployment of IMT-2020 (5G) from APT members

- Standardization and industry activities on IMT-2020(5G)

Questionnaire and Responses

Questionnaire

Questionnaire sent out to each of the APT Member Administration is shown below.

# 3.1.1.	Current status of 4G in your country

# Question 1: Is there any information on penetration rate and/or number of subscriptions of 4G technologies, such as LTE/LTE-Advanced, in your country?

If yes, please describe.

# 3.1.2	Future plan of implementation of IMT-2020 (5G) in your country

# Question 2: Is there any spectrum allocation plan for IMT-2020 (5G) in your country?

If yes, please describe the information including expected timeline, target frequency bands, etc.

# Question 3: Is there any current status for IMT-2020 (5G) trial/demonstration services in your country?

If yes, please describe the information including applied 5G services (eMBB, URLLC, mMTC), frequency bands, technical overview, test environments, main location and responsible organizations.

# Question 4: Is there any roadmap for commercialization of IMT-2020 (5G) in your country?

If yes, please describe.

# Question 5: Is there any other information for 5G preparations in your country?

If yes, please describe.

APT Member countries that submitted responses to APT during the development of this Report

The following member countries provided their responses to the Questionnaire.

- Australia

- Bangladesh

- China, People’s Republic of

- India, Republic of

- Indonesia, Republic of

- Iran, Islamic Republic of

- Japan

- Korea, Republic of

- Myanmar

- New Zealand

- Singapore

- Thailand

- Vietnam, Socialist Republic of

References

- Australia (AWG-25/INP-18)

- Bangladesh (AWG-24/INP-116, AWG-25/INP-16)

- China, People’s Republic of (AWG-24/INP-60, AWG-25/INP-62, AWG-26/INP-40(Rev.2), AWG-27/INP-72, AWG-27/INP-76)

- India, Republic of (AWG-24/INP-46)

- Indonesia, Republic of (AWG-24/INP-108, AWG-25/INP-80, AWG-26/INP-46, AWG-27/INP-87, AWG-28/INP-64)

- Iran, Islamic Republic of (AWG-24/INP-14)

- Japan (AWG-24/INP-68, AWG-25/INP-22, AWG-27/INP-30)

- Korea, Republic of (AWG-24/INP-85, AWG-28/INP-15)

- Myanmar (AWG-24/INP-24)

- New Zealand (AWG-24/INP-27)

- Singapore (AWG-24/INP-38)

- Thailand (AWG-24/INP-19, AWG-28/INP-35)

- Viet Nam, Socialist Republic of (AWG-24/INP-99, AWG-28/INP-86)

- GSMA (AWG-25/INP-54, AWG-28/INP-59)

- GSA (AWG-26/INP-55, AWG-27/INP-49, AWG-28/INP-80)

- IAFI (AWG-27/INP-61, AWG-28/INP-68(Rev.1))

Current status of 4G in Asia-Pacific region

Australia

Mobile network operators in Australia have extensive deployment of 4G (LTE/LTE-Advanced) services across Australia with consumers having a high take up of 4G capable handsets.

For example, the combined 3G and 4G coverage of Telstra’s mobile network now reaches 99.5 per cent of Australia’s population. The three largest mobile carriers are rolling out advanced 4G infrastructure in the 700 MHz and 850 MHz spectrum bands.

An estimated 34.84 million mobile voice and data services were in operation in Australia at June 2018. https://www.acma.gov.au/theACMA/communications-report.

Telstra’s 4G mobile network coverage area, reaches 99.2% of the Australian population. Telstra has more than 8750 4G enabled sites offering 4G speeds in all the capital CBD’s, many suburban areas and in over 1600 regional towns across Australia today.

https://www.telstra.com.au/content/dam/tcom/about-us/investors/pdf%20F/2018-Annual-Report.pdf

Australian mobile network operators do not differentiate service offerings between 3G and 4G and make available the best technology deployed that the mobile handset supports.

Bangladesh

Table Operators 4G network status in Bangladesh

| Operator`s Name | Total Subscriber (million) | Number of subscribers using 4G technologies (million) | 4G Penetration rate |
| --- | --- | --- | --- |
| Grameenphone Ltd | 74.781366 | 8.157869 | 11% |
| Robi Axiata Ltd | 47.7 | 6.47 | 14% |
| Banglalink Digital Communications Ltd | 34.54 | 2.62 | 7.60% |

*4G service was launched in Bangladesh on 19 February 2018.

China

According to the Statistic data released by Ministry of Industry and Information Technology, China. By the end of June 2018, the total number of mobile subscribers is 1.51 billion. The total 4G subscribers is 1.11 billion with 4G penetration rate to be 73.51%.

India

In India, as per Telecommunication Regulatory Authority of India (TRAI) August 2021 Report for the quarter ending March 2021, there are total of 1180.96 million mobile subscribers including 2G,3G & 4G subscribers. These include 825.30 million wireless Internet subscribers, the Rural mobile Tele-density is 60.08 % and Urban mobile Tele density is 137.08%. As on March 2021, the subscriber base of top four Wireless Broadband Service providers was as follows:

| As on March 2021 | Total Subscriber base (in million) | Urban Subscribers (in million) | Rural Subscribers (in million) |
| --- | --- | --- | --- |
| RJio | 426.25 | 246.91 | 179.34 |
| Airtel | 357.17 | 188.54 | 168.63 |
| VIL | 284.23 | 134.3 | 149.93 |
| BSNL& MTNL | 131.48 | 92.02 | 39.46 |

For details please see:

https://www.trai.gov.in/sites/default/files/QPIR_27082021.pdf

Network Deployments and Accessibility: The deployment pattern in India experience indicates that there is a considerable investment and deployment in 4G as is evident in the number of base stations deployed in the period depicted below which indicates the percentage wise growth since July 2016. This demonstrates that there is high demand for Mobile Broadband and the need to cater for the increasing appetite for data services in India. Details of growth in base stations is given in Figure below.

Source: Tarang Sanchar Portal

Indonesia

As of 4th Quarter in year 2019, 4G technologies, consists of LTE and LTE-Advanced, have already covered 70,670 of 83,218 villages in Indonesia.

Iran

Table Operators 4G network status in Iran

| Operator's name | Number of 4G subscriptions | Type of 4G technologies |
| --- | --- | --- |
| MCI | 11,844,145 | LTE/LTE-Advanced |
| Irancell | 7,155,753 |  |
| Total subscriptions | 18,999,898 |  |
| Country population | 79,926,270 |  |
| Penetration rate | 23.77% |  |

Japan

As shown in the table below, the penetration rate (i.e. the ratio of number of subscriptions to the total population*) of 4G (including LTE and LTE-Advanced) is 126.5% as of September 30 2020, 2018.

Table Number of subscriptions and penetration rate in Japan

|  | Number of subscriptions | Penetration rate |
| --- | --- | --- |
| Mobile Phone | 189,170,000 | 150.4% |
| LTE/LTE-Advanced | 159,150,000 | 126.5% |
| 3G | 29,340,000 | 23.3% |
| BWA | 73,660,000 | 58.5% |

*Total population of Japan is about 127,910,000.

Republic of Korea

The total number of mobile phone users exceeds 71.62 million in June 2021, surpassing Korea's population, which is about 51.82 million. These 71.62 million users consist of 50.84 million LTE users (LTE commercial service was launched in Korea on July 2011), 4.07 million WCDMA users, 1.26 million CDMA users and 16.4 million 5G users which are accounting for 70.9%, 5.69%, 0.33% and 22.9% of the total number of mobile phone users respectively. 5G commercial service was launched on April 2019.

Table number of mobile phone users with different services in Korea

| Service | Dec. 2016 | Dec. 2017 | Dec. 2018 | Dec. 2019 | Dec. 2020 | Jun. 2021 |
| --- | --- | --- | --- | --- | --- | --- |
| CDMA(2G) | 3,545,455 | 2,556,242 | 1,672,741 | 1,020,510 | 502,585 | 237,348 |
| WCDMA(3G) | 11,439,821 | 10,661,566 | 24,205,134 | 7,515,093 | 5,604,557 | 4,078,976 |
| LTE(4G) | 46,310,262 | 50,440,880 | 55,133,681 | 55,687,974 | 52,555,161 | 50,844,277 |
| 5G | - | - | - | 4,668,154 | 11,851,373 | 16,465,468 |
| Total | 61,295,538 | 63,658,688 | 66,355,778 | 68,892,541 | 70,513,676 | 71,626,069 |

Source: Ministry of Science and ICT, 2021.08.

Table Legacy IMT(2G/3G/4G) Spectrum Allocation Status in Korea

| Classification |  | Total |
| --- | --- | --- |
| 2G | 800 MHz (10 MHz), 1.8 GHz Band (20 MHz) | 30 MHz |
| 3G | 2.1 GHz Band (40 MHz) | 40 MHz |
| 4G | 800 MHz (50 MHz), 900 MHz (20 MHz), 1.8 GHz Band (90 MHz), 2.1 GHz Band (80 MHz), 2.6 GHz Band (100 MHz) | 340 MHz |
| Total |  | 410 MHz |

Myanmar

In Myanmar, radio frequency bands are assigned technology-neutrally for Operators no differentiation on 2G,3G,4G or LTE. According to the independent statistics, 80% of mobile subscribers use smart phones and over 40% of them connect 3G services. Last year PTD assigned 1800Mhz to nationwide operators to provide better QoS to the public.

New Zealand

New Zealand’s 4G mobile networks currently reach population coverage over 98%. The 4G technologies uptake is 58.86% (i.e. approximately 3.75 million 4G subscriptions from a total of about 6.37 million connections, excluding licensed IoT connections) after 6 years when 4G was first deployed in New Zealand.

Singapore

Based on May 2018 statistics, Singapore’s mobile penetration rate is about 148 percent. Of the 8.3 million mobile subscribers, about 75% of them are on 4G networks.

Thailand

- Table Operators 4G network status in Thailand

| Mobile Operator | No. of 4G Subscribers (Million) | No. of Mobile Subscriber(Million) |
| --- | --- | --- |
| Advanced Wireless Network Co. Ltd. | 31.91 | 41.44 |
| True Move H Universal Communication Co. Ltd. | 22.97 | 30.63 |
| DTAC Trinet Co. Ltd. | 12.20 | 18.86 |
| National Telecom Plc. | N/A | 2.52 |
| Total | 67.08 | 93.45 |

Source: Company Annual Report 2020/Information from mobile operators

Viet Nam

The 4G mobile broadband network has been developed relatively well. The percentage of population coverage by 4G reached 99.8% with average download speed up to 44.16 Mbps.

(Source: Speedtest.net)

More than 300 thousands of mobile base station installed, over 35% of which are 4G base stations. The mobile broadband penetration reached more than 68 million subscriptions. The smartphones penetration reached over 73% of mobile devices.

Status and future plan of IMT-2020(5G) in Asia-Pacific region

Spectrum plan for IMT-2020 (5G)

Australia

The ACMA has a technology flexible spectrum allocation plan for the band 24.25 – 27.5 GHz that can be used for IMT-2020 (5G) services. The spectrum plan is shown in Figure below.

- Class-licensing for indoor use—Australia-wide.

- Class-licensing for indoor and outdoor use—Australia-wide.

- Spectrum licensing—defined areas. Includes additional conditions to protect SRS earth stations.

- Spectrum licensing with additional FSS coexistence conditions within certain areas.

- Apparatus licensing—Australia-wide.

- Apparatus licensing—Australia-wide, except defined areas. Includes additional conditions to protect SRS earth stations.

- Apparatus licensing with additional conditions to protect FSS uplinks – Australia-wide except defined areas. New FSS earth stations will also be permitted (except in defined areas) on a first-in-time coordinated basis with apparatus licensed wireless broadband services.

- Figure Planned arrangements for wireless broadband services in the 26 GHz band

Further information can be found in the ACMA paper Future use of the 26 GHz band: Planning decisions and preliminary views.

The ACMA is currently undertaking a planning and consultation phase as part of the spectrum refarming of the 24.25 –27.5 GHz band and timeframes for this spectrum being available are yet to be determined.

Licences in Australia are issued on a technology flexible basis as such any of the following IMT bands will be able to be used in Australia to provide 5G services:

Table 5G licenses bands with 3GPP technology in Australia

| Frequency range | 3GPP Band |
| --- | --- |
| 703-748 / 758-803 MHz | 28 |
| 825-845 / 870-890 MHz only | 5 |
| 890-915 / 935-960 MHz only | 8 |
| 1710-1785 / 1805-1880 MHz | 3 |
| 1920-1980 / 2110-2170 MHz | 1 |
| 2302-2400 MHz | 40 |
| 2500-2570-2620-2690 MHz | 7 |
| 3400-3700 MHz | 42/43 Nr77/78 |
| 24250-27500 MHz(*) | N258 |

(*) Band is consistent with WRC-19 Agenda Item 1.13 being considered for IMT-2020 services

The bands 3400 – 3700 MHz and 24250 – 27500 MHz are likely to be the pioneer bands for IMT-2020 services in Australia with Telstra already launching 5G services in the 3400 – 3700 MHz band.

Bangladesh

Bangladesh is likely to launch IMT 2020 (5G) Service by 2020. The spectrum allocation for IMT 2020 (5G) is likely to take place before the introduction of 5G in Bangladesh.

China

In November 2017, Ministry of Industry and Information Technology (MIIT) of China has announced the frequency plan for the 3300–3400 MHz, 3400–3600 MHz, and 4800–5000 MHz bands for IMT-2020 system (3300-3400 MHz is limited to indoor scenario only).

On 6th June 2019, The MIIT officially granted the 5G licenses for commercial use to the nation's major three telecom carriers, including China Mobile, China Telecom and China Unicom, as well as the State-owned China Broadcasting Network Corp.

While, China Broadcasting Network Corp has been declared by the country’s government to deploy a 5G network across 16 cities using spectrum in the 4.9GHz band, as a 5G trail band. Moreover, 700MHz also is discussing possible allocation scheme for China Broadcasting Network Corp.Moreover, In December 2020, the MIIT officially issues a 10-year spectrum license for 5G medium and low frequency band frequency to China Mobile, China Telecom, China Unicom and China Broadcasting Network Corp..

The specific frequency allocation of 5G in China is shown below:

- 2 515MHz – 2 675MHz and 4 800MHz - 4 900MHz are allocated to China Mobile;

- 3 400MHz – 3 500MHz and 1 920 – 1 940/2 110 – 2 130(4G re-farming)are allocated to China Telecom;

- 3 500MHz – 3 600MHz and 1940 – 1 965/2 130 – 2 155(4G re-farming)are allocated to China Unicom;

- 703-733/758-788MHz allocated to China Broadcasting Network Corp.

By then end of June 2020, more than 40 thousand 5G Base Stations have been established nationwide and 66 million 5G terminal have connected to the 5G network. By the March 2021, 0.7 million 5G Base Stations has been established national wide, with SA network deployed in all cites, 180 million 5G terminal have connected to the 5G network.

In the future, additional spectrum within the middle range is needed to enable future China 5G deployment and meet the users’ increasing requirement of anytime anywhere high data rate communications. The spectrum within the 5 925-7 125 MHz frequency band can partially meet future IMT spectrum needs. In WRC-19, China proposed 5925-7125MHz, or parts thereof, for WRC-23 studies for IMT identification as additional mid-band to drive future 5G development together with other APT countries, i.e. Afghanistan, Cambodia, Lao, Mongolia, Myanmar, Nepal and PNG. China has started the studies on 6GHz and will contribute to ITU-R as appropriate.

India

Government of India, WPC Wing supports technology neutral spectrum. Telecom Service Providers can use any IMT technology in the bands identified for IMT. Government of India supported identification of frequency bands 24.25-27.5 GHz, 37-43.5 GHz (and parts thereof), 47.2-48.2 GHz and 66-71 GHz during the WRC-19. India recognizes the importance of the conditions which were identified during the WRC-19 with respect to the implementation of IMT in these bands. The government of India is working on its next steps for opening up of some or all of these frequency bands for deployment of IMT-2020. Expected roadmap of 5G deployment in India is as follows.

Spectrum Bands Used for Mobile Services in India: current scenario.

| No | BAND(MHz) | USAGE |
| --- | --- | --- |
| 1 | 800 | Initially used for 2G (CDMA). Currently used for LTE |
| 2 | 900 | Initially used for 2G (GSM). Currently used for 2G + LTE |
| 3 | 1800 | Originally only used for GSM, progressive redeployment to LTE |
| 4 | 2100 | Originally only used for 3G, progressive redeployment to LTE |
| 5 | 2300 | Originally used for Point to Point links, now a standardised LTE band for capacity |
| 6 | 2500 | New capacity band for LTE |

Indonesia

Initial indications show that Indonesia will stipulate the spectrum licenses for IMT-2020 (5G) in some new candidates band around 2022. Frequency bands which currently under consideration to be released in order to acquire the benefit from international ecosystem of IMT-2020 (5G) are 700 MHz, 2.6 GHz, 3.3 GHz, 3.5 GHz, 26 GHz and 28 GHz bands.

Iran

The current allocated spectrums could be used by operators for 5G. For millimeter-wave range, there isn't any published frequency band plan for IMT-2020 in our country, but in parallel to ITU-R studies on WRC-19 agenda item 1.13, related studies and arrangements happened for on time implementation of IMT 2020. The outcome of relevant studies were be tuned by WRC-19 results. For example following figure shows the frequency band 37-40.5 GHz that has been divided into smaller segments. FDD or TDD utilization would be chosen by operators themselves.

Japan

Toward the launch of 5G in 2020, we have assigned the slots of 100 MHz bandwidth in both 3.7 GHz band (3.6-4.1GHz) and 4.5 GHz band (4.5-4.6GHz), and 400 MHz bandwidth in 28 GHz band (27.0-28.2GHz and 29.1-29.5GHz) on April 2019 to the four mobile operators, taking frequency sharing with incumbent radio systems into account. The results are shown in figure below.

Figure The results of 5G frequency spectrum assignment

Japan adopted the so-called “beauty contest” on the assignment of 5G frequency spectrum. It had been conducted according to the establishment guidelines for base stations, which set indicators to evaluate the following points:

1.	Securing the deployment possibility to all over the country

2.	Starting the service early in rural areas

3.	Securing the diversity of services

Thus, Japan aims to realize the early nationwide 5G expansion not only to urban areas but also rural areas as follows:

1.	Developing 5G advanced base stations* for more than 50% meshes (geographical meshes of 10km square) within five years in both whole country and each region

2.	Starting services in all prefectures within two years after frequency spectrum assignment

3.	Establishing as many base stations as possible across the country

After the launch of 5G commercial services in March 2020 by NTT DOCOMO, KDDI and Softbank and in September 2020 by Rakuten mobile, number of subscriptions of 5G service is about 790,000 as of September 30 2020. The penetration rate of 5G is 0.6 %.

Figure Examples of Local 5G usage

In addition, Japan has worked on the implementation of a system “Local 5G”, in which 5G was used by different subjects to satisfy their regional or individual needs, in order to promote 5G use within local regions. Local 5G has been used in the frequency spectrum of 4.6 to 4.9GHz and 28.2 to 29.1GHz.

Figure above shows examples of Local 5G usage, and Figure below summarizes the bands for Local 5G.

Figure Bands for Local 5G

As Local 5G operators, there are 44 applicants from 15 Manufacturers, 12 Cable TV operators (Multiple system operators), 8 Telecommunications carriers/SIers, 4 universities/research institutes, 3 local/central governments and 2 companies in other industries as of 15 February 2021.  22 in 44 applicants have been licensed, the others are under review. Mobile operators have not been allowed to obtain a Local 5G band license for the time being.

Republic of Korea

5G spectrum auction has completed in Korea on June, 2018. Through this auction, Korea government supplied 280 MHz bandwidth with 28 blocks in 3.5 GHz band and 2.4 GHz bandwidth with 24 blocks in 28 GHz band at the same time.  Three operators have launched the world's first nationwide 5G mobile networks in April 3, 2019.

Frequency bands of auction:

3,420 - 3,700 MHz (total 280 MHz BW, 28 blocks/10MHz unit, Maximum(spectrum cap) 100 MHz per operator, 10 years’ license from Dec. 1st 2018)

26.5 - 28.9 GHz (total 2.4 GHz BW, 24 blocks/100MHz unit, Maximum(spectrum cap) 1 GHz per operator, 5 years’ license from Dec. 1st 2018)

Figure The results of 5G frequency spectrum allocation

The obligation of 5G Network Deployment is shown in Table below.

Table The obligation of 5G network deployment

| Classification | Reference station | % of obliged deployment by year |  |
| --- | --- | --- | --- |
|  |  | 3 Years (15%) | 5 Years (30%) (Sum) |
| 3.5 GHz Band | 150,000 Stations | 22,500 Stations | 45,000 Stations |
| 28 GHz Band | 100,000 (based on equipment) | 15,000 | - |

In case of 3.5 GHz band, it was imposed duty to deploy 22,500 stations (15%) in 3 years and 45,000 stations (30%) in 5 years of 150,000 stations as a reference number. In case of 28 GHz band, it was imposed duty to deploy 15,000 equipment and stations (15%) in 3 years of 100,000 equipment and stations, at the level of nationwide LTE obligation, as a reference number

In addition to the 5G spectrum auction, the government announced 'Local 5G Spectrum Plan’ in June 2021, which makes a total of 700 megahertz available in 4.7 GHz and 28 GHz bands for localized 5G networks in limited area such as campus, inside building.

100 megahertz in the 4.7 GHz band and 600 megahertz in the 28 GHz band is offered for 5G specialized network (a.k.a. Local 5G) services. It is expected that many enterprises across the country acquire frequencies in the two bands and then build specialized 5G networks in locations such as a campus or inside buildings. The frequencies are to be allocated administratively and there will be an annual radio wave usage fee.

Spectrum for specialized 5G is 10 blocks of 10 megahertz in 4.72-4.82 GHz and 12 blocks of 50 megahertz in 28.9-29.5 GHz. The basic charge for each band is KRW 100,000 ($88) per 10 megahertz in 4.7 GHz and KRW 50,000 ($44) per 50 megahertz in 28 GHz. Applicants can apply for a license duration from two to five-year term, with a deployment obligation within six months after being licensed. The assignment fee of which calculation formula uses factors such as the basic charge for each band, area, license duration and bandwidth is set higher for cities with a denser population and is also significantly lower for 28 GHz than 4.7 GHz.

Figure Specialized 5G spectrums and block sizes of each band

Myanmar

The following bands could potentially be made available by PTD for assignment in the next 5 years:

- Unassigned portions of the 850/900 MHz and 2100 MHz bands;

- 700 MHz;

- 1800 MHz;

- 2300 MHz; and,

- 2600 MHz.

These bands were identified for consideration in light of the allocations of the updated NTFA (National Table of Frequency Allocation), spectrum already assigned and spectrum release activities internationally with a particular emphasis of Asia-Pacific Telecommunity (APT) countries.

This Roadmap sets out the release of 2.6 GHz, a portion of 900 MHz, followed by 1.8 GHz, 700 MHz and the balance of unassigned 800 and 900 MHz over a 5-year period. The Roadmap is consistent with the priorities established by PTD and has considered the comments received from stakeholders in the consultation process. Previously, Ministry/PTD had made provision in a separate process for the 4th operator including a portion of spectrum in the 900MHz and 2100MHz bands.

New Zealand

New Zealand is currently considering the possible re-allocation of four frequency bands for IMT-2020. Among these bands, the 3.5 GHz band has higher potential for early re-allocation than others. The band plan and exact frequency boundaries are still under consideration.

The table below provides more information about these bands:

Table The obligation of 5G network deployment

|  | Band Name | Frequency range | Status |
| --- | --- | --- | --- |
| 1 | 3.5 GHz | 3 410 - 3 800 MHz | Consultation with industry stakeholders about possible early re-allocation in spite of the expiry date of incumbent spectrum rights is October 2022 Planned to be re-allocated for 5G use by 2020 New band plan and exact frequency boundaries are still under consideration |
| 2 | 26/28 GHz | 24.25 - 27.5 GHz / 27.5 - 28.35 GHz | Monitoring ITU-R studies on related WRC-19 agenda items, including Agenda item 1.13 (24.25-27.5 GHz) and Agenda item 1.5 (27.5-29.5 GHz) New band plan and exact frequency boundaries subject to review in 2019/2020 when there is more clarity about WRC-19 decisions and equipment tuning range Coordination regime between transmitting earth station (FSS uplink) and 5G are still under consideration to allow continuity of FSS operation in 27.5-28.35 GHz |
| 3 | 1.5 GHz | 1 427 - 1 518 MHz | Monitoring ITU-R studies on the sharing and compatibility analysis between IMT and mobile-satellite services Re-allocation for 5G to be considered once appropriate transitional programme for existing fixed links in this band is identified New band plan and exact frequency boundaries subject to review in 2020/2021 when there is more clarity on a harmonised band plan and broader equipment availability based on either FDD, TDD or SDL arrangement |
| 4 | 600 MHz | 610 - 698 MHz | Re-allocation for 5G to be considered once appropriate alternative solutions have been identified to address existing use of the band, including radio microphones Band plan is likely to follow 3GPP Band Class 71 |

Singapore

Singapore is looking to make available a mixture of low (below 1 GHz), high (1-6 GHz) and millimeter wave (above 6 GHz) frequencies for 5G from 2020 and beyond. Both the spectrum availability and timeline is dependent on several factors such as the outcome of the frequency bands identified for consideration at the 2019 World Radiocommunication Conference (WRC-19) for 5G deployments, spectrum harmonization with neighboring countries, and local spectrum reframing and migration timelines.

Thailand

NBTC has already held an auction for 700 MHz, 2600 MHz and 26 GHz frequency bands in 2020. Now, NBTC is implementing an IMT Spectrum Roadmap for 3500 MHz and 28 GHz, expected to be auctioned in 2022.

Viet Nam

Ministry of Information and Communication considers several new bands for 5G deployment as following:

| Band | Frequency range | Band-planning |
| --- | --- | --- |
| 700 MHz | 703-733 MHz / 758-788 MHz | Released |
| 1.5 GHz | 1427-1518 MHz | Considering |
| 2.3 GHz | 2300-2400 MHz | Released |
| 2.6 GHz | 2500-2690 MHz | Released |
| 3.8 GHz | 3600-4000 MHz | Consulted |
| 4.9 GHz | 4800-5000 MHz | Consulted |
| 26 GHz | 24.25-27.5 GHz | Released |

Beyond the above bands, mobile operators can deploy IMT/5G systems in their licensed bands, such as 850 MHz, 900 MHz, 1800 MHz and 2100 MHz.

Current status for IMT-2020 (5G) trial/demonstration services

Australia

Telstra launched commercial 5G services in selected areas of Australia on 28 May 2019, using 3.4-3.6 GHz spectrum.

Telstra has also conducted a number of 5G technology trials using Experimental/Scientific Licenses issued by ACMA. These licenses are issued on a no-interference, no-protection basis. Because much of the hardware available for trial is based on the technical requirements that apply to other overseas markets, the choice of operating frequency/band is limited and creates the need to carefully consider the incumbent licensees. In those cases, formal frequency coordination studies were conducted and interference mitigation processes are put in place as necessary.

| Band | Timing | Description | Location |
| --- | --- | --- | --- |
| 15 GHz | Mid 2016 | Early trial/demonstration using prototype test-bed hardware. Single site. | Melbourne |
| 3.6 GHz | Late 2017 | Vendor equipment evaluation. | Launceston and Ballarat |
| 27 GHz | Early 2018 | Trial/demonstration with a small cluster (10 sites) in a metropolitan area. | Gold Coast |
| 27 GHz | Mid 2018 | Vendor equipment evaluation with fixed terminal. | Bendigo |
| 27 GHz | Mid 2018 | Indoor coverage and technology demonstration. | Melbourne and Sydney |

While mobile network operators have undertaken IMT-2020 (5G) trials in Australia in a range of frequency bands, these trials utilise experimental/scientific licences and the ACMA provides no commitment that those bands will be made available for mobile broadband services in the longer term.

One trial/demonstration of 5G service has been conducted by Huawei Technologies (Bangladesh) Ltd with mobile operator Robi Axiata Ltd on 25th July 2018. This trial/demonstration was conducted in 28 GHz (26.65-29.19 GHz) frequency band and the maximum speed achieved was 4.5 Gbps.

Bangladesh

One trial/demonstration of 5G service has been conducted by Huawei Technologies (Bangladesh) Ltd with mobile operator Robi Axiata Ltd on 25th July 2018. This trial/demonstration was conducted in 28 GHz (26.65-29.19 GHz) frequency band and the maximum speed achieved was 4.5 Gbps.

China

The 5G Trials in China include two phases, Technology R&D Trial (Phase 1) and Product R&D Trial (Phase 2). The target of phase 1 is to promote 5G core technology research and development in China, to verify 5G technology schemes and to support global unified 5G standards. Then the phase 2 Trial aims to validate the 5G product.

Figure China 5G Trial plan

Technology R&D Trial (Phase 1): led by the IMT-2020 (5G) Promotion Group and conducted from 2016 to 2018. In January 2016, the Ministry of Industry and Information Technology (MIIT) in China has launched the 5G technology R&D Trial (Phase 1). The three steps of this trial are as follows:

- Step 1 (Key technology trial): implemented from January 2016 to September 2016, focusing on the test of 5G key technology prototypes, the evaluation of function and performance, and promoting the standards consensus building.

- Step 2 (Technology scheme trial): implemented from June 2016 to December 2017 targeting to test the performance of single base station and to verify the functionality and performance of different vendors' 5G technology schemes.

- Step 3 (System trial):  Implemented from June 2017 to December 2018, targeting to verify the networking and interconnection performance of 5G system, and demonstrate the 5G typical services.

Product R&D Trial (Phase 2): considered as the 5G scale trial in China as well, will be led by Chinese operators and conducted from 2018 to 2020. In the beginning of 2019, he Ministry of Industry and Information Technology (MIIT) in China has launched the 5G product R&D Trial (Phase 2). The two steps of this trial are as follows:

- Step 1 (IoT tests between chips and system): implemented from January 2019 to January 2020, focusing on Interoperability between chips companies and 5G system vendors; both 2.6GHz and 3.5GHz are trial bands; and Non-standalone (NSA) and Standalone (SA) are considered as 5G networks in the test.

- Step 2 (mmWave R&D  trial): implemented from January 2020 to December 2020 targeting to test the performance of mmWave base station and to verify the functionality and performance of different vendors' 5G technology schemes in mmWave.

India

Department of Telecommunication (DoT) in India has initiated 5G Hackathon (https://www.5ghackathon.in) which provides great platform for start-ups and SMEs to test their ideas and innovative skills. With this they can leverage state of the art technology for the solution of global problems.

5G Trials

Government of India issued a press release on 4th May 2021 approving permissions to Telecom Service Providers (TSPs) for conducting trials for use and applications of 5G technology with following objective:

- Testing 5G spectrum propagation characteristics

- Tuning/evaluation of chosen equipment/vendors

- Testing of indigenous technology

- Testing of applications/use cases

- Test 5G phones and devices.

Trials are required to be conducted in rural, semi-urban and urban settings for various use cases in these scenarios. Trials will be kept isolated and are not allowed to connect with existing networks of the telecom service providers. Duration of these trials is six months.

The experimental spectrum bands identified for these trials in the press release are:

- mid-band (3.2 GHz to 3.67 GHz),

- millimeter wave band (24.25 GHz to 28.5 GHz)

- Sub-Gigahertz band (700 MHz).

- TSPs will also be permitted to use their existing spectrum owned by them (800 MHz, 900 MHz, 1800 MHz and 2500 MHz) for conduct of 5G trials.

Indonesia

From 2017 until 2021, there has been a total of fourteen IMT-2020 (5G) trials conducted in several cities of Indonesia. The responsible organization for those trials is Ministry of Communications and Informatics (MCI), Republic of Indonesia. Those trails held in collaboration with mobile/cellular operators.

All trials applied several uses cases with various frequency bands being used as detailed in the Table shown below.

Table IMT-2020 trials information in Indonesia

| No | Operator | Frequency Band | Use Cases | Date |
| --- | --- | --- | --- | --- |
| 1 | PT XL Axiata, Tbk. | 15 GHz | Peak throughput demonstration | March 2017 |
| 2 | PT Telekomunikasi Selular | 72 GHz | Peak throughput demonstration | May 2017 |
| 3 | PT Telekomunikasi Selular | 28 GHz | - 5G Autonomous Electric Vehicle - 5G Experience Center - VR Cycling, Driving, Football - Suten Robot - 5G Tablet - 3D Gold Medal - 5G Live Streaming - Interactive Time Slice - Throughput Performance Test - Channel BW Configuration - Power Output Tuning - Indoor/outdoor test | August 2018 (during Asian Games XVIII event) |
| 4 | PT XL Axiata, Tbk. | 15 GHz & 28 GHz | - Showcase - VR Experience - Waste Management - Water Management - Water Monitoring - Beam Tracking - Technical Test - Reflector existence - Distance configuration - Angle between AAU & TUE configuration | August 2018 |
| 5 | PT Indosat, Tbk. | 28 GHz | - 3D Augmented Reality - Technical Test - Peak Throughput - Beam Tracking - 4K Video Streaming (latency) | November 2018 |
| 6 | PT Smart Telecom | 28 GHz | “Industry 4.0 in Industrial Area” - Showcase - Remote industry monitoring with 360o camera and VR - Remote troubleshooting with drone and VR view - Technical test - Peak throughput - UL throughput when execute remote monitoring and troubleshooting | August 2019 |
| 7 | PT XL Axiata, Tbk. | 28 GHz | “Holographic Call for Productivity” - Holographic call demonstration - Coverage test - 5G-LTE Dual Connectivity - 5G Handover Mobility | August 2019 |
| 8 | PT Hutchison 3 Indonesia | 28 GHz ( for access), 23 GHz & 80 GHz (for transport) | “Real Time Interactive e-Education” - Showcase: Remote class with interactive holographic call and interactive white board - Technical Test: - 5G – LTE Dual Connectivity - 5G Data Call Setup - 5G DL/UL Throughput and Latency Test - 5G Mobility Test - 5G Voice Call : VoLTE Fallback | September 2019 |
| 9 | PT Telekomunikasi Selular | 28 GHz & 3.5 GHz (indoor) | - Showcase5G Video Call - Smart surveillance for security and traffic monitoring - Smart air patrol - Immersive Collaboration - Seamless Gaming - Technical Test - Channel collaboration with VR - Immersive entertainment - Bandwidth configuration - Obstacle existence test - Coverage & power test - Mobile walk test: RSRP, RSRQ and SINR distribution - Throughput Performance Test | November 2019 |
| 10 | PT Telekomunikasi Selular | 28 GHz | - Showcase - 5G Smart Agriculture - 5G Public Safety: Video Surveillance - 5G AR Remote Assistance - Peak throughput test | December 2019 |
| 11 | PT Telekomunikasi Selular | 3.5 GHz | - Coexistence trial between 5G and FSS in 3.5 GHz with: • 5G in 3.5 - 3.6 GHz (BW 100 MHz) • 100 MHz guard band (3.6 – 3.7 GHz) • Band pass filter 3.7 GHz – 4.2 GHz (FSS Receiver) - Trial activities: • 5G Static and Mobility Performance Test • Field Trial case 3.5 GHz Adjacent channel Co-existence test • Band Pass Filter verification via Laboratory Testing - This trial is in collaboration with PT Telekomunikasi Satelit Indonesia and Telkom University | October -November 2020 |
| 12 | PT XL Axiata, Tbk. | 1800 MHz | - Trial for a 5G NR Non-Stand Alone (NSA) network with Dynamic Spectrum Sharing (DSS) feature in Live Network with 20 MHz Bandwidth, which 2100 MHz band was used as the anchor and 1800 MHz was used for shared band between 4G and 5G traffic. - Trial activities: • Internetworking between 4G and 5G, sharing frequency and sharing RAN through NSA • Comparison between 4G and 5G throughput with sharing frequency in live network • Mobility test (handover) of 5G network using DSS • Characteristic of the 5G NR coverage area in 1800 MHz. • Effect of 5G implementation with DSS technology which is limited in a maximum bandwidth of 20 MHz. | December 2020 |
| 13 | PT Smartfren Telecom, Tbk. | 28 GHz | - Showcase • Remote Monitoring Using VR (5G NSA); - VR Immersive live monitoring - Cloud VR Gaming - VR Touring holiday destination - Remote video experience using AR glass - Technical test • Peak throughput - Stable throughput with >1 Gbps for DL | June 2021 |
| 14 | PT Telekomunikasi Selular | 3.5 GHz | - 5G laboratory to conduct use case testing, device testing and application testing with: - Indoor coverage - Small cell BTS - Frequency range 3400 – 3500 Mhz (bandwidth 100 MHz) - Power 4 x 200 mW - 5G radio communication tester for wireless devices - Application platform tester for application performance and experience - Trial activities: - Discover 5G use case for industry and society - 5G application lab testing running on the platform include user experience, network experience, and etc - 5G device lab testing working on the 5G NR include sensitivity, OTA, and etc This trial is in collaboration with PT Telkom Indonesia Tbk | June 2021 |

Iran

This administration encourages operators, vendors and searching laboratories to conduct 5G tests. Accordingly, one operator crossed 1 Gbps down-streaming record in-building. There are also several NB MTC-IoT networks in various areas such as smart-cities, portal operations, transportation, etc. which are mainly implemented by collaboration of start-ups and operators.

Japan

We had conducted 5G Field Trials in both urban cities and rural areas from fiscal year (FY) 2017 to FY 2019. Table below shows the last trials conducted in FY 2019 that focused on more rural areas as implementation sites. The details of 5G Field Trials in Japan can be viewed from https://5gmf.jp/en/whitepaper/

Table 5G Field Trials in Japan in FY 2019

| Technology | Target | Overview | Main locations | Organization |
| --- | --- | --- | --- | --- |
| eMBB | Avg. 4-8 Gbps (base station) Avg.2-4 Gbps (user terminal) |  |  | (Organizer) NTT DOCOMO, Inc. |
|  |  | 1 Safety operation of crane work with high-definition images | 1 Ehime Pref. | 1 Ehime University |
|  |  | 2 Monitoring behavior at nursing homes | 2 Hiroshima city, Hiroshima Pref. | 2 Sompo Holdings, Inc. |
|  |  | 3 Realtime cloud-editing and relay of video | 3 Sendai city, Miyagi Pref. | 3 Sendai Television Incorporated |
|  |  | 4 Passing down of traditional culture(remote education) | 4 Tono area, Gifu Pref. | 4 CBC Creation Co., Ltd. |
|  |  | 5 Livelihood support through sound visualization | 5 Tono area, Gifu Pref. | 5 SUNCORPORATION |
|  |  | 6 Experience-based sightseeing with VR and Body Sharing | 6 Naha city, Okinawa Pref. | 6 H2L Inc. |
|  |  | 7 Advanced remote medical care | 7 Wakayama city. et al. | 7 Wakayama Pref. |
|  |  | 8 Advanced emergency transport | 8 Maebashi City, Gunma Pref. | 8 Maebashi city |
|  | Avg. 1 Gbps (mobility) |  |  | (Organizer) NTT Communications Corporation |
|  |  | 1 Snow removal countermeasures | 1 Eiheiji Town, Fukui Pref. | 1 Eiheiji Town |
|  |  | 2 Driving assistance in dense fog | 2 Oita Pref. | 2 Oita Pref. |
|  |  | 3 Round assistance on golf courses | 3 Nagano city, Nagano Pref. | 3 MIRAIT Corporation |
|  |  | 4 Support for ensuring safety in underground railways | 4 Osaka city, et al. | 4 ITOCHU Techno-Solutions Corporation |
|  | Avg. 300 Mbps (Indoor) |  |  | (Organizer) Advanced Telecommunications Research Institute International |
|  |  | 1 Watching sports with togetherness in the stadium | 1 Higashiosaka city, Osaka Pref. | 1 Jupiter Telecommunications Co., Ltd. |
|  |  | 2 Increasing efficiency in the dairy and livestock industries | 2 Kamishihoro town, Hokkaido | 2 Tokachi Murakami Farm |
|  |  | 3 Support for bloodhorse breeding industry | 3 Niikappu town, Hokkaido | 3 Hidaka Keishuba Kyoudou Ikusei Kousha Corp. |
| URLLC | 10 ms latency (End to End) |  |  | (Organizer) Wireless City Planning Inc. |
|  |  | 1 Evacuation guidance and traffic control in disasters | 1 Kitakyushu city, Fukuoka Pref. | 1 NIPPON SIGNAL CO., LTD. |
|  |  | 2 Truck platooning, Remote monitoring and control | 2 Hamamatsu city, et al., Shizuoka | 2 Advanced Smart Mobility Co., Ltd. |
|  | URLLC with Avg. 300 Mbps (terminal) |  |  | (Organizer) KDDI CORPORATION |
|  |  | 1 Support system for Mountain climbers | 1 Komagane city, Nagano Pref. | 1 Shinshu University |
|  |  | 2 Operational support of sports (Slack line) | 2 Obuse town Nagano Pref. | 2 Goolight Co., Ltd. |
|  |  | 3 Tourism promotion using VR | 3 Minamiaso village, Kumamoto Pref. | 3 Tokai University. |
|  |  | 4 Remote control of construction machines | 4 Iga city, Mie Pref. | 4 OBAYASHI CORPORATION |
| mMTC | 1 million devices/km2 density |  |  | (Organizer) Wireless City Planning Inc. |
|  |  | 1 Safety management of workers in tunnels | 1 Hokkaido | 1 TAISEI CORPORATION |
|  |  | 2 Increasing efficiency in logistics through visualization | 2 Nerima-ku, Tokyo | 2 NIPPON EXPRESS CO., LTD. |

Under lines indicate that they are based on the 5G utilization idea contest.

Republic of Korea

As of the end of June 2021, the number of 5G subscribers in the country stood at over 16.4 million, accounting for 22.9 % of the total 71.62 million mobile subscribers, two years later  after the 5G networks first went live in April 2019.

5G Trials for Olympic Winter Games PyeongChang 2018

Olympic Winter Games PyeongChang 2018 (2018.2.9~25) provided a whole new Olympics experience with the 5 advanced ICT Services which were demonstrated as listed in below.

(Source: Ministry of Science and ICT)

Figure Pyeongchang Olympic 5G trial: 5 advanced ICT services

Korea successfully showed various 5G trial services such as autonomous 5G car and 360 degree VR live show by using these 5 advanced ICT technologies. Regarding 5G trial network, Korea operators built a pre-standard 5G network running in 28 GHz band with vendors for Winter Olympics, and recorded peak data rate of up to 3.5 Gbps on a 5G mobile tablet using the network. The system (comprising 22 5G links at 10 different sites) delivered 3,800 terabytes of data during the two-week event.

(Source: Ministry of Science and ICT)

Figure Pyeongchang Olympic 5G trial: Network

5G Trials for Vertical Industries

Due to the saturation of the traditional mobile service market, there are many attentions to new 5G service market for vertical industries. Considering this trend, MSIT has started the 5G Trials for Vertical Industries from Apr. 2018 to Dec. 2020. The detailed information about 5G trail service for vertical industries is shown in Table below.

Table 5G Trials for Vertical Industries

| Organization/ Services | Main partners | Services | Main locations | Technology |
| --- | --- | --- | --- | --- |
| KT | ⋅IT Telecom ⋅Unmanned Solution ⋅E-Intelligence | ⋅Intersection safety ⋅Commute Shuttle bus ⋅Personal mobility | ⋅Seoul Sangam ⋅Gyungi Pangyo ⋅Daegu Alphacity | eMBB/URLLC |
| Autonomous Driving |  |  |  |  |
| ETRI | ⋅KT ⋅Jinyoung I&C ⋅Intellivix | ⋅Smart Street ⋅Safety/Convenience | ⋅Daejeon ⋅Daegu | eMBB/mMTC |
| Smart City |  |  |  |  |
| SKT | ⋅KETI ⋅Wizcore ⋅Schaffler Korea | ⋅Machine Vision ⋅Multi-function robot ⋅Facility mgmt. AR | ⋅Ansan ⋅Banweol | eMBB/URLLC |
| Smart Industry |  |  |  |  |
| KT | ⋅ETRI ⋅TI Square ⋅Texcell Netcom | ⋅Fire/Tunnel accident ⋅Pedestrian accident intelligent support | ⋅Gangwon Prov. ⋅Pyeongchang | eMBB/mMTC |
| Disaster/ Security |  |  |  |  |
| SKB | ⋅SKT ⋅Looxid ⋅Avavision | ⋅XR service for underprivileged ⋅360/interactive VR | ⋅Busan city | eMBB/URLLC |
| AR/VR |  |  |  |  |

New Zealand

Two industry-led 5G trials were undertaken in New Zealand since 2018. The first trial was conducted by one mobile network operator in the frequency ranges 3.6-3.7 GHz and 26.65-27.45 GHz for outdoor trials in Auckland and Wellington. The second trial was conducted by another mobile network operator in the frequency range 27.525-28.325 GHz for indoor and restricted outdoor trials in Auckland.

Singapore

Singapore has waived the frequency fees associated with 5G trials from May 2017 to December 2019 to facilitate the deployment of 5G services in Singapore. A total of 14 different frequency bands are eligible for frequency fees waiver for 5G trials if their use cases fall under one of the three categories defined by ITU-R (i.e. eMBB, uRLLC, mMTC) and subject to assessment of trial plans and the technical conditions of use. More information is available from this url:

https://www.imda.gov.sg/regulations-licensing-and-consultations/frameworks-and-policies/spectrum-management-and-coordination/spectrum-planning/5g-technology

Thailand

Thailand has launched Phase One of 5G pilot project starting from August 2020. The project deliverables include:

1. Smart Hospital (Location: Bangkok): by applying 5G technology (cloud computing, AI, Big Data and IoTs) on 2600 MHz band to improve patient experience and hospital operation such as wearable devices for personalized treatment, AI assisted diagnosis monitoring and prediction treatment, remote health monitoring (telemedicine) and tracking inventory.

2. Smart Agriculture (Location: Chiang Rai): by using IoTs, AI and Big Data on 2600 MHz band to collect data and automate management, through sensors, for analyzing, monitoring and forecasting to increase productivity.

Phase Two of the 5G pilot project, which consists of several sectors such as Smart Railway Station, Smart Irrigation and Smart Manufacturing, is currently in progress.

Viet Nam

5G licences have been granted for technical trial and services trial in small and medium scale. Spectrums in low-band (sub 1 GHz), mid-band (1-6 GHz) and high-band (mmWave) also be released for such trials.

Some major information of trial cases are below:

- Network architecture: 5G NSA (option 3x), EN-DC with existing 4G-LTE

- BTS: DRAN/CRAN with massiveMIMO 64T64R (some are homemade)

- Spectrum: 100 MHz 5G TDD @ mid-band, with anchor of 20 MHz 4G FDD @ 1800 MHz

- Transportation network: Site router 100 Gbps (some are homemade)

- TDD synchronization: GNSS synchronization signal and 1588v2 hot-standby

- Use case: Video 4K/Basic AR/VR, Video 8K/Immersive AR/VR, mobile internet access, fixed wireless access

Experiment result of 5G service:

- Peak downlink data rate: 1200 Mbps

- Average downlink data rate: 500 Mbps

- Average uplink data rate: 67 Mbps

- User experienced data rate: 300 Mbps @ 10% point of CDF; 100 Mbps @ 0% point of CDF of user throughput.

Roadmap for commercialization of IMT-2020 (5G)

Australia

Licences in Australia are issued on a technology flexible basis. This allows licensees to develop their own timeframes for the deployment of new technologies.

On 28 May 2019, Telstra launched commercial 5G services using the 3.6 GHz band at more than 200 sites across 10 cities. The roll-out is still in its early stages and will grow in the coming months and years. Telstra notes that the ACMA is prioritizing a 26 GHz spectrum auction where 5G is expected to be the deployed technology.

Bangladesh

Bangladesh is likely to adopt IMT 2020 (5G) Service by 2020

China

The operators in China pay a lot of attention on 5G, they make great efforts on 5G R&D and standardization. Currently, the operators are working on their initial strategy of 5G network evolution and pre-commercial plan. The operators plan to conduct large-scale pre-commercial trial with more than 100 sites per city (several cities will be chosen) in 2019 and deploy more 5G commercial sites in 2020.

China issued 5G commercial operation licenses on 6th June to 4 mobile network operators in 2019. This document provides an introduction on 5G system implementation of Chinese MNOs.

China Telecom, China Unicom, China Mobile and China Broadcasting Network obtained 5G commercial operation licenses and announced their 5G Logos, listed in Table below in detail.

Table 5G commercial operation licenses in China

| Mobile network operator | 5G brand Logo |
| --- | --- |
| China Telecom | 5GHello |
| China Unicom | 5Gn |
| China Mobile | 5G+ |
| China Broadcasting Network | 5GG |

China Telecom

China Telecom has conducted 5G innovation demonstration trials in 17 cities including Beijing, Shanghai and Shenzhen, and is planning to expand to more than 40 cities. China Telecom has extensive cooperation in 5G application innovation to jointly build a new 5G ecosystem. It has developed more than 200 trial customers and partners, focusing on key business scenarios of vertical industries of government affairs, manufacturing, transportation, logistics, education, medical care, media, police, tourism and environmental protection.

China Unicom

In April 2019, China Unicom officially opened 5G pre-commercial services in 7 cities (Shanghai, Beijing, Guangzhou, Shenzhen, Nanjing, Hangzhou and Xiong’an). 5G is accessible in the World Garden Expo and Winter Olympics Stadiums in Beijing.

China Unicom released the "7+33+N" network deployment plan, which is to provide a continuous urban coverage in the 7 cities, hotspot coverage in other 33 cities and 5G dedicated networks for vertical industries in N cities to promote 5G application and industrial upgrading.

China Unicom established the 5G Application Innovation Alliance with partners from Intelligent Transportation, New Media, Industry Internet, Medical care, Education and Tourism. Several 5G trials have already been launched in vertical industries.

China Unicom also released its 5G terminals, including 6 smart phones, 5 industry terminals and 4 modules.  5G Terminal Innovation Research and Development Center will be established.

China Mobile

In December 2018, China Mobile launched 5G scale trials. Trial networks have been built mainly in five cities in China (Hangzhou, Guangzhou, Shanghai, Suzhou and Wuhan), in which more than 500 stations were built and the trial covered 26 scenarios. SA and NSA chipsets were available in the first quarter in 2019 and 5G terminals in the second quarter of 2019. Pre-commercial use of 5G NSA and SA vertical industries is expected to start in the second half of 2019. And scale commercial use will begin in 2020.

China Mobile has established 5G Joint Innovation Center in 2016, complaining partners from network equipment, instruments, chipsets and terminals to give full play to the influence of 5G eco-system and promote integrated development with various industries.

India

Department of Telecommunications recently auctioned spectrum in 700, 800, 900, 1800, 2100, 2300, and 2500 MHz bands. The details of auction result are available at https://dot.gov.in/spectrum-management/2828. The government of India is also planning to open up other spectrum bands which were identified for IMT in WRC-19 for deployment of IMT-2020.

Indonesia

At present, the addition of radio frequency spectrum that can be used for the 5G implementation has been included in the National Medium Term Development Plan (RPJMN) target for 2020-2024. It is expected that 5G can be implemented within this time period by also considering the demand for 5G needs in Indonesia. By 2024, it is projected that we need at least 1882 MHz spectrum bandwidth. Currently, in the year of 2020, our mobile broadband spectrum availability is 767 MHz. Our target is to achieve additional spectrum bandwidth at least 1310 MHz in a period of 2020-2024 by releasing spectrums in low, middle, and high band. Recently, Indonesia has stipulated a primary legislation which supports the presence of 5G in Indonesia through a more comprehensive sharing policy.

Indonesia is still in the process of conducting trials and related studies in order to develop the roadmap of IMT-2020 (5G) implementation in Indonesia. In order to accelerate the development of the roadmap, Minister of Communications and Informatics has formed 5G Task Force in August 2020 that consist of 5 Working Groups:

- Working Group Radio Frequency Spectrum

- Working Group Business Model

- Working Group Infrastructure

- Working Group Device and Ecosystem

- Working Group Regulation

Indonesia has launched 5G commercial service started from May 27th 2021 in existing frequency bands since Technology Neutrality policy is adopted based on IMT standard for mobile broadband. Three cellular operators which launched 5G commercial service as follow:

- PT Telekomunikasi Selular on May 27th 2021 in 9 cities by deploying 5G Non-Stand Alone (NSA) using E-UTRAN New Radio Dual Connectivity (ENDC) feature on 2300 MHz (n40) frequency band with 30 MHz bandwidth (plan to be extended up to 50 MHz in some areas).

- PT Indosat Tbk. on June 22nd, 2021 in 6 cities by deploying 5G NSA using Dynamic Spectrum Sharing (DSS) feature on 1800 MHz (n3) frequency band with 20 MHz bandwidth.

- PT XL Axiata Tbk. on August 18th, 2021 in 4 cities by deploying 5G NSA using DSS feature on 1800 MHz (n3) frequency band with 20 MHz bandwidth.

Iran

In Iran research and pilot projects on new applications such as car networking as a new 5G future markets have been started. We expect that the volume of 5G commercialization will grow by the new 5G applications, similar to global trends.

Japan

We have launched the commercial 5G service in March 2020 successfully. Towards this commercialization, we had conducted various R&D/field trial projects, international collaboration/cooperation, and development of 5G technical specifications as summarized in Figure below.

Figure Roadmap for commercialization of 5G in Japan

Republic of Korea

The Republic of Korea is preparing for the world`s first country to commercialize 5G based on our 5G policy experience. In the 2019, when 5G is commercialized, the Republic of Korea will promote trial service in various fields using 5G technology. In 2022, the Republic of Korea plans to promote more advanced services such as autonomous vehicle with 5G.

(Source: Ministry of Science and ICT)

Figure 5G Roadmap in Korea

New Zealand

New Zealand is in the process of developing a Spectrum Roadmap for 5G. The roadmap will likely cover government’s policies for facilitating the timely release of multiple spectrum bands and the deployment of 5G network based on use cases of eMBB, URLLC and mMTC.

New Zealand held a series of industry workshops in late October 2017 to discuss views on 5G requirements, possible 5G applications and 5G spectrum demands with stakeholders. Subsequently, a formal public consultation commenced in March 2018. Submissions closed on 30 April 2018. Refer to https://www.rsm.govt.nz/projects-auctions/current-projects/preparing-for-5g-in-new-zealand/submissions-received.

Singapore

Singapore plans to publish our plans and roadmap for 5G around the end of 2018. More information will be made available in this url:

https://www.imda.gov.sg/regulations-licensing-and-consultations/consultations/consultation%20papers/2017/public-consultation-on-5g-mobile-services-and-networks

Thailand

Thailand has already launched the commercial 5G services since February 2020.

Viet Nam

According to research by the National Institute of Information and Communications Strategy of MIC-Viet Nam, 5G's contribution to GDP growth is forecast to reach 7.34% by 2025. World Bank research also shows that for every 10% increase in subscribers broadband, GDP will increase by 0.1% respectively.

The Ministry of Information and Communications has also determined that 5G commercial deployment in Vietnam will be proactive and at the same pace with the world. The initial 5G commercial plan will be in the end of year 2021.

Other information for 5G preparations

Australia

The Australian Government considers that 5G is more than an incremental change for mobile communications. Instead, it provides the underlying architecture that will enable the next wave of productivity and innovation across different sectors of the Australian economy. Efficient rollout of 5G and uptake of the services it supports has the potential to produce far-reaching economic and social benefits and support growth of Australia’s digital economy. This will be supported by the rollout of the National Broadband Network (NBN) allowing greater capabilities for the seamless delivery of services across high speed mobile, fixed line and fixed wireless networks.

The Australian Government wants to create an environment that allows Australia’s telecommunications industry to be at the forefront of seizing the benefits of 5G across the economy. The communications sector will lead the rollout of 5G networks in Australia. However, the Government can create the policy and regulatory environment to support a more efficient rollout, given its potential benefits to the economy.

The Australian Government’s direction will be to support the timely rollout of 5G in Australia to enable the next wave of broad-based industry productivity, and support the growth of Australia’s digital economy.

Further information is included in Australian Government’s 5G—Enabling the future economy paper published by the Department of Communications and the Arts.

Bangladesh

We have conducted one trial/demonstration with only one of the mobile phone operators. However, we may conduct further demonstration by other operators in near future before the launch of 5G service.

China

In WRC-19, China proposed 5925-7125MHz, or parts thereof,  for WRC-23 studies for IMT identification as additional mid-band to drive future 5G development together with other APT countries including Afghanistan, Cambodia, Lao, Mongolia, Myanmar, Nepal. In order to accommodate different views, it was finally agreed to study frequency-related matters for the terrestrial component of IMT identification 7025-7125 MHz in all 3 regions and 6425-7025 MHz in Region 1, which was included in WRC-23 AI 1.2.

In the future, additional spectrum within the middle range is needed to enable future China 5G deployment and meet the users’ increasing requirement of anytime anywhere high data rate communications. The spectrum within the 5 925-7 125 MHz frequency band can partially meet future IMT spectrum needs. In WRC-19, China proposed 5925-7125MHz, or parts thereof, for WRC-23 studies for IMT identification as additional mid-band to drive future 5G development together with other APT countries, i.e. Afghanistan, Cambodia, Lao, Mongolia, Myanmar, Nepal and PNG. China has started the studies on 6GHz and will contribute to ITU-R as appropriate.

China is in favor of the identification of the identification of 6GHz band for IMT (for different usage scenarios including Industrial Internet scenario) with the appropriate technical conditions (protecting incumbent services), within the framework of ITU-R.

IMT-2020 Candidate Technology Submission of China

China submitted the IMT-2020 candidate technology to ITU-R WP 5D on IMT-2020 terrestrial radio interfaces (Seoul, January 2018) and then proposed the RIT developed in accordance with the latest 3GPP Technical, including NR plus NB-IoT as a RIT at the 29th ITU-R WP 5D meeting (Japan, October 2018).

China submitted a contribution on final proposal for IMT-2020 candidate technology based on 3GPP technology (NR+NB-IoT) at the 32nd ITU-R WP 5D meeting (Brazil, July 2019).

India

National Digital Communication Policy (https://dot.gov.in/sites/default/files/EnglishPolicy-NDCP.pdf) is oriented towards next-generation-networks like 5G. Besides this, the 5G high level forum for 5G has ambitious plans for 5G preparations. Premier institutions in the country have planned indigenous 5G test beds in India. Besides this, the 5G high level forum for 5G has ambitious plans for 5G preparations.

Indonesia

In Asian Games XVIII (August 18th – September 2nd 2018, a Joint Cooperation between Ministry of Communications and Informatics (MCI) Republic of Indonesia and Ministry of Science and ICT (MSIT) Republic of Korea, produced a showcase and trial of IMT-2020 (5G) in the city of Jakarta. The 5G showcase was operated and supported by PT Telekomunikasi Selular (TELKOMSEL).

Newly enacted Law of the Republic of Indonesia No. 11 of 2020 (Omnibus Law) mandated ASO date on analogue TV is targeted to be completed on 2 November 2022 at the latest. The possibility of having additional radio frequency spectrum at 700 MHz from Digital Dividend that can be used for mobile broadband is expected. This regulation allows telecommunication operators to share the spectrums for the advanced technology such as 5G, which will create an ultimate efficiency combining with the share of passive infrastructure and active networks.

Indonesia has established OpenRAN laboratory since July 2020, the first community laboratorium in South East Asia which is jointly supported by Telkom University and Telecom Infra Project (TIP) community lab to accelerate and support the implementation of 5G networks in Indonesia.

Republic of Korea

IMT-2020 Candidate Technology Submission of Korea

The Republic of Korea indicated the willingness to submit the IMT-2020 candidate technology to ITU-R WP 5D in Workshop on IMT-2020 terrestrial radio interfaces (Munich, 4 October 2017) and then proposed RIT (initial description) developed in accordance with the latest 3GPP New Radio Technical Specifications (Release 15) at the 29th ITU-R WP 5D meeting (Seoul, January 2018).

The Republic of Korea will submit a contribution updated proposal for IMT-2020 candidate technology based on 3GPP New Radio (NR) RIT in the future ITU-R WP 5D meeting.

Figure IMT-2020 candidate technology submission plan of Korea and 5G 28GHz Frontier Workshop Activity

After WRC-15, among several administrations and mobile industry there were demands to discuss the 28 GHz band for 5G continuously outside of ITU-R/WRC. It was decided to organize the 28 GHz Frontier Workshop according to the agreements and efforts from administrations and mobile industries to share recent actions and developments related to the 28 GHz frequency band and to discuss possible steps to progress further towards opening a global mass market for 5G equipment and services in this 5G Frontier band.

The 1st 28 GHz Frontier Workshop was held in December 2016 at TTA and 3rd 28 GHz Frontier Workshop was held in January 2018 at Seoul in Korea. The 5th 28 GHz Frontier Workshop will be held in October 2018 at Fukuoka in Japan. All the relevant information of 28 GHz Frontier band can be seen at the 28 GHz Frontier Website*.

*URL: http://5g-28frontier.org/

New Zealand

For more IMT-2020/5G information, please refer to https://www.rsm.govt.nz/projects-auctions/current-projects/preparing-for-5g-in-new-zealand

Singapore

More IMT-2020/5G information will be made available in this url:

https://www.imda.gov.sg/regulations-licensing-and-consultations/consultations/consultation%20papers/2017/public-consultation-on-5g-mobile-services-and-networks

Thailand

Thailand is studying the possibility of refarming 3500 MHz band from fixed satellite service to IMT to support future demand for 5G usage.

Viet Nam

Mobile operators plan to deploy 5G NSA network in the first phase and synchronously deploy SA from second phase when the ecosystem and demand for mMTC and URLLC services, terminals SA support becomes more popular.

Mobile operators also are trying Open RAN technology to identify the technical readiness, performance and requirements.

Ministry of Information and Communication is considering a novel regime of 5G infrastructure sharing in order to facilitate the fastest implementation of nationwide 5G.

Activities of Industry for IMT-2020(5G) in Asia-Pacific region

Standardisation status and plans onwards

ITU-R

For more than 30 years, ITU-R has been coordinating efforts of government, industry and private sector in the development of a global broadband multimedia international mobile telecommunication system, known as IMT. In early 2012, ITU-R embarked on a program to develop “IMT for 2020 and beyond”, setting the stage for 5G research activities that are emerging around the world. Through the leading role of Working Party 5D, ITU-R has finalized its view of a timeline towards IMT-2020, which can be found in Figure below. The detailed investigation of the key elements of 5G are already well underway, once again utilizing the highly successful partnership ITU-R has with the mobile broadband industry and the wide range of stakeholders in the 5G community.

In September 2015, ITU-R has finalized its “Vision” of the 5G mobile broadband connected society. The latest member of the IMT family is IMT-2020 which calls for support for enhanced mobile broadband (eMBB) and for new ‘use cases’ that require massive machine-type communications (mMTC) and ultra-reliable and low latency communications (URLLC).

Figure Timeline and Process for IMT-2020 in ITU-R

On October 4, 2017 ITU-R WP5D organized in Munich, Germany, a Workshop on IMT-2020 terrestrial radio interfaces. The Workshop was organized with three main objectives, including explained the procedures for submission of proposals and subsequent acceptance, introduced possible proposals of radio interface technologies to be included in IMT-2020, and introduction of the Independent Evaluation Groups (IEG) which verify the performance of candidates.

After the Workshop through the essential process of proposals, consensus building and evaluation by IEGs, ITU-R WP5D finally completed the IMT-2020 specifications in November 2020. This has been approved by the ITU Member States as ITU Recommendation M.2150 in Feburary 2021, including “3GPP 5G-SRIT”, “3GPP 5G-RIT” and “5Gi” .

3GPP

The 3rd Generation Partnership Project (3GPP) unites seven telecommunications standard development organizations (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC), known as “Organizational Partners” and provides their members with a stable environment to produce the Reports and Specifications that define 3GPP technologies.

Figure Timeplan on 3GPP self-evaluation

From 2015, 3GPP aims to the definition of a full system (Radio and Core Network), and 3GPP specifications is labeled “5G” from Release 15 onwards. Further 5G standards evolution is realized in 3GPP Release 16. The work programme for Release 16 was agreed in 2018 and covers various functions for use of 5G in vertical markets (such as 5G V2X, industrial IoT and URLL improvements, as well as use of 5G NR in unlicensed spectrum bands). It also covers generic improvements in areas such as positioning, use of MIMO and power consumption. Release 16 specification was released in July 2020.

Release 17 - the third release of 5G specifications - has been under pressure due to COVID-19 restrictions, but despite making the move to e-meetings, the group’s experts have managed to ensure positive progress towards the freeze of the RAN1 physical layer specifications on schedule, by December 2021. This is to be followed by the Stage 3 freeze (RAN2, RAN3 and RAN4) by March 2022 and the ASN.1 freeze and the performance specifications completion by September 2022.

For the start of work on 5G-Advanced, online RAN workshop was held from June 28 to July 2,2021 to define what will be in the Release 18. The on-going work will culminate with Release 18 Package Approval, at the December 2022 Plenary (RAN#94). During the Release 18 planning process, some capacity must be kept in hand; keeping around 10% of WG effort in reserve, for workload management and to meet late, emerging critical needs from commercial deployments.The high-level objective of the workshop was to gather company proposals in three areas:

- eMBB driven work;

- Non-eMBB driven functionality;

- Cross-functionality for both.

From Spectrum aspect:

The frequency ranges in which NR can operate according to this version of the specification are identified below table:

Table 3GPP NR frequency range

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1 | 410 MHz – 7125 MHz |
| FR2 | 24250 MHz – 52600 MHz |

The following NR bands, including FR1 and FR2 have been specified as in Table 5.2-1 of 3GPP TS 38.101 (https://www.3gpp.org/ftp/Specs/archive/38_series/38.101-1 and https://www.3gpp.org/ftp/Specs/archive/38_series/38.101-2).

In addition, 3GPP has started the standardization of new frequency bands of 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz, as in New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz.Furthermore, a New Work Item(WI) on introduction of 6GHz NR licensed bands is approved in 3GPP RAN#89e meeting.

From 5G/IMT-2020 evaluation aspects:

Then, 3GPP self-evaluation was triggered from 2017; as well as a specific evaluation plan was approved. Based on the evaluated results presented, 3GPP concluded that there is a high level of confidence in the community that 3GPP’s submission to meet the ITU requirements.

In conclusion, IMT-2020 standards and evaluation activities reflect perfectly 5G research project’s pre-standards work and also the priorities for ITU-R WP5D and 3GPP work from Release 15 onwards. The alignment between IMT-2020, 5G R&D and the 3GPP work plan is an important achievement, which helped greatly at the World Radiocommunication Conference 2019, where deliberations on additional spectrum in support of IMT took place.

Worldwide status of 5G trials

This information is provided on global basis statistics. The latest GSA data reveals that 413 operators in 131 countries/ territories are investing in 5G networks in the form of tests, trials, pilots, planned and actual deployments. Globally, many mobile operators have announced over 600 separate demonstrations, tests or trials that we have been able to identify.

Early tests and trials focused on: new radio (NR) interfaces operating in spectrum bands not previously used for mobile telecoms services; network slicing to support delivery of services tailored to specific types of customer or service; combinations of technologies such as massive MIMO, or complex beam-forming that are needed to achieve very high speeds; and backhaul, cloud- and edge-computing arrangements to support very low latencies. Tests being conducted by more advanced players then evolved to include: the launch of pilot 5G networks; delivery of calls end-to-end; and tests of 5G applications such as 5G connected drones, stadium applications, holograms and connected vehicles. Recent tests have been looking at issues such as: interoperability of SA 5G NR with core network systems; delivery of 4K ultra-HD CCTV and broadcast services; and use of 5G for robotic surgery. Trials have also focused on testing the spectrum bands likely to be used in any given country, with a lot of activity in C-band spectrum and mmWave.

Network throughput

One of the key metrics that has been reported is the peak downlink throughput of the various demonstrations, tests and trials. The demonstrations and trials are not really comparable, as they use varying amounts of spectrum and different types of equipment, in contrasting physical environments and for a range of applications. Nonetheless, it is interesting to note that many of them report that speeds well in excess of 1 Gigabit per second have been achieved. Trials for very high speeds are proofs of concept; it is not expected that commercial 5G networks will be able to deliver the very highest speeds indicated in the chart below for some time to come. Figure below summarizes the results of the demonstrations and trials where information has been made available: 216 in total.

Figure Network throughput (DL) reported in 5G demonstrations and trials (base: 216 demos/ trials)

Source: GSA, Global Progress to 5G – Trials, Deployments and Lunches

Speeds reported have most commonly fallen into the 1 to 4.99 Gbps range. Within this range, the average speed reported in the trials is 2.26 Gbps. If these speeds are eventually realised in commercial networks, they would represent substantial improvements on current LTE speeds. In some LTE networks, DL maximum throughput speeds have now reached 1.2–1.6 Gbps within small geographical pockets, but in networks into which LTE-Advanced technologies have been introduced, the average maximum DL throughput remains 399 Mbps (based on data for 259 networks). Data on the maximum DL throughput speeds available from early 5G services is not commonly provided by the operators yet. GSA will track this information as it is released to assess to real speeds of live 5G networks.

Latency

5G networks are expected to have substantially reduced latency compared with current mobile networks. This is another key metric for demonstrations and trials, as vendors and operators seek to achieve the 5G benchmarks. Once again, Figure below reported by demonstrations and trials are not comparable, as they involve very different configurations (air interface latency, ‘end-to-end’ latency, etc.) and even in field trials, do not take place under real network conditions but are illustrative of the fact that low latencies are being achieved. It is interesting to point out that most trials (for which data has been reported) have achieved latencies of between 1 and 1.99 ms.

Figure Latencies reported in 5G demonstrations and trials (base: 69 demos/trials)

Source: GSA, Global Progress to 5G – Trials, Deployments and Lunches

Early commercial 5G implementations and views from operators

This information is provided on global basis statistics. Until February 2021, The Global mobile Suppliers Association (GSA) announced that 61 countries/territories now have live 5G networks. As of the end of January 2021, the GSA recorded 144 commercial 5G networks present worldwide with more to come.

169 individual operators have launched commercial 5G networks (mobile and/or FWA). Frequency information is available for 72% of these: 78% of 5G networks have been launched using mid-band spectrum. However, operators are advancing the launch of 5G services that use mmWave bands as the assignment of spectrum in this band paces up.

Sources: GSMA Intelligence in Q2 2021

Figure 7-3-1 5G launched commercial spectrum by band range

3.5 GHz remains the main choice – The 3.5 and 3.7 GHz bands accounts for 50% of 5G network launches (excluding frequencies not reported). However, as operators expand their 5G footprint, 700 MHz is emerging as the key coverage band, accounting for nearly 65% of the 5G network launches in low-band spectrum. As 5G spectrum has yet to be assigned in many markets, operators are using DSS and also refarming existing spectrum bands to support 5G deployments and launches. Refarming is mainly seen in 1800, 2100 and 2600 MHz bands.

Sources: GSMA Intelligence in Q2 2021

Figure 5G network launches by spectrum frequency (up to Q2 2021)

GSMA’s report on mid-band spectrum needs for 5G implementations from 2025 onwards

GSMA conducted a study to consider how much mid-band spectrum will be required to fulfil the ITU’s requirements for IMT-2020 and especially how sensitive user data rate and area traffic capacity are to spectrum capacity. While several of 5G capabilities are inherent in 5G technology, to ensure a consistently high user experienced data rate and a high area traffic capacity more spectrum is required.

The report considered several Asian cities amongst its global sample, and these showed varied results as would be expected from a large and varied continent. Some of Asia’s cities have amongst the highest population densities in the world and this fact is clearly reflected in the data which shows high mid-band spectrum demand in some cases.

5G is being used in countries across the world as the central pillar of economic development. This was true before the COVID pandemic, but as the world seeks to recover from the economic impact of COVID, the importance of 5G and its ability to have a profound impact on productivity throughout the global economy has become universally clear.

The GSMA’s findings highlight some of the barriers to wider mobile take-up, including the assignment of sufficient mobile spectrum. Although spectrum in these bands is increasingly scarce, providing sufficient mid-band spectrum for affordable 5G can provide a robust driver for economic growth towards 2030.

The possibilities for digital society and industrial development

One of the obvious differences between IMT-2020 and previous generations of cellular networks lies in IMT-2020’s strong focus on machine-type communication and the Internet of Things (IoT). The capabilities of IMT-2020 thus extend far beyond mobile broadband with ever increasing data rates. More specifically, IMT-2020 supports communication with unprecedented reliability and very low latencies, and also massive IoT connectivity. This paves the way for numerous new use cases and applications in many different vertical domains, including automotive, healthcare, agriculture, energy and manufacturing sectors. In manufacturing in particular, IMT-2020 may have a disruptive impact as related building blocks, such as wireless connectivity, edge computing or network slicing, find their way into future smart factories.

In order to ensure the success of 5G for vertical industry aspect, there are several points for further consideration. For example, it is necessary on globally or regionally harmonized spectrum, for both licensed and license-exempt allocations. Moreover, the ability to deploy and operate private 5G networks in well-defined areas (e.g. within a factory). The required spectrum could be acquired directly from the spectrum regulators through regional/ local licenses or – in some cases – obtained from a mobile network operator through subleasing in a specific geographic area. Besides, it is important for indoor and outdoor user equipment positioning to reach certain level of accuracy. Additionally, the ability to monitor the current network state continuously in real-time even as a user, to take quick and automated action in the event of problems and to perform efficient root-cause analyzes. Meanwhile, E2E network slicing across heterogeneous technologies, countries and network operators, with facility for dynamic and user-friendly establishment and release of fine granular, application-specific network slices characterized by well-defined QoS and security properties. Furthermore, other factors also can further create more advantages for industrial development by IMT-2020 technologies, such as very low E2E latencies, high synchronicity between different devices, potentially ultra-high communication service availability, appropriate security concepts, highly flexible and versatile air interface capable and Support of multiple tenants using the same physical connectivity infrastructure in a factory.

From ITU-R side, ITU-R WP5D has a new Sub-Working Group under the WG General Aspects called “Specific Applications” to study under Question 262/5 “Usage of the terrestrial component of IMT systems for specific applications”, including PPDR, what are the specific industrial applications with their emerging usages, and their functionalities, that can be supported by IMT. Currently, the new Sub-Working Group is developing of new Report ITU-R M.[IMT.C-V2X] “The use of the Terrestrial Component of IMT for Cellular-Vehicle-to-Everything Application”, which will address the mutual relationship between IMT technologies and Cellular-Vehicle-to-Everything(C-V2X) as specific application and elements of functions in IMT technologies. Future, other possible vertical applications supported by IMT technologies will be studied in this SWG.

Furthermore, more specifically, from 3GPP side, in order to be able to deploy 5G for new and diverse use cases, including industry 4.0 and factory automation, the 5G system must work in harmony with the communications technologies used in such industries. For this purpose, 3GPP has now made significant progress towards the integration of the 5G system with IEEE 802.1 working group specifications covering Time Sensitive Networking (TSN).

The ‘Vertical_LAN’ work item, in 3GPP Release-16, introduces the following three new and distinct 5G enablers for Industry 4.0;

- Support for Time Sensitive Communications by integrating the 5G system as a bridge to IEEE TSN.

- Support for Non-Public Networks.

- Support for a 5G-LAN type service.

- _______________________

Class licensing—broad spectrum access arrangements which are open to all users. Intention is to facilitate localised private 5G networks

Spectrum licensing—use of a specified frequency band in a defined geographical area

Apparatus licensing—use of individual devices, usually on a site-specific basis. Intention is to facilitate localised private 5G networks

https://www.itu.int/md/R16-WRC19-C-0110/en

Government of India Press Release on 5G Trials

https://www.itu.int/md/R16-WRC19-C-0110/en

ITU-R WP5D website on “On the road to IMT-2020 and the globalization of 5G.” ( https://www.itu.int/en/ITU-R/study-groups/rsg5/rwp5d/Pages/on-road-IMT-2020.aspx)

RP-200513, New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz, RAN#87e, Electronic Meeting, March 16-19, 2020

RP-202114, New WID on introduction of 6GHz NR licensed bands, RAN#89e, Electronic Meeting, September 14-18, 2020

GSA, Global Progress to 5G – Trials, Deployments and Lunches.

https://www.gsma.com/spectrum/resources/gsma-intelligence-mobile-spectrum/

Note: These figures refer to launches, not to individual operators. A range of operators have launched their 5G networks on more than one frequency.

The full study can be accessed from GSMA website (www.gsma.com).

3GPP WID: 830042 (Vertical_LAN) 5GS Enhanced support of vertical and LAN services.