Report of the Agenda Item Coordinator during WRC-19

Name and email of the Coordinator: Phung Nguyen Phuong,

phuongpn@rfd.gov.vn

Report Date: 01/11/2019

- Agenda Item 7 issue C1, C2, C3, C6, C7

To consider possible changes, and other options, in response to Resolution 86 (Rev. Marrakesh, 2002) of the Plenipotentiary Conference, an advance publication, coordination, notification and recording procedures for frequency assignments pertaining to satellite networks, in accordance with Resolution 86 (Rev.WRC-07), in order to facilitate rational, efficient and economical use of radio frequencies and any associated orbits, including the geostationary-satellite orbit.

Issue C1 - Inconsistency between provisions of RR No.11.43A of RR Article 11 and paragraph 8.13 of Article 8 of RR Appendix 30B

Issue C2 - Clarification of the possibility to notify/bring into use only one of the blocks/one sub-band under AP30B Article 6

Issue C3 - AP30B MOD to Article 6 No. 6.10

Issue C6 – Single AP4 notice for entry into the RR Appendix 30B List (under § 6.17) and Notification (under § 8.1)

Issue C7 – Harmonization of AP30B with AP30/30A on Possibility of Obtaining Agreement for a Specific Period

- APT Common Proposals and APT Views for WRC-19 (which has been submitted to WRC-19)

Issue C1 - Inconsistency between provisions of RR No.11.43A of RR Article 11 and paragraph 8.13 of Article 8 of RR Appendix 30B

- APT Members support the single method in the draft CPM text to address this issue by aligning the text of paragraph 8.13 of Article 8 of RR Appendix 30B with that of RR No. 11.43A of RR Article 11 while ensuring that this alignment should not impact on any other current regulatory practice.

Issue C2 - Clarification of the possibility to notify/bring into use only one of the blocks/one sub-band under AP30B Article 6

- APT Members support the single method in the draft CPM text which can allow administrations to submit an application for one of the blocks/sub-bands of 250 MHz (10.7-10.95 GHz or 11.2-11.45 GHz for downlink and 12.75-13.0 GHz or 13.0-13.25 GHz for uplink) in an explicit submission of one of the blocks/sub-bands under RR Appendix 30B.

Issue C3 - AP30B MOD to Article 6 No. 6.10

- APT Members support the single method in the draft CPM text to add a new provision in Article 6 of RR Appendix 30B to clearly state that § 6.13 to 6.15 of RR Appendix 30B do not apply in the context of requirements associated with §6.6 of RR Appendix 30B.

Issue C6 – Single AP4 notice for entry into the RR Appendix 30B List (under § 6.17) and Notification (under § 8.1)

- APT Members support the single method in the draft CPM text to allow a single submission to be treated both in respect of entry into the List under §6.17 and notification under §8.1 of RR Appendix 30B to reduce workload of both administration and the Bureau.

Issue C7 – Harmonization of AP30B with AP30/30A on Possibility of Obtaining Agreement for a Specific Period

APT Members support the single method in the draft CPM text to add a new provision 6.15bis to Article 6 and a new provision §8.16bis to Article 8 of RR Appendix 30B in order to recognize the possibility of obtaining agreement from affected administrations for a specified period. It is also proposed to modify § 5.2.6 of Article 5 of Appendix 30A to Radio Regulations.

- Topics proposed by other regional Groups or ITU Members which are not included in no. 2 above

| RCC | CITEL | CEPT | ATU | ASMG | APT |
| --- | --- | --- | --- | --- | --- |
| Support single method | Support single method | Support single method | Support single method | Support single method | Support single method |

- Progress of discussion during WRC-19 on the Agenda Item

Based on the common support of the method to satisfy these issues, the meeting agreed to create the Note by the Chairman of Committee 5 to Plenary on these issues. During the discussion on Com 5, the meeting considers the minor change on the regulatory texts for issues C1, C2 and adds the note to issue C6. This document agreed and would be sent to Committee 7. The Plenary would be invited to endorse this conclusion.

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

None.

Note: Coordinators are encouraged to conduct informal consultation with interested APT Members on the issues/topics under no. 3 and inform the outcomes of consultation to the Coordination Meeting. Coordinators can also organize coordination meetings on the respective agenda items whenever necessary.