Report of the Agenda Item Coordinator during WRC-19

Name and email of the Coordinator:Iraj Mokarrami

Iraj.mokarrami@cra.ir

Report Date: 08/11/2019

- Agenda Item 7 issue A

Bringing into use of frequency assignments to all non-GSO systems, and consideration of a milestone-based approach for the deployment of non-GSO systems in specific frequency bands and services

- APT Common Proposals and APT Views for WRC-19 (which has been submitted to WRC-19)

BIU definition:

Continuous period of 90 days for frequency assignments of the FSS/MSS/BSS, and no fixed period for frequency assignments other than the FSS/MSS/BSS.)

With respect to the regulatory provision No. 11.44C of the BIU, notified orbital planes, APT Members could support Option 2, as outlined in the CPM19-2 report.

Milestone based approach:

| Milestones | Milestone timing (Number of years after the end of the seven-year regulatory period or after 1st January 2021, whichever falls later) | Minimum required % of satellites deployed to meet the milestone |
| --- | --- | --- |
| 1st | 2 to 3 years | 10% |
| 2nd | 4 to 5 years | 30-50% |
| 3rd | 7 years | 90-95% /100% |

Transitional Measures : Option1(the commencement date of the milestone process: 1 Jan. 2021)

Frequency bands and services for application of the milestone-based approach

APT Members support application of the milestone-based approach to non-GSO systems operating in the FSS, BSS and MSS, but not those operating in the RNSS.

APT Members do not object, at this stage to the application of the approach to the following MSS frequency bands for which no consensus was reached and listed in the CPM Report: 137-137.025 MHz, 137.025-137.175 MHz, 137.175-137.825 MHz, 137.825-138 MHz, 148-149.9 MHz, 149.9-150.05 MHz (This band 149.9-150.05 MHz appears to have been mistakenly listed in the CPM Report as 137-138 MHz), 399.9-400.05 MHz, and 400.15-401MHz.

Milestone based approach: consequences of non submission of milestone information (resolves 11 to 11ter) : APT Members expressed preference for Option 1.

Milestone based approach – reuse of spacecraft to BIU or count towards milestones of other systems (resolves 12) : APT Members expressed slight preference for Alternative 2(NOC)

tolerances in orbital characteristic values : further studies needed

- Topics proposed by other regional Groups or ITU Members which are not included in no. 2 above

| RCC | CITEL | CEPT | ATU | ASMG |  |
| --- | --- | --- | --- | --- | --- |
| Option C | Option A | Option A | Option A | Option A | BIU |
| 2 years 10% --- 4 years 30% --- 7 years 90% | 3 years 10% --- 5 years 50% --- 7 years 100% | 2years 10% -- 4years 30% --- 7years 100% | support the principle of milestone based approach | Option F | Milestone based approach |
| 1 Jan 2021 | 1 Jan 2021 | Option 1 Date: TBD | 1 Jan 2021 | option 1 1 Jan 2021 | Transitional measure |

- Progress of discussion during WRC-19 on the Agenda Item

After several difficult meetings, there is preliminary convergence on a milestones options framework.

The concept of the framework is still in development but it is essentially:

01/01/2021, 2,5,7yrs 10,40,100%; if milestone-1 is not met then administrations may choose to move onto alternative timeline below.

01/01/2021, 3,5,7yrs 10,60,100%.

This puts a penalty in the form of a higher threshold for the second milestone.

- Issues which require discussion at APT Coordination Meetings and seek guidance thereafter

None, for time being.