|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The 5th Meeting of the APT Conference Preparatory Group for WRC-2012 (APG2012-5) |  | Document No : APG2012-5 /PACP-11 |  |
|  | 29 August – 03 September 2011, Busan, Rep. of Korea |  |  |  |

Source: Document APG2012-5/OUT-05(Rev.1)

Asia-Pacific Telecommunity (APT)

Preliminary APT Common ProposalS

for the work of the Conference

(Agenda Item 1.11)

Agenda Item 1.11

1.11	to consider a primary allocation to the space research service (Earth-to-space) within the band 22.55-23.15 GHz, taking into account the results of ITU-R studies, in accordance with Resolution 753 (WRC-07);

Introduction

ITU-R studies have concluded that a primary allocation can be made to the Space Research Service (Earth-to-space) in the band 22.55 – 23.15 GHz, without causing undue constraints to the services allocated in accordance with the Radio Regulations in this band.  On the basis of the ITU-R studies, APT Members support a primary allocation to the space research service (Earth-to-space) in the band 22.55–23.15 GHz as described in the single Method of the CPM Report to WRC-12.

The related proposals are given below.

Proposals

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	ASP /1.11/1

22-24.75 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| ... |  |  |
| 22.55-23.15 FIXED INTER-SATELLITE 5.338A MOBILE SPACE RESEARCH (Earth-to-space) ADD 5.A111 5.149 |  |  |
| 23.15-23.55 FIXED INTER-SATELLITE 5.338A MOBILE |  |  |
| ... |  |  |

ADD	ASP/1.11/2

5.A111	The location of earth stations in the space research service shall maintain a separation distance of at least 54 km from the respective border(s) of neighbouring administrations to protect the existing and future deployment of fixed and mobile services unless a shorter distance is otherwise agreed between the corresponding administrations. Nos. 9.17 and 9.18 do not apply.

Reason:  These changes to Article 5 of the ITU-R Radio Regulations are consequential to an allocation being made by WRC-12 to the Space Research Service (Earth-to-space).

ARTICLE  21

Terrestrial and space services sharing frequency bands above 1 GHz

Section III  –  Power limits for earth stations

MOD	ASP/1.11/3

TABLE  21-3  (end)     (WRC-12)

| Frequency band |  | Services |
| --- | --- | --- |
| 17.7-18.1 GHz |  | Fixed-satellite |
| 22.55-23.15 GHz |  | Earth exploration-satellite |
| 27.0-27.5 GHz6 | (for Regions 2 and 3) | Mobile-satellite |
| 27.5-29.5 GHz |  | Space research |
| 31.0-31.3 GHz | (for the countries listed in No. 5.545) |  |
| 34.2-35.2 GHz | (for the countries listed in No. 5.550 with respect to the countries listed in No. 5.549) |  |

Reason:  These changes to Article 21 of the ITU-R Radio Regulations are consequential to an allocation being made by WRC-12 to the Space Research Service (Earth-to-space).

SUP	ASP/1.11/4

RESOLUTION  753  (WRC-07)

Use of the band 22.55-23.15 GHz by the space research service

Reason: Studies required by this Resolution have been completed.

_______________