Attachment 15

Draft CEPT Brief on agenda item 1.15

1.15	to consider possible allocations in the range 3-50 MHz to the radiolocation service for oceanographic radar applications, taking into account the results of ITU R studies, in accordance with Resolution 612 (WRC 07);

## Issue

This agenda item covers the following issues:

Resolution 612 (WRC-07):

1	to invite ITU-R to identify high-frequency oceanographic radar system applications between 3 and 50 MHz, including bandwidth requirements, appropriate portions of this band for these applications, and other characteristics necessary to conduct sharing studies;

2	to invite ITU-R to conduct sharing analyses between the radiolocation service applications identified under resolves 1 and incumbent services in the bands identified to be suitable for the operation of high-frequency oceanographic radar systems;

3	that, if compatibility with existing services is confirmed under resolves 2, to recommend that WRC-11 consider allocations to the radiolocation service in several suitable bands between 3 and 50 MHz, as determined in the ITU-R studies, each band not exceeding 600 kHz, for the operation of oceanographic radars,

Preliminary CEPT position

TBD

Background

WRC-11 Agenda item 1.15 calls for the consideration of the creation of radiolocation allocations in the 3 to 50 MHz range. These allocations will be used for the operation of oceanographic radars that monitor the sea surface for wave heights, currents and tracking of large objects. These radars will have an operational range which will not be greater than [400 km]. Oceanographic radars have been operating in the 3 to 50 MHz range for more than 30 years on an experimental, non-interference basis. Increased reliance on the data from these systems for maritime safety, oceanographic, climatological, meteorological and disaster response operations have driven the need to improve the regulatory status of the spectrum which is used by Oceanographic radars while taking into account the protection of existing allocated services. WRC-11 Agenda item 1.15 was established with the understanding that spectrum would be allocated on a shared basis. Reallocation of spectrum from an existing allocated radio service to the radiolocation services is not the intent.

Multiple sub-bands are required within the range of 3 to 50 MHz for providing long range data as well as high resolution data. Lower frequencies support long-range operation whereas higher frequency ranges support high resolution data collection. High resolution data collection at long ranges is not achievable with current technology as they are mutually exclusive characteristics. Manufacturers and researchers have settled on frequencies near 4.5 MHz±1 MHz, 9 MHz±1 MHz, 13 MHz±1 MHz, 16 MHz ±1 MHz,  27 MHz±3 MHz and 42 MHz±3 MHz to meet the various scientific and operational requirements. This does not imply that 2 to 6 MHz are required in each range for operation of a network of high-frequency oceanographic ocean observing radars. The actual spectrum requirements are much lower. For long-range operation (low frequencies around 4.5 MHz, 9 MHz, 13 MHz and 16 MHz) 2 separate bandwidths of around 50 kHz are needed in each range of operation and per oceanographic radar. For short-range operation (high frequencies around 27 MHz and 42 MHz), 2 separate bandwidths of around 150 kHz are needed in each range of operation and per oceanographic radar.

[Editor’s Note: Further study is needed for the aggregate spectrum requirement.]

Sharing studies with the Fixed Service in the bands 4.5 MHz ±1 MHz, 9 MHz ±1 MHz, 13 MHz ±1 MHz, 16MHz ± 1MHz, 27 MHz ±3 MHz and 42 MHz ±3 MHz

TBD

Sharing studies with the Mobile Service in the bands 4.5 MHz ±1 MHz, 9 MHz ±1 MHz, 16MHz ± 1MHz, 27 MHz ±3 MHz and 42 MHz ±3 MHz

TBD

Sharing studies with the Land Mobile Service in the bands 4.5 MHz ±1 MHz and 27 MHz ±3 MHz

Sharing studies with the Amateur Service in the bands 4.5 MHz ±1 MHz, 16MHz ± 1MHz, and 27 MHz ±3 MHz

TBD

Sharing with the Aeronautical Mobile, the Aeronautical Mobile (OR) and the Aeronautical Mobile (R) Services in the bands 4.5 MHz ±1 MHz, 9 MHz ±1 MHz, 16MHz ± 1MHz, and 13 MHz ±1 MHz

TBD

Sharing with the Broadcasting Service in the bands 4.5 MHz ±1 MHz, 9 MHz ±1 MHz, 13 MHz ±1 MHz, 16MHz ± 1MHz, and 27 MHz ±3 MHz

TBD

Sharing with the maritime Mobile Service in the bands 4.5 MHz ±1 MHz, 9 MHz ±1 MHz, 13 MHz ±1 MHz, 16MHz ± 1MHz, and 27 MHz ±3 MHz

TBD

Sharing with the Standard Frequency and time signal Service in the bands 4.5 MHz ±1 MHz, 9 MHz ±1 MHz, 16MHz ± 1MHz, and 27 MHz ±3 MHz

TBD

Sharing studies with the Radio Astronomy in the bands 13 MHz ±1 MHz

TBD

List of relevant documents

WPDNR ITU-R M.[HF-RADAR]

ITU-R new report on sharing studies

## Actions to be taken

- Aggregate spectrum requirements for radiolocation systems

- Elaborate the WPRND ITU-R M [HF-RADAR] on oceanographic radars characteristics

- Elaborate the ITU-R new report on sharing studies

- Fixed, Mobile, Broadcasting, Maritime mobile, Standard frequency and time signal, Land mobile, Amateur, AM(R), AM(OR), and RAS services systems characteristics

- Protection criteria of existing services

- Propagation model for sharing studies

## Relevant information from outside CEPT

European Union

Regional telecommunication organisations

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (date of proposal)

RCC (date of proposal)

[EBU]

International organisations

[ITU (date of proposal)]

[ICAO (date of proposal)]

[IMO (date of proposal)]

The Group noted the information provided by the Secretariat with regard to WRC-11 Agenda item 1.15 (IMO/ITU EG 4/5/4) and considered that the use of the radiolocation service between 3 and 50 MHz to support HF oceanographic radar operations of sea states and wave conditions. The Group noted that the advertised studies could be of benefit to the maritime community. However, the Group further noted that this was a complex issue with many questions still pending. In particular, it was recognized that some consideration was being given to higher output powers and surveillance for surface objects out to a range of 200 NM. It was considered that such use could close off the development of short range and/or high bandwidth HF applications under agenda items 1.9 and 1.10. It was further considered to be of utmost importance to watch any developments regarding this agenda item very closely.

[NATO (date of proposal)]

SFCG (September 08)

SFCG supports the protection of existing space science service allocations.  No allocation changes to support HF oceanographic radar operations should be made in space science service bands unless acceptable sharing criteria are developed.

WMO (July 08)

WMO confirms the importance of oceanographic radars for meteorology, climatology and environmental activities and supports a relevant radiolocation service allocation within the 3-50 MHz band to enable the implementation and operation of such radars.

Regional organisations

ESA (November 08)

Same as SFCG position

EUMETNET (September 08)

Same as WMO position

[Eurocontrol (date of proposal)]

[Other relevant information]

Annex

|  |  |  |  |  |  |  |  | Service allocations between 3.5 and 5.5 MHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kHz | 3 500 – 3 750 | 3 750 - 3 800 | 3 800 - 3 900 | 3 900 - 3 950 | 3 950 - 4 000 | 4 000 - 4 063 | 4 063 - 4 438 | 4 438 - 4 650 | 4 650 - 4 700 | 4 700 - 4 750 | 4 750 - 4 850 | 4 850 - 4 995 | 4 995 - 5 003 | 5 003 - 5005 | 5 005 - 5 060 | 5 060 - 5 250 | 5 250 - 5 450 | 5 450 - 5 480 | 5 480 - 5 680 |
| REGION 1 |  |  |  | OR |  |  |  |  | R | OR |  |  | SF TS | SF TS* |  |  |  |  | R |
|  |  |  | OR |  |  |  |  | Exc R |  |  | OR |  |  |  |  | s Exc Aero | Exc Aero | OR |  |
|  | Exc Aero | Exc Aero |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REGION 2 |  |  |  |  |  |  |  |  | R | OR |  |  | SF TS | SF TS* |  |  |  | R | R |
|  |  |  |  |  |  |  |  | Exc R |  |  | Exc R |  |  |  |  | s Exc Aero | Exc Aero |  |  |
|  |  | Exc R | Exc R | Exc R | Exc R |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REGION 3 |  |  |  |  |  |  |  |  | R | OR |  |  | SF TS | SF TS* |  |  |  |  | R |
|  |  |  |  |  |  |  |  | Exc Aero |  |  |  |  |  |  |  | s Exc Aero | Exc Aero | OR |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | * = Space Research |  |  |  |  |  |
|  | Amateur |  | AS | Amateur-satellite |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  |  | Exc Aero | Mobile except aeronautical mobile |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |  |  |  |  |  |  |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  | Service allocations between 8 to 10 MHz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| kHz | 7 450 - 8 100 | 8 100 - 8 195 | 8 195 - 8 815 | 8 815 - 8 965 | 8 965 - 9 040 | 9 040 - 9 400 | 9 400 - 9 500 | 9 500 - 9 900 | 9 900 - 9 995 | 9 995 - 10 003 |  |  |
| REGION 1 |  |  |  | R | OR |  |  |  |  | SF TS |  |  |
|  | Exc R |  |  |  |  |  |  |  |  |  |  |  |
| REGION 2 |  |  |  | R | OR |  |  |  |  | SF TS |  |  |
|  | Exc R |  |  |  |  |  |  |  |  |  |  |  |
| REGION 3 |  |  |  | R | OR |  |  |  |  | SF TS |  |  |
|  | Exc R |  |  |  |  |  |  |  |  |  |  |  |
|  | Amateur |  | AS | Amateur-satellite |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  |  | Exc Aero | Mobile except aeronautical mobile |
|  |  |  |  |  |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  | Service allocation between 12 to 14 MHz |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 650 - 12 050 | 12 050 - 12 100 | 12 100 - 12 230 | 12 230 - 13 200 | 13 200 - 13 260 | 13 260 - 13 360 | 13 360 - 13 410 | 13 410 - 13 570 | 13 570 - 13 600 | 13 600 - 13 800 | 13 800 - 13 870 | 13 870 - 14 000 |
| REGION 1 |  |  |  |  | OR | R |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | s Exc R |  |  |  | s Exc R |
| REGION 2 |  |  |  |  | OR | R |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | s Exc R |  |  |  | s Exc R |
| REGION 3 |  |  |  |  | OR | R |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | s Exc R |  |  |  | s Exc R |
|  | Amateur |  | AS | Amateur-satellite |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  | Exc Aero | Mobile except aeronautical mobile |  |
|  |  | s Exc R | Mobile except aeronautical mobile (R) secondary allocation |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |
|  | Radio Astronomy |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  | Service allocation between 15 to 17 MHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14 990 – 15 005 | 15 005 – 15 010 | 15 010 – 15 100 | 15 100 – 15 800 | 15 800 – 16 360 | 16 360 – 17 410 |  |  |  |  |  |  |  |  |
| REGION 1 | ST TS | ST TS + sr | OR |  |  |  |  |  |  |  |  |  |  |  |
| REGION 2 | ST TS | ST TS + sr | OR |  |  |  |  |  |  |  |  |  |  |  |
| REGION 3 | ST TS | ST TS + sr | OR |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 5.153 |  |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  | Exc Aero | Mobile except aeronautical mobile |  |  |  |
|  |  | s Exc R | Mobile except aeronautical mobile (R) secondary allocation |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |  |  |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
| sr | Space research (secondary allocation) |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.153	In Region 3, the stations of those services to which the band 15 995-16 005 kHz is allocated may transmit standard frequency and time signals.

|  |  |  | Service allocation between 24 to 30 MHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 24 000-24 890 | 24 890-24 990 | 24 990-25 005 | 25 005-25 210 | 25 010-25 070 | 25 070-25 210 | 25 210-25 550 | 25 550-25 670 | 25 670 - 26 100 | 26 100 - 26 175 | 26 175 - 27 500 | 27 500 - 28 000 | 28 000-29 700 | 29 700-30 000 |
| REGION 1 |  |  | SF TS | SF TS |  |  |  |  |  |  |  | METOC |  |  |
|  |  | AS |  |  | Exc Aero |  | Exc Aero |  |  |  | Exc Aero |  | AS |  |
| REGION 2 |  |  | SF TS | SF TS |  |  |  |  |  |  |  | METOC |  |  |
|  |  | AS |  |  | Exc Aero |  | Exc Aero |  |  |  | Exc Aero |  | AS |  |
| REGION 3 |  |  | SF TS | SF TS |  |  |  |  |  |  |  | METOC |  |  |
|  |  | AS |  |  | Exc Aero |  | Exc Aero |  |  |  | Exc Aero |  | AS |  |
|  | Amateur |  | AS | Amateur-satellite |  |  |  |  |  |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  | Exc Aero | Mobile except aeronautical mobile |  |  |  |
|  |  | s Exc R | Mobile except aeronautical mobile (R) secondary allocation |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |  |  |  |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Radio Astronomy |  |  |  |  |  |  |  |  |  |  |  |  |  |
| METOC | Meteorological aids |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  | Service allocation between 39 to 45 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 39-39.986 | 39.986-40.02 | 40.02-40.98 | 40.98-41.015 | 41.015-44 | 44-45 |  |  |  |  |  |
| REGION 1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | * |  | * |  |  |  |  |  |  |  |
| REGION 2 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | * |  | * |  |  |  |  |  |  |  |
| REGION 3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | * |  | * |  |  |  |  |  |  |  |
|  | Amateur |  | AS | Amateur-satellite |  | * = Space Research |  |  |  |  |  |
|  | Fixed |  |  |  |  |  |  |  |  |  |  |
|  | Mobile | Exc R | Mobile except aeronautical mobile (R) |  |  | Exc OR | Mobile except aeronautical mobile (OR) |  |  | Exc Aero | Mobile except aeronautical mobile |
|  |  | s Exc R | Mobile except aeronautical mobile (R) secondary allocation |  |  |  |  |  |  | s Exc Aero | Mobile except aeronautical mobile (secondary allocation) |
| OR | Aeronautical Mobile (OR) |  |  |  |  |  |  |  |  |  |  |
| R | Aeronautical Mobile (R) |  |  | Aeronautical Mobile |  |  |  |  |  |  |  |
|  | Land Mobile |  |  |  |  |  |  |  |  |  |  |
|  | Maritime Mobile |  |  |  |  |  |  |  |  |  |  |
|  | Broadcasting |  |  |  |  |  |  |  |  |  |  |
| SF TS | Standard frequency and time signal |  |  |  |  |  |  |  |  |  |  |
|  | Radio Astronomy |  |  |  |  |  |  |  |  |  |  |