Attachment 25

Draft CEPT Brief on agenda item 1.25

1.25	to consider possible additional allocations to the mobile-satellite service, in accordance with Resolution 231 [COM6/21] (WRC-07);

## Issue

“to complete, for WRC-11, studies of possible bands for new allocations to the mobile-satellite service in the Earth-to-space and space-to-Earth directions, with particular focus on the range 4 GHz to 16 GHz, taking into account sharing and compatibility, without placing undue constraints on existing services in this band”

Preliminary CEPT position

CEPT supports the allocation of a justified amount of additional spectrum to the MSS without unduly constraining existing, planned and future radio applications operated in the existing services.

Background

During the last study period ITU-R has studied traffic forecasts for the satellite component of IMT for the time period 2010-2020.  The results of the studies are contained in Report ITU-R M.2077 and provided the technical basis for related spectrum requirements identified in the CPM-Report on WRC-07 Agenda Item 1.4.  The Report indicates a shortfall of spectrum available for the satellite component of IMT in the Earth-to-space direction of between 19 and 90 MHz and in the space-to-Earth direction of between 144 and 257 MHz for the year 2020, depending on the traffic scenario.  Two scenarios, a low traffic and a high traffic scenario had been studied.

Currently 2 x 121.5 MHz of spectrum is allocated to the MSS in the frequency range 1-3 GHz but only about 2 x 86.5 MHz are available on a global basis. Worldwide availability of MSS spectrum, however, is an almost essential condition for the satellite component of IMT.

WRC-07 took account of the CPM-Report which led to WRC-11 Agenda Item 1.25 and Resolution 231 (WRC-07) inviting ITU-R to complete studies on possible frequency bands for additional allocations for the MSS, in particular in the frequency band 4-16 GHz. However, the frequency range 1-6 GHz has been indicated in Report ITU-R M.2077 as a preferred range for the satellite component of IMT.

Consideration has been given also to MSS systems which are not related to IMT and may also require additional spectrum.

The frequency bands from about 4-16 GHz are allocated to a large number of services on primary and secondary basis with numerous major utilisations within CEPT, as given in ERC Report 25.  For the protection of the incumbent services it is necessary to carry out sharing studies with characteristics of potential MSS systems to facilitate the identification of frequency band suitable as additional allocations for the MSS.

WP 4C has been identified as the responsible group for studies on this agenda item within ITU-R and concluded at its first meeting that there will be three main elements required for the studies: 1) to establish technical characteristics and deployment scenarios of new MSS systems that might operate in the frequency range in question; 2) sharing studies with other services; and 3) studies or rationale for spectrum requirements for new MSS applications (which may have already been included in Report ITU-R M.2077).  With a view on the second element in particular, Annex 1 contains an overview of the services listed in the Article 5 of the Radio Regulations in the frequency range of about 4-16 GHz. European allocations and footnotes are also incorporated, marked in yellow, as given in the ERC Report 25 (Nice 2007).  Major European utilisations as well as initial outlines of information on the individual frequency bands are given in order facilitate the identification of bands.

WP4C at its meeting in April 2008 has not identified any candidate bands at this time, it established a provisional work plan for the development and finalising of CPM-Text.  The provisional work plan is contained in document ITU-R  4C/66 Annex 15.

The next ITU-R WP4C meeting is scheduled for 29 September – 8 October 08 where the items of the work plan will be further discussed. It is expected that the discussions will focus on the establishment of technical characteristics, deployment scenarios and potential future trends for MSS in these bands.

List of relevant documents

## ITU-R Report M.2077

ITU-R 4C/66

ITU-R 4C/66 Annex 15

ITU-R 4C/71

ITU-R 4C/74

ITU-R 4C/118

ITU-R 4C/137

ERC Report 25

ECC/CPG11 PT-D(08)032

ECC/CPG11 PT-D INFO4

ECC/CPG11 PT-D INFO5

## Actions to be taken

## In order to facilitate sharing studies and the selection of suitable additional frequency band for the MSS, .in support of ITU-R studies:

## 1) to establish technical characteristics and deployment scenarios of new MSS systems that might operate in the frequency range in question;

## 2) sharing studies with other services;

## 3) studies or rationale for spectrum requirements for new MSS applications (some of which may have already been included in Report ITU-R M.2077)

## Relevant information from outside CEPT

European Union

Regional telecommunication organisations

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL (September 2008)

Canada is of the view that it should be ensured that the many existing services in the band 4-16 GHz are not impacted.

RCC (date of proposal)

International organisations

[ITU (date of proposal)]

[ICAO (date of proposal)]

[IMO (date of proposal)]

[NATO (date of proposal)]

SFCG (September 08)

SFCG supports the protection of all space science service bands in the 4-16 GHz frequency range.  SFCG member agencies make substantial use of a large number of frequency bands in this range that are critical to space research and Earth exploration-satellite service missions.

WMO (July 08)

Should identification or allocations for Mobile Satellite Service be considered in meteorological bands, WMO urges that compatibility with related applications be assessed and adequate protection be ensured.

Regional organisations

ESA (November 08)

Same as SFCG position

[EUMETNET (September 08)

Same as WMO position

[Eurocontrol (date of proposal)]

[Other relevant information]

annex

Extract of the Table of Allocations of Article 5 of the Radio Regulations of 2004, including modifications made at WRC-07 and relevant details extracted from the ERC Report 25 (Nice 07)

(Elements in the Table of Allocations marked in yellow are taken from the ERC Report 25 (Nice 07))

| Region 1 | Region 2 | Region 3 | Initial outline of information ( to be developed further) | Major utilisation within CEPT |
| --- | --- | --- | --- | --- |
| 3 400-3 600 FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile ADD 5.AAA Radiolocation 5.431 | 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile ADD 5.ZZZRadiolocation 5.433 5.282 5.432 | 3 400-3 500 FIXED FIXED-SATELLITE (space-to-Earth) Amateur Mobile ADD 5.BBB ADD 5.AAA1 Radiolocation 5.433 5.282 .432 | - Lower bands likely to be used for terrestrial IMT/BWA in some countries. - FSS downlink subject to RR Art 21 pfd limits - For FS characteristics, see Rec. F 758-4 | - Amateur (3400-3410 MHz) - BWA (3400-3800 MHz) - Mobile applications - Radars (upper limit for airborne radars is 3410 MHz) |
|  | 3 500-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 5.433 5.435 | 3 500-3 600 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile ADD 5.CCC Radiolocation 5.433 5.435 |  |  |

| Region 1 |  | Region 2 |  |  |  | Region 3 |  |  |  | Initial outline of information ( to be developed further) |  | Major utilisation within CEPT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 600-4 200 FIXED FIXED-SATELLITE (space-to-Earth) Mobile |  |  |  |  |  | 3 600-3 700 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile Radiolocation 3 5.435 |  |  |  |  |  | - BWA (3400-3800 MHz) - FSS - Medium/high capacity FS links |
|  |  | 3 700-4 200 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |  |  |  | 3 700-4 200 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile |  |  |  |  |  |  |
| 4 200-4 400 AERONAUTICAL RADIONAVIGATION 5.438 5.439 5.440 EU18 |  |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. |  | - Altimeters - Passive sensors (satellite) |
| 4 400-4 500 FIXED MOBILE ADD 5.4B01 EU2 EU20 EU27 |  |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for fixed and mobile systems) - Mobile applications |
| 4 500-4 800 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 MOBILE ADD 5.4B01 EU20 EU27 |  |  |  |  |  |  |  |  |  | - FSS use under Appendix 30B. - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for fixed and mobile systems) - FSS (FSS not to be implemented in NATO Europe. FSS frequency plan in 4500-4800 MHz) - Mobile applications |
| 4 800-4 990 FIXED MOBILE MOD 5.442 ADD 5.4B01 Radio astronomy 5.149 5.339 5.443 EU20 EU27 |  |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for fixed and mobile systems) - Mobile applications - Passive sensors (satellite) - Radio astronomy |
| 4 990-5 000 FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY Space research (passive) 5.149 EU20 EU27 |  |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for fixed and mobile systems) - Mobile applications - Radio astronomy |
| 5 000-5 010 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (Earth-to-space) Radio Astronomy Space Research (passive) 5.367 |  |  |  |  |  |  |  |  | - Also allocated to AMS(R)S through No. 5.367. - See Resolution 420 [COM4/9] (WRC-07) for potential use by AM(R)S under WRC-11 agenda item 1.4. |  | - Galileo (future use) - Radio Astronomy - Satellite Navigation Systems (Aeronautical Radionavigation and FSS envisaged in some countries) |  |
| 5 010-5 030 AERONAUTICAL RADIONAVIGATION RADIONAVIGATION-SATELLITE (space-to-Earth) (space-space) 5.328B 5.443B Radio Astronomy Space Research (passive) 5.367 |  |  |  |  |  |  |  |  | - Also allocated to AMS(R)S through No. 5.367. - See Resolution 420 [COM4/9] (WRC-07) for potential use by AM(R)S under WRC-11 agenda item 1.4. |  | - Galileo C1 - Radio Astronomy - Satellite Navigation Systems (Aeronautical Radionavigation and FSS envisaged in some countries) |  |
| 5 030-5 091 AERONAUTICAL RADIONAVIGATION 5.367 MOD 5.444 EU18 |  |  |  |  |  |  |  |  | - Also allocated to AMS(R)S through No. 5.367. - Used or planned to be used for MLS. |  | - MLS - Aeronautical Radionavigation and FSS envisaged in some countries |  |
| 5 091-5 150 AERONAUTICAL RADIONAVIGATION AERONAUTICAL MOBILE ADD 5.4B03 5.367 MOD 5.444 MOD 5.444A EU18 |  |  |  |  |  |  |  |  | - Also allocated to AMS(R)S through No. 5.367. - See Resolution 748 [COM4/4] (WRC-07) for sharing between AM(R)S and FSS. - See Resolution 418 [COM4/7] (WRC-07) for use by AMT. - See Resolution 419 [COM4/8] (WRC-07) for use by AMS. |  |  |  |
| 5 150-5 250 AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE except aeronautical mobile MOD 5.446A 5.446B 5.446 5.447 5.447B 5.447C ADD 5.4B04 |  |  |  |  |  |  |  |  | - See Resolution 418 [COM4/7] (WRC-07) for use by AMT. - See Resolution 229 (WRC-03) for use by wireless access systems. |  | - Feeder links for MSS (Aeronautical Radionavigation and FSS envisaged in some countries) - WAS/RLANs (within the band 5130-5350 MHz and 5470-5725 MHz) |  |
| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) MOBILE except aeronautical mobile 5.446A 5.447F RADIOLOCATION SPACE RESEARCH 5.447D 5.447E 5.448 5.448A EU2 EU22 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. - See Resolution 229 (WRC-03) for use by wireless access systems. - See Recommendation ITU-R RS.1632 for sharing between EESS (active) and WAS. |  | - Active Sensors - Defence Systems (Tactical weapon system radars) - Position fixing - Shipborne and VTS radar - WAS/RLANs (within the band 5130-5350 MHz and 5470-5725 MHz) - Weather radars (ground based and airborne) |  |
| 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) MOBILE except aeronautical mobile 5.446A 5.447F 5.447E 5.448 5.448A EU2 EU22 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. - See Resolution 229 (WRC-03) for use by wireless access systems. - See Recommendation ITU-R RS.1632 for sharing between EESS (active) and WAS. |  | - Active Sensors - Defence Systems (Tactical weapon system radars) - Position fixing - Shipborne and VTS radar - WAS/RLANs (within the band 5130-5350 MHz and 5470-5725 MHz) - Weather radars (ground based and airborne) |  |
| 5 350-5 460 EARTH EXPLORATION-SATELLITE (active) 5.448B SPACE RESEARCH (active) 5.448C AERONAUTICAL RADIONAVIGATION 5.449 RADIOLOCATION 5.448D Fixed EU2 EU22 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. |  | - Active Sensors - Defence Systems (Tactical weapon system radars) - Position fixing - Shipborne and VTS radar - Weather radars (ground based and airborne) |  |
| 5 460-5 470 RADIONAVIGATION 5.449 EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) RADIOLOCATION 5.448D 5.448B EU2 EU22 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. |  | - Active Sensors - Defence Systems (Tactical weapon system radars) - Position fixing - Shipborne and VTS radar - Weather radars (ground based and airborne) |  |
| 5 470-5 570 MARITIME RADIONAVIGATION MOBILE except aeronautical mobile 5.446A 5.450A EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) RADIOLOCATION 5.450B 5.448B 5.450 5.451 EU2 EU 22 |  |  |  |  |  |  |  |  | - See Resolution 229 (WRC-03) for use by wireless access systems. - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. |  | - Active Sensors - Defence Systems (Tactical weapon system radars) - Position fixing - Shipborne and VTS radar - WAS/RLANs (within the band 5130-5350 MHz and 5470-5725 MHz) - Weather radars (ground based and airborne) |  |
| 5 570-5 650 MARITIME RADIONAVIGATION MOBILE except aeronautical mobile 5.446A 5.450A RADIOLOCATION 5.450B 5.450 5.451 5.452 EU2 EU22 |  |  |  |  |  |  |  |  | - See Resolution 229 (WRC-03) for use by wireless access systems. - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. |  | - Defence Systems (Tactical weapon system radars) - Position fixing - Shipborne and VTS radar - WAS/RLANs (within the band 5130-5350 MHz and 5470-5725 MHz) - Weather radars (ground based) |  |
| 5 650-5 725 RADIOLOCATION MOBILE except aeronautical mobile 5.446A 5.450A Amateur Amateur-satellite (Earth-to-space) Space research (deep space) 5.282 5.451 5.453 5.454 5.455 EU2 EU17 EU22 EU23 |  |  |  |  |  |  |  |  | - See Resolution 229 (WRC-03) for use by wireless access systems. - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. |  | - Amateur (5660-5670 MHz) - Amateur-satellite (5660-5670 MHz) - Defence Systems (Tactical weapon system radars) - Position fixing - Shipborne and VTS radar - WAS/RLANs (within the band 5130-5350 MHz and 5470-5725 MHz) - Weather radars (ground based and airborne) |  |
| 5 725-5 830 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Mobile |  |  |  | 5 725-5 830 RADIOLOCATION Amateur |  |  |  |  | - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. - See Recommendation ITU-R S.524 for off-axis e.i.r.p. values for FSS earth stations. - Designated for ISM (see No. 5.150). |  | - Amateur - BFWA (5725-5875 MHz) - Defence Systems (Tactical weapon system radars) - ISM (5725-5875 MHz) - Non-Specific SRDs (5725-5875 MHz) - RTTT (5795-5805 MHz). RTTT in the band 5805-5815 MHz on national basis - Weather radars (ground based and airborne) |  |
| 5.150 5.451 5.453 5.455 5.456 EU2 EU22 |  |  |  | 5.150 5.453 5.455 |  |  |  |  |  |  |  |  |
| 5 830-5 850 FIXED-SATELLITE (Earth-to-space) RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) Mobile |  |  |  | 5 830-5 850 RADIOLOCATION Amateur Amateur-satellite (space-to-Earth) |  |  |  |  | - See Recommendation ITU-R M.1638 for characteristics and protection criteria for radiolocation, aeronautical radionavigation and meteorological radars. - See Recommendation ITU-R S.524 for off-axis e.i.r.p. values for FSS earth stations. - Designated for ISM (see No. 5.150). |  | - Amateur-satellite - BFWA (5725-5875 MHz) - Defence Systems (Tactical weapon system radars) - ISM (5725-5875 MHz) - Non-Specific SRDs (5725-5875 MHz) - Weather radars (ground based and airborne) |  |
| 5.150 5.451 5.453 5.455 5.456 EU2 EU22 EU23 |  |  |  | 5.150 5.453 5.455 |  |  |  |  |  |  |  |  |
| 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE |  |  |  | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Amateur Radiolocation |  |  |  | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Radiolocation | - See Recommendation ITU-R S.524 for off-axis e.i.r.p. values for FSS earth stations. - 5 125-5 875 MHz designated for ISM (see No. 5.150). - For FS characteristics, see Rec F.758-4 - Considered for Gateway feederlinks for HAPS under WRC-11 agenda item 1.20 |  | - BFWA (5725-5875 MHz) - FSS - ISM (5725-5875 MHz) - Non-Specific SRDs (5725-5875 MHz) |  |
| 5.150 |  |  |  | 5.150 |  |  |  | 5.150 |  |  |  |  |
| 5 925-6 700 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B MOBILE ADD 5.4B02 Earth exploration-satellite (passive) [6425-6700 MHz] 5.149 5.440 5.458 |  |  |  |  |  |  |  |  | - 5 925-6 425 MHz may be used by ESVs in accordance with Resolution 902 (WRC-03). - See Recommendation ITU-R S.524 for off-axis e.i.r.p. values for FSS earth stations. - For FS characteristics, see Rec F.758-4 - Considered for Gateway feederlinks for HAPS under WRC-11 agenda item 1.20 |  | - Fixed links (P-P) - FSS - UWB (6000-8500 MHz) - Passive sensors (satellite) in the band 6425-6700 MHz) |  |
| 6 700-7 075 FIXED FIXED-SATELLITE (Earth-to-space) (space-to-Earth) 5.441 Earth exploration-satellite (passive) MOBILE 5.458 5.458A 5.458B 5.458C |  |  |  |  |  |  |  |  | - See Recommendation ITU-R S.524 for off-axis e.i.r.p. values for FSS earth stations. - FSS uplinks limited to feeder links for non-GSO MSS (see No. 5.458B). - See No. 22.5A for pfd limit applicable to non-GSO FSS uplinks. - 6 725-7 025 MHz is subject to Appendix 30B. - For FS characteristics, see Rec F.758-4 - Considered for Gateway feederlinks for HAPS under WRC-11 agenda item 1.20 |  | - Fixed links (P-P) - Feeder links for MSS (6925-7075 MHz) - FSS - UWB (6000-8500 MHz) - Passive sensors (satellite) |  |
| 7 075-7 145 FIXED MOBILE Earth exploration-satellite (passive) 5.458 5.459 |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Fixed links (P-P) - UWB (6000-8500 MHz) - Passive sensors (satellite) |  |
| 7 145-7 235 FIXED MOBILE SPACE RESEARCH (Earth-to-space) 5.460 Earth exploration-satellite (Earth-to-space) Space operation (Earth-to-space) 5.458 5.459 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R SA.1016 for sharing considerations related to space research. - For FS characteristics, see Rec F.758-4 |  | - Fixed links (P-P) - UWB (6000-8500 MHz) - Passive sensors (satellite) |  |
| 7 235-7 250 FIXED MOBILE Earth exploration-satellite (Earth-to-space) Space research (Earth-to-space) 5.458 |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Fixed links (P-P) - UWB (6000-8500 MHz) - Passive sensors (satellite) |  |
| 7 250-7 300 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE 5.461 EU2 EU27 |  |  |  |  |  |  |  |  | - 7 250-7 375 MHz allocated to MSS downlinks, subject to No. 9.21 (see No. 5.461). - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for satellite operations) - Fixed links (P-P; Fixed and mobile services not to be implemented in most NATO countries) - MSS applications (7250-7375 MHz) - UWB (6000-8500 MHz) |  |
| 7 300-7 450 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.461 EU2 EU27 |  |  |  |  |  |  |  |  | - 7 250-7 375 MHz allocated to MSS downlinks, subject to No. 9.21 (see No. 5.461). - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for satellite operations) - Fixed links (P-P) - MSS applications (7250-7375 MHz) - UWB (6000-8500 MHz) |  |
| 7 450-7 550 FIXED FIXED-SATELLITE (space-to-Earth) METEOROLOGICAL-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile 5.461A EU2 EU27 |  |  |  |  |  |  |  |  | - MetSat limited to GSO (see No. 5.461A). - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for satellite operations) - Fixed links (P-P) - Meteorological Satellites - UWB (6000-8500 MHz) |  |
| 7 550-7 750 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile EU2 EU27 |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for satellite operations) - Fixed links (P-P) - UWB (6000-8500 MHz) |  |
| 7 750-7 850 FIXED METEOROLOGICAL-SATELLITE (space-to-Earth) 5.461B MOBILE except aeronautical mobile EU2 |  |  |  |  |  |  |  |  | - MetSat limited to non-GSO (see No. 5.461B). - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for satellite operations) - Fixed links (P-P) - Meteorological Satellites - UWB (6000-8500 MHz) |  |
| 7 850-7 900 FIXED MOBILE except aeronautical mobile |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 - Considered for possible MetSat allocation under WRC-11 agenda item 1.24 |  | - Defence systems - Fixed links (P-P) - UWB (6000-8500 MHz) |  |
| 7 900-8 025 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE 5.461 EU2 EU27 |  |  |  |  |  |  |  |  | - Allocated to MSS uplinks, subject to No. 9.21 (see No. 5.461A). - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for satellite operations) - Fixed links (P-P; Fixed and Mobile services not to be implemented above 7975 MHz in NATO countries) - MSS applications - UWB (6000-8500 MHz) |  |
| 8 025-8 175 EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) MOBILE 5.463 5.462A EU2 EU27 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R SA.1277 for sharing between EESS and other services. - See Recommendation ITU-R SA.1810 for design guidelines for EESS. - See No. 22.5 for pfd limits for EESS. |  | - Defence systems (Harmonised military band for satellite operations) - Earth Exploration-Satellite - Fixed links (P-P) - MS applications - UWB (6000-8500 MHz) |  |
| 8 175-8 215 EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) METEOROLOGICAL-SATELLITE (Earth-to-space) MOBILE 5.463 5.462A EU2 EU27 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R SA.1277 for sharing between EESS and other services. - See Recommendation ITU-R SA.1810 for design guidelines for EESS. - See No. 22.5 for pfd limits for EESS. - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for satellite operations) - Earth Exploration-Satellite - Fixed links (P-P) - MS applications - UWB (6000-8500 MHz) |  |
| 8 215-8 400 EARTH EXPLORATION-SATELLITE (space-to-Earth) FIXED FIXED-SATELLITE (Earth-to-space) MOBILE 5.463 5.462A EU2 EU27 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R SA.1277 for sharing between EESS and other services. - See Recommendation ITU-R SA.1810 for design guidelines for EESS. - See No. 22.5 for pfd limits for EESS. - For FS characteristics, see Rec F.758-4 |  | - Defence systems (Harmonised military band for satellite operations) - Earth Exploration-Satellite - Fixed links (P-P) - Radio Astronomy - UWB (6000-8500 MHz) |  |
| 8 400-8 500 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-Earth) 5.465 5.466 Radiolocation |  |  |  |  |  |  |  |  | - See Recommendation ITU-R SA.1016 for sharing considerations related to space research. - For FS characteristics, see Rec F.758-4 |  | - Fixed links (P-P) - Space Research - UWB (6000-8500 MHz) |  |
| 8 500-8 550 RADIOLOCATION 5.468 5.469 EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 8 550-8 650 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) 5.468 5.469 5.469A EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Active Sensors (satellite) - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 8 650-8 750 RADIOLOCATION 5.468 5.469 EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 8 750-8 850 RADIOLOCATION AERONAUTICAL RADIONAVIGATION 5.470 5.471 EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 8 850-9 000 RADIOLOCATION MARITIME RADIONAVIGATION 5.472 5.473 EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 9 000-9 200 AERONAUTICAL RADIONAVIGATION 5.337 RADIOLOCATION Space Research MOD 5.471 ADD 5.475A EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 9 200-9 300 RADIOLOCATION MARITIME RADIONAVIGATION 5.472 Space Research 5.473 5.474 EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Equipment for detecting movement and alert (9200-9975 MHz) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 9 300-9 500 RADIONAVIGATION 5.476 EARTH EXPLORATION-SATELLITE (active) SPACE RESEARCH (active) RADIOLOCATION 5.427 5.474 MOD 5.475 ADD 5.475B MOD 5.476A ADD 5.4B07 EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Equipment for detecting movement and alert (9200-9975 MHz) - Radars (Shipborne, airborne, land and surveillance and weapon) - Weather Radars (Ground and airborne) |  |
| 9 500-9 800 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION RADIONAVIGATION SPACE RESEARCH (active) MOD 5.476A EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Active Sensors (satellite) - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Equipment for detecting movement and alert (9200-9975 MHz) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 9 800-9 900 RADIOLOCATION Earth exploration-satellite (active) Space research (active) Fixed 5.477 5.478 ADD 5.xyz ADD 5.xyy EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Equipment for detecting movement and alert (9200-9975 MHz) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 9 900-10 000 RADIOLOCATION Fixed 5.477 5.478 5.479 EU2 EU24 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Aeronautical Radionavigation (Civil and military e. g. airfield approach) - Equipment for detecting movement and alert (9200-9975 MHz) - Radars (Shipborne, airborne, land and surveillance and weapon) |  |
| 10-10.45 FIXED MOBILE RADIOLOCATION Amateur EU17A (10.15-10.45 MHz) | 10-10.45 RADIOLOCATION Amateur |  |  |  | 10-10.45 FIXED MOBILE RADIOLOCATION Amateur |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. - FS characteristics may be included in Rec F.758-4 |  | - Amateur - Civil and military radar (Low power radars in certain subbands (10.15-10.45 MHz) - Fixed Links (10.15-10.3 GHz) - FWA incl. P-M (10.15-10.3 GHz) - Non civil radar (10-10.15 GHz) - SAP/SAB |  |
| 5.479 EU2 | 5.479 5.480 |  |  |  | 5.479 |  |  |  |  |  |  |  |
| 10.45-10.5 MOBILE FIXED RADIOLOCATION Amateur Amateur-satellite 5.481 EU2 EU17 EU17A EU23 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1796 for characteristics and protection criteria for terrestrial radars. |  | - Amateur - Amateur-Satellite - Civil and military radar) - Fixed Links (10.15-10.3 GHz) - FWA incl. P-M - SAP/SAB |  |
| 10.5-10.55 FIXED MOBILE Radiolocation EU17A | 10.5-10.55 FIXED MOBILE RADIOLOCATION |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Equipment for detecting movement and alert (10.5-10.6 GHz) - Fixed links - FWA - SAP/SAB |  |
| 10.55-10.6 FIXED MOBILE except aeronautical mobile Radiolocation EU17A |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Equipment for detecting movement and alert (10.5-10.6 GHz) - Fixed links - FWA - SAP/SAB |  |
| 10.6-10.68 EARTH EXPLORATION-SATELLITE (passive) FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY SPACE RESEARCH (passive) Radiolocation 5.149 MOD 5.482 ADD 5.BA01 EU17A |  |  |  |  |  |  |  |  | - See Resolution 751 [COM5/5] (WRC-07) for sharing criteria applicable to the FS and EESS. - For FS characteristics, see Rec F.758-4 |  | - Fixed links - FWA (10.6-10.65 GHz) - Passive Sensors (satellite) - Radio Astronomy - SAP/SAB |  |
| 10.68-10.7 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.483 |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. |  | - Passive Sensors (satellite) - Radio Astronomy |  |
| 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A (Earth-to-space) 5.484 MOBILE except aeronautical mobile | 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A MOBILE except aeronautical mobile |  |  |  | 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A MOBILE except aeronautical mobile |  |  |  | - See RR Table 21-4 for pfd limits applicable to the FSS downlinks. - The use of the bands 10.7-10.95 GHz (space-to-Earth), 11.2-11.45 GHz (space-to-Earth) is in accordance with Appendix 30B. - For FS characteristics, see Rec F.758-4 |  | - AES - Fixed links - FSS (10.7-10.95 GHz and 11.2-11.45 GHz) - HEST - LEST |  |
| 11.7-12.5 FIXED BROADCASTING BROADCASTING-SATELLITE MOBILE except aeronautical mobile | 11.7-12.1 FIXED 5.486 FIXED-SATELLITE (space-to-Earth) 5.484A Mobile except aeronautical mobile 5.485 5.488 |  |  |  | 11.7-12.2 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE |  |  |  | - See RR Table 21-4 for pfd limits applicable to the FSS downlinks. |  | - HEST - LEST - Satellite Broadcasting |  |
|  | 12.1-12.2 FIXED-SATELLITE (space-to-Earth) 5.484A |  |  |  |  |  |  |  |  |  |  |  |
|  | 5.485 5.488 5.489 |  |  |  | 5.487 5.487A 5.492 |  |  |  |  |  |  |  |
|  | 12.2-12.7 FIXED MOBILE except aeronautical mobile BROADCASTING BROADCASTING-SATELLITE |  |  |  | 12.2-12.5 FIXED FIXED-SATELLITE (space-to-Earth) MOBILE except aeronautical mobile BROADCASTING |  |  |  | - See RR Table 21-4 for pfd limits applicable to the FSS downlinks. - For FS characteristics, see Rec F.758-4 |  |  |  |
| 5.487 5.487A 5.492 |  |  |  |  | 5.484A 5.487 |  |  |  |  |  |  |  |
| 12.5-12.75 | 5.487A 5.488 5.490 5.492 |  |  |  | 12.5-12.75 |  |  |  | - See RR Table 21-4 for pfd limits applicable to the FSS downlinks. |  | - AES - FSS - HEST - LEST |  |
| FIXED-SATELLITE (space-to-Earth) 5.484A (Earth-to-space) 5.494 5.495 5.496 | 12.7-12.75 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile |  |  |  | FIXED FIXED-SATELLITE (space-to-Earth) 5.484A MOBILE except aeronautical mobile BROADCASTING- SATELLITE 5.493 |  |  |  |  |  |  |  |
| 12.75-13.25 FIXED FIXED-SATELLITE (Earth-to-space) 5.441 MOBILE Space research (deep space) (space-to-Earth) |  |  |  |  |  |  |  |  | - The use of the 12.75-13.25 GHz (Earth-to-space) is in accordance with Appendix 30B. - Off axis e.i.r.p. limits apply to FSS earth stations (see No. 22.29). - For FS characteristics, see Rec F.758-4 |  | - Fixed Links - FSS |  |
| 13.25-13.4 EARTH EXPLORATION-SATELLITE (active) AERONAUTICAL RADIONAVIGATION 5.497 SPACE RESEARCH (active) 5.498A 5.499 EU26 |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. |  | - Active Sensors (satellite; altimeters, scatterometers, precipitation radars) - Doppler Navigation Aids - Ship Berthing Radars |  |
| 13.4-13.75 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.501A Standard frequency and time signal-satellite (Earth-to- space) 5.499 5.500 5.501 5.501B EU2 EU26 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R RS.1281 for pfd values to protect radiolocation form EESS (active). |  | - Active Sensors (satellite; altimeters, scatterometers, precipitation radars) - Data Relay Satellites - Defence Systems (Military Radars) - Doppler Navigation Aids - Equipment for detecting movement and Alert (13.4-14.0 GHz) - Ship Berthing Radars |  |
| 13.75-14 FIXED-SATELLITE (Earth-to-space) 5.484A RADIOLOCATION Earth exploration-satellite Standard frequency and time signal-satellite (Earth-to- space) Space research 5.499 5.500 5.501 5.502 5.503 EU2 EU26 |  |  |  |  |  |  |  |  | - Off axis e.i.r.p. limits apply to FSS earth stations (see No. 22.29). - See No. 5.502 for limits on FSS operations to protect radiolocation. |  | - Data Relay Satellites - Defence Systems (Military Radars) - Equipment for detecting movement and Alert (13.4-14.0 GHz) - FSS - Navigation Radars - Passive Sensors (satellite) - Ship Berthing Radars |  |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.504C 5.506A Space research 5.504A 5.505 |  |  |  |  |  |  |  |  | - Off axis e.i.r.p. limits apply to FSS earth stations (see No. 22.29). - 14.0-14.5 GHz may be used by ESVs in accordance with Resolution 902 (WRC-03). |  | - AES - ESV - HEST - LEST - MSS - VSAT/SNG |  |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) 5.506A 5.508A Space research 5.504A 5.505 5.508 5.509 |  |  |  |  |  |  |  |  | - Off axis e.i.r.p. limits apply to FSS earth stations (see No. 22.29). - 14.0-14.5 GHz may be used by ESVs in accordance with Resolution 902 (WRC-03). |  | - AES - ESV - MSS - VSAT/SNG |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.506A 5.509A Radionavigation-satellite 5.504A |  |  | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.506 5.506B Mobile-satellite (Earth-to-space) 5.506A Radionavigation-satellite 5.504A |  |  |  | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.484A 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.506A 5.509A Radionavigation-satellite 5.504A |  | - Off axis e.i.r.p. limits apply to FSS earth stations (see No. 22.29). - 14.0-14.5 GHz may be used by ESVs in accordance with Resolution 902 (WRC-03). - FS characteristics may be included in Rec F.758-4 |  | - AES - ESV - FSS - MSS - VSAT/SNG |  |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.506A 5.509A Space research (space-to-Earth) 5.504A |  |  |  |  |  |  |  |  | - Off axis e.i.r.p. limits apply to FSS earth stations (see No. 22.29). - 14.0-14.5 GHz may be used by ESVs in accordance with Resolution 902 (WRC-03). - For FS characteristics, see Rec F.758-4 |  | - AES - ESV - FSS - MSS - VSAT/SNG |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.457A 5.457B 5.484A 5.506 5.506B MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) 5.504B 5.506A 5.509A Radio astronomy 5.149 5.504A |  |  |  |  |  |  |  |  | - Off axis e.i.r.p. limits apply to FSS earth stations (see No. 22.29). - 14.0-14.5 GHz may be used by ESVs in accordance with Resolution 902 (WRC-03). - For FS characteristics, see Rec F.758-4 |  | - AES - ESV - FSS - MSS - Radio Astronomy - VSAT/SNG |  |
| 14.5-14.8 FIXED FIXED-SATELLITE (Earth-to-space) 5.510 MOBILE Space research EU20 EU27 |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Defence Systems (The band 14.62-15.23 GHz is a harmonised military band for fixed and mobile services) - Fixed Links - Radio Astronomy |  |
| 14.8-15.35 FIXED MOBILE Space research 5.339 EU20 EU27 |  |  |  |  |  |  |  |  | - Summary of relevant information to be added – dependent on subsequent WP 4C review of necessary studies. - For FS characteristics, see Rec F.758-4 |  | - Defence Systems (The band 14.62-15.23 GHz is a harmonised military band for fixed and mobile services) - Fixed Links - Radio Astronomy |  |
| 15.35-15.4 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.511 |  |  |  |  |  |  |  |  | - Passive only. |  | - Passive Sensors (satellite) - Radio Astronomy |  |
| 15.4-15.43 AERONAUTICAL RADIONAVIGATION 5.511D |  |  |  |  |  |  |  |  | - Band is within the scope of WRC-11 Agenda item 1.21. |  | - Doppler Radar Low Power Sensing - Ground Movement Radars |  |
| 15.43-15.63 FIXED-SATELLITE (Earth-to-space) 5.511A AERONAUTICAL RADIONAVIGATION 5.511C |  |  |  |  |  |  |  |  | - Band is within the scope of WRC-11 Agenda item 1.21. |  | - Doppler Radar Low Power Sensing - FSS (MSS feeder links) - Ground Movement Radars |  |
| 15.63-15.7 AERONAUTICAL RADIONAVIGATION 5.511D |  |  |  |  |  |  |  |  | - Band is within the scope of WRC-11 Agenda item 1.21. |  | - Doppler Radar Low Power Sensing - Ground Movement Radars |  |
| 15.7-16.6 RADIOLOCATION 5.512 5.513 EU27 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1730 for characteristics and protection criteria for radiolocation. |  | - Defence Systems (Harmonised military band for land, airborne and navel radars) |  |
| 16.6-17.1 RADIOLOCATION Space research (deep space) (Earth-to-space) 5.512 5.513 EU27 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1730 for characteristics and protection criteria for radiolocation. |  | - Defence Systems (Harmonised military band for land, airborne and navel radars) |  |
| 17.1-17.2 RADIOLOCATION 5.512 5.513 EU2 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1730 for characteristics and protection criteria for radiolocation. |  | - Defence Systems (Military radar applications) - WAS/RLANs (17.1-17.3 GHz) |  |
| 17.2-17.3 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) 5.512 5.513 5.513A EU2 |  |  |  |  |  |  |  |  | - See Recommendation ITU-R M.1730 for characteristics and protection criteria for radiolocation. |  | - Defence Systems (Military radar applications) - WAS/RLANs (17.1-17.3 GHz) |  |

________________