Attachment 9

Draft CEPT Brief on agenda item 1.9

1.9	to revise frequencies and channelling arrangements of Appendix 17 to the Radio Regulations, in accordance with Resolution 351 (Rev.WRC-07), in order to implement new digital technologies for the maritime mobile service;

## Issue

Appendix 17 outlines the frequencies and channelling arrangements in the high-frequency bands for the maritime mobile service (MMS). During WRC-03, changes were made to Appendix 17 to allow for the use of digital technology on a no-protection, non-interference basis in certain bands (footnote “p”).

There is a requirement within the maritime mobile service for improving the utility of the present spectrum in the high-frequency bands for the maritime mobile service by allowing new digital technologies to use certain parts of RR Appendix 17 to provide additional flexibility and efficiency.

Resolution 351 (revised WRC-07) invites WRC-11 to consider necessary changes and calls for studies to identify any necessary modifications to the frequency table contained within Appendix 17; to identify any necessary transition arrangements for the introduction of new digital technologies and any consequential changes to Appendix 17 and to recommend how digital technologies can be introduced whilst ensuring compliance with distress and safety requirements.

## Preliminary CEPT position

CEPT supports a revision of Appendix 17 to facilitate the use of new digital communication systems, whilst maintaining sufficient provisions for remaining operational requirements involving Narrow Band Direct Printing (NBDP) within the GMDSS.

CEPT is of the opinion that any changes to Appendix 17 could  be implemented within a certain transition period.

## Background

The future spectrum needs of the maritime mobile service in the HF bands are closely related to the introduction of new digital technologies in the maritime mobile service as an alternative standard for narrow-band direct printing (NBDP). The use of NBDP is in a rapid decline. IMO has noted that NBDP is currently used for broadcasting of MSI, ship reporting, weather forecasts and for business communications, e.g. by fishing fleets. All these functions could also be provided by alternative digital technologies. Furthermore NBDP is specified as one of the methods for subsequent distress communications in the GMDSS as well as for search and rescue communications.

GMDSS compliance needs to be addressed before the NBDP system can be removed from Appendix 17. NBDP remains useful for distress communications in the polar regions (sea area A4) when other terrestrial means of communication are no longer reliable, and where there is no coverage from geostationary satellites.

## Actions to be taken

List of relevant documents

## Relevant information from outside CEPT

European Union

Regional telecommunication organisations

APT (date of proposal)

ATU (date of proposal)

Arab Group (date of proposal)

CITEL: 19 September 2008

- ISSUES:

- Timing and scope in transition arrangements.

- Co-existence between analog and digital systems.

- Amendments to Appendix 17 may have consequential impact to Appendix 25.

- Requirement, if any, for worldwide interoperability of equipment on ships.

PRELIMINARY VIEWS:

Given the outdated channel designations for Morse telegraphy, the decline in public correspondence by radiotelephony, and the current data and e-mail services provided in the maritime mobile service in the HF bands, Canada is of the view that amendments to Appendix 17 are required in order to accommodate new digital technologies while ensuring that:

  - distress and safety frequencies are protected;

  - provisions for radiotelephony and other designated uses are maintained, as required; and

  - any necessary transition arrangements are provided.

RCC (date of proposal)

International organisations

[ITU (date of proposal)]

[ICAO (date of proposal)]

IMO [preliminary position of the Joint IMO – ITU Experts Group]: 12 June 2008

The Group recalled the decision of COMSAR 12 that the frequencies currently allocated for use by the GMDSS needed to be retained because IMO had no intention to change the requirements for NBDP and DSC at this moment in time and these requirements should be retained in Appendix 15.  The Group noted that the spectrum that would have to remain dedicated to NBDP and DSC, in order to support the functional requirements of distress communications and the promulgation of MSI, only amounted to a small fraction of the Appendix 17 bands, the major portion of which would then become available for new digital technologies for the maritime mobile service.  It was also considered that the frequency bands allocated for Morse could be used for technologies within the maritime community giving in the same time the possibility for the Administrations who wish to continue to use them to do so without claiming protection.  The Group recognized that the channel bandwidths within Appendix 17 were only adequate for narrow band systems.  Therefore it was considered to support the idea of the creation of wide band channels within Appendix 17 for new technologies.  Finally it was considered that the frequencies for MSI within Appendix 15 have to be kept untouched.

[NATO (date of proposal)]

[SFCG (date of proposal)]

Regional organisations

[ESA (date of proposal)]

[Eumetnet (date of proposal)]

[Eurocontrol (date of proposal)]

[Other relevant information]