APT REPORT

On

CHARACTERISTICS AND REQUIREMENTS OF OPTICAL AND ELECTRICAL COMPONENTS FOR MILLIMETER-WAVE RADIO OVER FIBER SYSTEMS

No. APT/ASTAP/REPT-03(Rev.4)

Edition: September 2015

Source Document: ASTAP-26/OUT-12

Adopted by

The 19th APT Standardization Program Forum (ASTAP-19)

24 – 26 October 2011, Manila, Philippines

4th Revision at

The 26th APT Standardization Program Forum (ASTAP-26)

9 – 12 September 2015, Bangkok, Thailand

Table of contents

1. Introduction

2. Scope

3. Abbreviations and acronyms

4. References

5. Outline of RoF systems

6. Key optical devices for millimeter-wave RoF systems

6.1 E/O conversion devices

6.2 O/E conversion devices

6.3 Impact of optical amplifier transient on link performances

7. Possible applications of millimeter-wave RoF systems

7.1 Application to mobile service: Microwave/Millimeter-Wave Dual-Band RoF System

7.2 Application to fixed service: 120-GHz band high-speed wireless system based on millimeter-wave RoF technology

7.3 Application of radiolocation service: Foreign object debris detection system

8. Summary

Annex 1: Characterization of E/O and O/E conversion devices for RoF

Annex 2: Applications of microwave RoF to terrestrial services

Annex 3: RoF system of microwave Electronic News Gathering (ENG) for broadcasting

1. Introduction

Radio over Fiber (RoF) system can provide low-cost and high-performance radio-wave distribution by using radio access units (RAUs) where lightwave modulated by radio-wave is converted to radio-wave by a high-speed photodetector (PD) [1]. In microwave bands, RoF systems are successfully used for mobile services to provide signals to the mobile terminals located at such shadowing areas as underground, tunnel and inside buildings. As well known, millimeter-wave can carry high-speed data streams [2]. However, very high-frequency signals should be synthesized at a number of base stations (BSs) to cover a large area. RoF systems can provide low loss and cost effective millimeter-wave signal distribution and collection of millimeter-wave signals at many small cells. Thus, millimeter-wave broadband wireless systems with RoF would be promising a new generation wireless access or local area networks that would bring the cost-effective broadband infrastructure for access systems for APT countries.

This Report provides information on characteristics and requirements of optical and electrical components for millimeter-wave RoF systems. Components of high-speed electric-to-optical (E/O) and optical-to-electric (O/E) conversion devices designed for optical digital systems would be useful for millimeter-wave RoF systems. Because requirements of components for RoF systems are quite different from those for digital systems not only the system characterization techniques but also E/O and O/E components specialized for RoF are very important for designing RoF systems. Annex 1 summarizes information on characterization of E/O and O/E conversion devices for information.

2. Scope

This report provides information on optical and electric components for RoF systems in millimeter-wave bands and guidance for each administration to design cost-effective and broadband infrastructure using millimeter-wave RoF systems. The outline and applications of the RoF systems are also described to define requirements of the components.

3. Abbreviations and acronyms

10GbE		10 Gigabit Ethernet

AWG		Arrayed waveguide

BER		Bit-error-rate

BS		base station

CA		Cassegrain antenna

CU		Central unit

DUT		Device under test

EA		Electro-absorption

EO		Electro-optic

E/O		Electric-to-optical

FOD		Foreign object debris

GOA		Gaussian optic lens antenna

LN		Lithium niobate

MZM		Mach-Zehnder modulator

OC-192	Optical carrier 192

A SONET rate of 9953.28 Mbit/s (payload: 9621.504 Mbit/s; overhead: 331.776 Mbit/s) over optic fiber lines. SONET stands for Synchronous Optical NETwork.

O/E		Optical-to-electric

PD		Photodetector

RAU		Radio access unit

RoF		Radio over fiber

SFN		Single frequency network

UTC-PD	Uni-travelling-carrier photodetector

VNA		Vector network analyzer

4. References

- M. Sauer, A. Kobyakov, and J. George, Radio Over Fiber for Picocellular Network Architectures, J. Lightwave Technol. 25, 3301-3320 (2007)

- Report ITU-R F.2107-1 Characteristics and applications of fixed wireless systems operating in the 57 to 130GHz bands

- T. Kawanishi, S. Sakamoto and M. Izutsu, High-Speed Control of Lightwave Amplitude, Phase, and Frequency by Use of Electrooptic Effect, IEEE Journal of Selected Topics in Quantum Electronics, 13, 79-91 (2007)

- A. Kanno, T. Sakamoto, A.Chiba, T. Kawanishi, K. Higuma, M. Sudou and J. Ichikawa,120-Gb/s NRZ-DQPSK signal generation by a thin-lithium-niobate-substrate modulator, IEICE Electronics Express, 7, 817-822 (2010)

- H. Ito, T. Furuta, S. Kodama and T. Ishibashi, InP/InGaAs Uni-Traveling-Carrier Photodiode with 310 GHz Bandwidth, Electron. Lett., 36, 1809-1810, 2000.

- T. Umezawa, K. Akahane, A. Kanno and T. Kawanishi, Investigation of a 1.5-um-wavelength InAs-quantum-dot absorption layer for high-speed photodetector, Applied Physics Express, 7, 032201 (2014)

- T. Kawanishi, Pure two-tone optical signal generation by using precise and high-speed modulation, Mo.C1.4, the 13th International Conference on Transparent Optical Networks (ICTON 2011), Stockholm, Sweden, 26 – 30 June 2011

- Y. Awaji, T. Kawanishi, K. Inagaki, N. Wada, Impact of gain transience of EDFA on RoF/WDM and mitigation, WA2-5, The 5th Asia-Pacific Microwave Photonics Conference (APMP), Hong Kong, China, April 26 – 28, 2010.

- T. Kuri, K. Ikeda, H. Toda, K. Kitayama, and Y. Takahashi, A compact remote antenna base station for microwave/millimeter-wave dual-band radio-on-fiber systems, MF3,The 17th Annual Meeting of the IEEE Lasers and Electro-Optics Society (LEOS 2004), Puerto Rico, USA,  November 7-11, 2004.

- A. Hirata, T. Kosugi, H. Takahashi, R. Yamaguchi, F. Nakajima, T. Furuta, H. Ito, H. Sugahara, Y. Sato, and T. Nagatsuma, 120-GHz-band. Millimeter-wave Photonic Wireless Link for 10-Gb/s Data. Transmission, IEEE Trans. Microwave. Theory Tech., 54, 1937-1944 (2006)

- S. Oikawa, T. Kawanishi and M. Izutsu, Measurement of chirp parameters and halfwave voltages of Mach-Zehnder-type optical modulators by using a small signal operation, IEEE Photon. Tech. Lett., 15, 682-684 (2003)

- K. Inagaki, T. Kawanishi, and M. Izutsu, Optoelectronic frequency response measurement of photodiodes by using high-extinction ratio optical modulator, IEICE Electronic Express, February 2012

- T. Tangmala, S. Potha, U. Mankong, K. Inagaki and T. Kawanishi, Optoelectronic Frequency Response Measurement Using Standard Mach Zehnder Modulator, the 11th International Conference on Optical Communications and Networks (ICOCN2012), Pattaya, Thailand, 28 -30 November 2012

- K. Inagaki, T. Kawanishi, M. Ameya, S. Kurokawa and Y. Oikawa, Uncertainty Budget of PD’s Frequency Response Measurement Using Heterodyne Technique, WPF-31, 2013 Conference on Lasers and Electro-Optics Pacific Rim (CLEO-PR), Kyoto, Japan, June 30 – July 4 2013

- Optical Remote Antenna System, Fiber Optics Product Catalogue, SEIKOH GIKEN Co. Ltd., http://www.seikoh-giken.co.jp/en/products/pdf/fop_catalogue2011.pdf

- RoF-Link, Stack Electronics Co. Ltd., http://www.stack-elec.co.jp/english/index.html

- http://www.jmcia.or.jp/main/002gaiyou.htm (in Japanese only)

- F. Futatsumori, et al, “Evaluation of radio-over-fiber characteristics at airport for optically-connected runway surface foreign object debris detection millimeter-wave radar”, The 39th International Conference on Infrared, Millimeter, and Terahertz Waves (IRMMW-THz 2014), Sept. 2014.

5. Outline of RoF systems

Figure 1 shows a schematic of the RoF system consisting of components for E/O or O/E conversion and of an optical fiber for transmission. The loss in the fiber is very small. For example, the transmission loss of a standard single mode fiber is 0.2dB/km for lightwave with wavelength of 1550nm. In conventional RF signal distribution systems, transmission loss in metallic cables or waveguides would be particularly large for high-frequency signals. The loss largely depends on the signal frequency, while that of RoF systems is almost constant. Thus, the transmission loss would be negligible in RoF systems for short distance transmission (shorter than 10km). The total gain or loss of the systems is largely depends on the efficiency of E/O and O/E conversion. The intensity and quality of millimeter wave radiated from antennas at BSs directly depend on the performance of the components in the systems, while the millimeter wave depends only on the performance of BSs in millimeter wave systems with digital transmission based backbones.

Figure 1 Basic concept of RoF system.

6. Key optical devices for millimeter-wave RoF systems

E/O and O/E conversion play important roles also in digital data transmission systems. By using broadband electric and optical components, very high-speed optical links whose modulation speed is larger than 50 Gbaud have been demonstrated, recently. The components and sub systems designed for high-speed digital modulation and demodulation can be used for E/O and O/E conversion of millimeter-wave signals in RoF systems. Total performances of RoF systems rely on other components such as amplifiers, as well as on E/O or O/E devices of which overviews are in the following two sub-sections. As an example, impact of optical amplifier transient on link performance is briefly described in section 6.3.

6.1 E/O conversion devices

Direct modulation of a laser diode provides a simple E/O conversion setup. However, due to limitation of intrinsic response of the laser, it is difficult to modulate light by millimeter-wave. In addition, direct modulation generates undesired parasitic phase modulation which would degrade intensity modulated signal waveform by dispersion effect in the optical fiber. External modulation can provide high-speed E/O conversion. There are two types of external modulators: electro-optic effect (EO) modulator and electro-absorption effect (EA) modulator. EO modulators using lithium niobate (LN) which can provide high-speed and precise lightwave control would be useful for characterization of components for RoF systems, as well as for RoF systems themselves [3]. EO effect shifts the optical phase, so that optical amplitude or intensity can be controlled by using optical interferometer structures. A most popular one is a Mach-Zehnder modulator (MZM) consisting of a Mach-Zehnder interferometer which can provide wideband operation, as shown in figure 2 [4].

Figure 2 Frequency responses of high-speed MZMs of various device thicknesses (t). Solid lines show S21 of electrodes in MZMs. Diamonds are half-wave voltages (V) for push-pull operation with dual electrode MZMs [4].

6.2 O/E conversion devices

For O/E conversion, high-speed PDs have been developed to achieve wideband response (see figure 3). Uni-travelling carrier PDs (UTC-PDs) can provide high-speed and high-power operation, so that they would be useful for millimeter wave RoF systems [5]. The O/E conversion efficiency can be enhanced by avalanche effect in photo diodes, which would be utilized by quantum dots with simple device structures [6]. Required bandwidth for the components would be 0.7-0.8xM-GHz for M-Gbaud binary digital modulation format transmission systems, so that technologies for 100-Gbaud can be applied for millimeter-wave RoF systems where the frequency would be 30-70GHz. However, the efficiency of the E/O and O/E conversion is directly connected to the intensity of millimeter-wave generated by the O/E conversion, while small deformation of the efficiency in frequency domain does not affect on the performance of the digital transmission. Components for digital transmission are designed for broadband operation from a few kHz to millimeter-wave frequency. On the other hand, RoF generates signals in a particular frequency region, so that O/E or E/O devices using resonance can be used where the efficiency is enhanced at a desired frequency. Thus, precise estimation of the O/E and E/O conversion efficiency in particular frequency ranges would be very important to assemble RoF systems (see annex 1).

Figure 3 PD frequency response measured by optical two-tone stimulus method [7].

6.3 Impact of optical amplifier transient on link performances

Large scale RoF systems would have optical amplifiers to compensate optical loss at power splitters, optical filters, etc. Dynamic gain transition in the amplifiers would have negative impact on the performance of RoF. When the amplifiers are shared with digital systems, bursty digital signals cause optical surge or distortion of RoF signal envelop. The transience is driven by cross gain saturation. Thus, bursty digital channels will affect the effective gain of channels assigned for RoF, and cause fluctuation of millimeter wave power radiated from antennas. Transient suppressed optical amplifiers using large core are proposed to control radiation power precisely [8].

7. Possible applications of millimeter-wave RoF systems

This section provides examples of millimeter-wave RoF systems. The microwave RoF systems which were implemented for mobile and broadcasting services are attached in Annexes 2 and 3.

7.1 Application to mobile service: Microwave/Millimeter-Wave Dual-Band RoF System

A dual-band RoF system, which simultaneously transmits millimeter-wave signals for high-speed data as well as conventional microwave-band signals, has been proposed [9]. Figure 4 shows a prototype BS for the dual band system. A high-speed PD with the 3-dB bandwidth of 60 GHz can generate millimeter-wave and microwave signals simultaneously from a RoF signal. An EA modulator converts millimeter-wave and microwave signals into a lightwave signal. The size was 220 mm (width) x 150 mm (height) x 70 mm (depth) with the exception of projecting parts of microwave-band antennas. Frequency bands of the millimeter-wave downlink and uplink were designed to be in 59 to 60 GHz and 61 to 62 GHz, respectively. The microwave link band was designed to be in 2412 to 2484 MHz. The transmission distance from a central unit (CU) to the BS was 2-km. The bitrate of the millimeter-wave link was 156-Mb/s.

Figure 4 Dual-band BS prototype: (a) photograph and (b) configuration [9] .

7.2 Application to fixed service: 120-GHz band high-speed wireless system based on millimeter-wave RoF technology

Recently, 10Gb/s 120GHz-band wireless transmission was demonstrated by using millimeter-wave RoF technology [10]. As shown in figure 5, signal transmitter consists of optical millimeter-wave  signal generator, optical modulator and high-speed PD. Lightwave signal modulated by a millimeter wave signal of 125GHz is generated at the optical  millimeter-wave signal generator, by using an MZM for sideband generation and an arrayed waveguide (AWG) for filtering. Another optical intensity modulator modulates the lightwave signal with 10-Gb/s data. Finally, the lightwave which carries a millimeter-wave signal intensity modulated by 10-Gb/s data is applied to the high-speed PD through an optical fiber. A UTC-PD was used to achieve high-speed response and high-power output. The transmission was demonstrated with 9.953 Gb/s (OC-192) and 10.3125 Gb/s (10GbE). Figure 6shows the experimental 120GHz radio station which has a Cassegrain antenna (CA). A Gaussian optic lens antenna (GOA) was also tested. Table 1 shows the specification of the radio station. BER of 10-12 was achieved with a received power of -32.5dBm for 9.953 Gb/s, and -30.2dBm for 10.3125 Gb/s.  The wireless transmission distance was 200m for 9.953 Gb/s and 300m for 10.3125 Gb/s.

Figure 5 Schematic of 10-Gb/s wireless link by RoF technology [10].

Figure 6 Experimental 120GHz radio station [11].

7.3 Application of radiolocation service: Foreign object debris detection system

After the fatal accident of the Concorde at Paris Charles-de-Gaulle airport in 2000, the detection of the foreign object debris (FOD) on the airport surface is the most important safety issues for the airport management. Even if they are small in volume and light in weight, the FOD must be removed to ensure safety and security. The FOD detection system utilizing W-band millimeter-wave radar technology provides high-sensitivity and high-range resolution performance. The radar system consists of a central station (a radar signal generation unit and a signal processing unit) and some radar front-ends (antenna units), which cover small FOD detection areas, as shown in Figure 7. The radar transmission signal is generated at the central station. The complicated radar modulation signal is transmitted to the radar antenna units through optical fibers using the Radio-over-fiber (RoF) technology. Additionally the digitized receiving signals obtained at the antenna units are also transmitted to the central station by using the optical fiber.

Figure 7 Blockdiagram of FOD radar system using millimeter-wave RoF technologies.

Optical frequency quadrupling by high-extinction ratio optical modulator is used for high-speed and broadband FM-CW signal generation. High-linear analog photoreceiver with InGaAs photodiode is integrated with low-noise amplifier MMIC at 30 GHz. Figure 8 shows RF signal distribution network using a coupler-integrated arrayed waveguide grating. WDM optical signals are demultiplexed with high unwanted component suppression by interleaving arrayed waveguide grating. The suppression ratio over 35 dB is achieved between separated signals. Each WDM signals are transmitted to each radar antenna through optical fiber cables.

Figure 8 RF distribution network using WDM technologies.

Table 1 Specification of the experimental radio station [10].

8. Summary

This report considers characteristics and requirements of optical and electric components for millimeter-wave RoF systems. In millimeter-wave region, signal transmission by metal cables is not feasible because the loss in the cables would be significantly large. On the other hand, the loss in optical fibers does not depend on the signal frequency. Thus, we can achieve long distance transmission of millimeter-wave by using RoF. Lasers, optical modulators and PDs play important role in RoF systems. Optical components designed for high-speed digital optical communication systems could be used for analog systems including RoF systems. However, the requirements would be different from digital systems, so that characterization and measurement of components would be very important to assemble RoF systems.

Annex 1: Characterization of E/O and O/E conversion devices for millimeter-wave RoF systems

As described in the previous sections, precise characterization of optical devices plays important roles in millimeter-wave RoF systems. Vector network analyzers (VNAs) are often used to estimate frequency response of high-speed electric components, such as couplers, transmission lines, amplifiers, etc., as shown in figure 7.

Figure 9 Frequency response measurement for electric components using a VNA.

A VNA with optical interfaces consisting of E/O and O/E converters can be used for estimation of O/E or E/O conversion devices, where optical devices in the optical interfaces should be precisely calibrated. A calibrated O/E converter is needed to measure the efficiency of E/O conversion, as shown in Figure 8. Similarly, a calibrated E/O converter should be used for estimation of O/E devices (see Figure 9).

Figure 10 Frequency response measurement setup for O/E conversion.

Figure 11 Frequency response measurement setup for E/O conversion.

On the other hand, recently, measurement techniques for characteristics of MZMs based on optical spectrum analysis have been proposed, where relative optical intensities of sideband components are measured by an optical spectrum analyzer without using a high-speed PD [11]. These techniques are specialized for MZMs, but MZMs can provide high-speed and precise lightwave control. Thus, a reference signal for measurement of performance of various types of E/O and O/E conversion devices can be generated by an MZM. A pure two-tone optical signal can be generated by using high extinction ratio optical modulation [3]. The two-tone optical signal can be used for a reference signal to calibrate O/E conversion devices.

The following steps provide calibration of a PD and an optical modulator.

1) To measure an MZM by using optical spectrum analysis.

2) To generate a two-tone optical signal by using the calibrated MZM.

3) To measure frequency response of a PD by feeding the two-tone signal.

4) Any optical modulators can be calibrated by using the calibrated PD.

Finally, a pair of optical interfaces with E/O and O/E conversion can be calibrated. Thus, a measurement setup for any E/O or O/E conversion devices can be constructed, where the VNA is connected to the calibrated interfaces. In this procedure, the measurement technique for MZMs based on optical spectrum analysis, and the frequency response measurement using an optical two-tone signal for high-speed PDs play important role for low-cost and precise measurement of performance of optical components designed for millimeter-wave RoF systems.

In the above O/E characterization technique, two-tone optical signal is used to stimulate O/E response at the difference frequency. The two-tone is generated by an MZM with the controlled bias at its minimum transmission point to suppress the optical carrier. RF signal is also supplied to the MZM so that, when the carrier is suppressed, two-tone optical sideband signal separated by twice of the RF frequency is obtained. The quality of two-tone signal is evaluated by output spurious suppression ratio. For example, 50 dB is obtained by a high extinction ratio MZM and 30 dB a commercial MZM [12, 13]. This characterization technique is performed by a set up consisting of optical power meter and RF power sensor, as shown in Fig. 10. The frequency response of an O/E device is calculated by the ratio of the square root of converted RF power and the optical power.  Error sources are caused by power difference, spurious tones, third-order intermodulation harmonics, optical power measurement uncertainty and RF power measurement uncertainty. Such errors should be minimized to reduce the uncertainty in the results. The frequency responses of several PDs are shown in Fig. 11. Four PDs of different bandwidths are characterized and the results correspond approximately to their bandwidth specifications. The O/E response using this technique is derived from the fundamental principle and the setup consisting of conventional instruments. The measurement uncertainty of this method can be estimated from those of the RF power sensor and optical power meter. As shown in Figure 12, the expected uncertainty is 0.34 dB at 10 GHz [14].

Figure 12 Two-tone O/E characterization technique using MZM

Figure 13 Frequency responses of PD with bandwidths 15, 18, 20 and 50 GHz

Figure 14 Uncertainty of frequency response measurement [14]

For frequency characterization of E/O conversion devices, it can be performed using an O/E conversion device which has been pre-calibrated by the two-tone method described above. The E/O characterization system is shown in Figure 13. The RF signal modulates the optical signal through an E/O device under test (DUT) which is a directly modulated laser or an external modulator. The modulated optical signal is then converted to RF signal by the pre-calibrated O/E device (photodiode), which has wideband frequency responses. Hence the frequency response of the DUT can be obtained by subtracting the total system response from the O/E response, providing that impedances of RF and optical connections are matched.  Figure 14 shows frequency response of a directly modulated laser of 2 GHz bandwidth using this method. Thus it is a useful method for O/E frequency characterization up to millimeter wave frequencies.

Figure 15 E/O Characterization using a calibrated O/E device, for example, by a two-tone method

Figure 16 Frequency response of a directly modulated laser with bandwidth 2 GHz

Annex 2: Applications of microwave RoF to terrestrial services

This annex provides examples of microwave RoF systems for terrestrial services. A remote antenna system with RoF technologies has been developed for relaying radio waves in single frequency networks (SFNs), as shown in figure 13 [15]. This system has been developed for digital terrestrial TV broadcast radio waves. For SFNs, the receiving antenna should be separated from the antenna which transmits the radio wave received at the received antenna. A receiving unit (RU) has an antenna for receiving the radio wave from a parent station, and an optical modulator for generation of a RoF signal. A transmission unit (TU) transmits an unmodulated lightwave for modulation at the RU, and a high-power lightwave as a power source of the RU. The RoF signal transmitted from the RU is converted into an electric signal by O/E conversion at the TU. The distance between the RU and TU can be larger than 1km. In addition, lightning damage can be suppressed because the RU is electrically isolated from the TU. Figure 14 shows a compact RoF analog link system which can transmit signals from 60MHz to 3GHz, where the RF output power is equalized to be 0 dBm. The system consists of an E/O and O/E modules. The size of modules is 110 (135 include RF Connector) (W) × 46.5 (D) × 27(H) mm [16]. Optical and RF connectors, amplifiers and other components are installed in the small modules. Each module has an N-connector, so that it can be easily applicable to extension of RF signal transmission in conventional systems. Thus, it would be useful for filling gaps of broadcasting or mobile communication services in skyscrapers, underground shopping centers, subways, etc. Figure 15 shows a schematic of a gap filling system for mobile services in tunnels, where the system can be shared by plural service operators [17]. In subways in Tokyo and Osaka, Japan, 3G mobile services using this system will be started by March, 2012.

Figure 17 Remote antenna system for relay of digital terrestrial TV radio wave [15].

Figure 18 Compact RoF link system [16].

Switching facilities

Base stations

Optical transmitter

Optical splitter

Optical fiber

Antennas

Repeaters

Operator A

Operator B

Operator C

Figure 19 RoF system for filling gaps in tunnels [17].

Annex 3: RoF system of microwave Electronic News Gathering (ENG) for broadcasting

In oredr to operate microwave ENG in Tokyo metroplitan area where a line-of-sight link is difficult to realize in a large area, its receiving antenna should be equiped in very high towers or buildings. The maintenace of microwave ENG systems operation should also be cost-effective. One Japanese braodcasting company recently introduced microwave ENG systems whose receiving antennas are equiped in the Tokyo Skytree whose hight is approximately 630 meters. Figure 13 shows the system illustration of microwave ENG using RoF system. The RoF connects the microwave recievers installed on the Tokyo Skytree and the control station in the TV broadcasting headquater.

Figure 20 System illustration of microwave ENG using RoF.

To make 360-degree coverage area from Tokyo Skytree, 4 receiving antennas whose coverage area is 90 degree are used to receive the ENG signals, as shown in Figure 14. Four laser diodes whose wavelengthes are slightly different are intensity modulated by the received signal, respectively. Four intensity modulated optical carriers are multipled by optical multiplexer and transmitted to TV broadcasting station. The typical length of fiber cable between Tokyo Skytree and TV broadcasting station is about 15 km and the loss of optical signal is about 15 dB. IN order to compensate the the optical loss, the link gain of 30 dB is designed  using microwave amplifiers. This system is now operated by TV broadcaster for ENG in Tokyo metropolitan area.

Figure 21 Blockdiagram of microwave RoF systems using wavelength division multiplexing.