APT REPORT

On

“Guidelines on technical conditions for the use of mobile phones onboard vessels”

No. APT/AWG/REP-22

Edition: September 2011

Adopted by

The 11th APT Wireless Group Meeting

14 – 17 September 2011

Chiang Mai, Thailand

|  | ASIA-PACIFIC TELECOMMUNITY |  |  |  |
| --- | --- | --- | --- | --- |
|  | The APT Wireless Group |  |  |  |

Source: AWG-11/OUT-07(Rev.1)

APT REPORT ON

“Guidelines on technical conditions for the use of mobile phones onboard vessels”

Note: This guidelines document does not presume that an administration has authorized the mobile phones onboard vessel service. The guidelines presented here are provided as supporting information for national administrations when considering the technical conditions of the onboard cellular service.

This document defines the guidelines on technical conditions for the use of mobile phones onboard vessels. The technical guidelines have been defined by analyzing the frequencies and technologies present in the Asia Pacific region and proposing the maximum permitted power possible in a technology neutral manner.

The technical guidelines have been created from the following steps:

- Step One: Frequency bands and technologies covered in the Asia Pacific Region
Identification of all spectrum bands and technologies operating in the various countries/regions of the Asia Pacific Region. The analysis is based on publicly available information on the cellular frequencies used in the Asia Pacific region, and consequently may need to be updated.

- Step Two: Derivation of control bands, connectivity bands 
Identification of the bands required to control and the potential bands that could operate an onboard service, based on the spectrum band and technologies found in step 1. It is highlighted that connectivity can only occur if the cellular system has a pre defined roaming infrastructure, this reduces the possibility of offering connectivity service to only MAP based (e.g. GSM or UMTS) and IS 41 based (e.g.cdma2000) cellular networks.

- Step Three: The Guidelines on Technical Conditions for the Use of Mobile Phones Onboard Vessels 
Proposed AWG Guidelines On Technical Conditions For The Use Of Mobile Phones Onboard Vessel.

The following provides the technical guideline for national administrations to use when considering the technical conditions for the use of mobile phones onboard vessels framework.

1： Step One: Frequency bands and technologies covered in the Asia Pacific Region

The following table provides the analysis of technologies and frequencies currently deployed in the Asia Pacific region. The information is based on publicly available information. Where a country/region is missing or information is not present the authors would request to be updated.

| Technology | Frequency | Band |
| --- | --- | --- |

| Australia |  |  |
| --- | --- | --- |
| CDMA800 / UMTS (WCDMA) 800 | 825 – 845 / 870 – 890 | A |
| GSM900 | 890 – 915 / 935 – 960 | B |
| GSM1800 | 1710 – 1785 / 1805 -1880 | C |
| UMTS(WCDMA) | 1920 – 1980 / 2110 -2170 | E |

| Bangladesh |  |  |
| --- | --- | --- |
| CDMA800 | 824 – 849 / 869 – 894 | A |
| GSM900 | 890 – 915 / 935 – 960 | B |
| GSM1800 | 1710 – 1785 / 1805 -1880 | C |
| UMTS (WCDMA) | 1920 – 1980 / 2110 -2170 | E |

| Bhutan |  |  |
| --- | --- | --- |
| GSM900 | 890 – 915 / 935 – 960 | B |

| Brunei Darussalam |  |  |
| --- | --- | --- |
| GSM900 | 890 – 915 / 935 – 960 | B |
| UMTS(WCDMA) | 1920 – 1980 / 2110 -2170 | E |

| Cambodia |  |  |
| --- | --- | --- |
| CDMA450 | 450 – 460 / 460 - 470– | F |
| GSM900 | 890 – 915 / 935 – 960 | B |
| GSM1800 | 1710 – 1785 / 1805 -1880 | C |

| China (People’s Republic of China) |  |  |
| --- | --- | --- |
| CDMA800 | 825 – 835 / 870 – 880 | A |
| GSM900 | 890 – 915 / 935 – 960 | B |
| GSM1800 | 1710 – 1755 / 1805 -1850 | C |
| CDMA2000 | 1920-1935/2110-2125 | E |
| UMTS(WCDMA) | 1940-1955/2130-2145 | E |
| TD-SCDMA | 1880 – 1900 TDD | M2 |
| TD-SCDMA | 2010 – 2025 TDD | N |
| TD-SCDMA | 2300 – 2400 TDD | P |

| Guam |  |  |
| --- | --- | --- |
| CDMA800 / TDMA800 | 824 – 849 / 869 – 894 | A |
| CDMA1900 / GSM1900 | 1850 – 1910 / 1930 – 1990 | D |

| India (Republic of) |  |  |
| --- | --- | --- |
| CDMA800 | 824 – 849 / 869 – 894 | A |
| GSM900 | 890 – 915 / 935 – 960 | B |
| GSM1800 | 1710 – 1785 / 1805 -1880 | C |

| Indonesia (Republic of) |  |  |
| --- | --- | --- |
| CDMA800 | 825 – 845 / 870 – 890 | A |
| GSM900 | 890 – 915 / 935 – 960 | B |
| GSM1800 | 1710 – 1785 / 1805 -1880 | C |
| PCS1900 | 1903.125-1910/1983.125-1990 | D |
| UMTS(WCDMA)2100 | 1920 – 1980 / 2110 -2170 | E |
| CDMA450 | 450-457.5 / 460-467.5 | F |

| Japan |  |  |
| --- | --- | --- |
| UMTS(WCDMA)2100/CDMA 2000 | 1920 – 1980 / 2110 -2170 | E |
| E-UTRA(LTE) | 1950 – 1960 / 2140 – 2150 | E2 |
| PDC800 | 940 – 948 / 810 – 818 930 – 940 / 875 – 885 | G |
| UMTS (WCDMA)800 / CDMA800 | 815 – 845 / 860 – 890 | I |
| CDMA800 | 898 – 901 / 843 – 846 915 – 925 / 860 – 870 | J |
| UMTS(WCDMA)1500/CDMA1500 | 1427.9 – 1437.9 / 1475.9 – 1485.9 | K |
| UMTS (WCDMA)1700 / CDMA1700 | 1754.9 – 1764.9 / 1849.9 – 1859.9 1769.9 – 1784.9 / 1864.9 – 1879.9 | L |
| PHS | 1884.5 - 1918.4 (TDD) | M |
| BWA systems (WiMAX,XGP) | 2545 – 2575 (TDD) 2595 – 2625 (TDD) | O |

| Korea (Democratic People’s Republic of) |  |  |
| --- | --- | --- |
| GSM900 | 890 – 915 / 935 – 960 | B |

| Korea (Republic of) |  |  |
| --- | --- | --- |
| CDMA800 | 824 – 849 / 869 – 894 | A |
| CDMA1800 | 1750 – 1780 / 1840 – 1870 | C |
| UMTS(WCDMA)2100 | 1920 – 1980 / 2110 -2170 | E |
| Mobile WiMAX (WiBro) | 2300 – 2400 | P2 |

| Laos |  |  |
| --- | --- | --- |
| GSM900 | 890 – 915 / 935 – 960 | B |
| GSM1800 | 1710 – 1785 / 1805 -1880 | C |

| Macau |  |  |
| --- | --- | --- |
| CDMA800 | 825-845/870-890 | A |
| GSM900 | 890 – 915 / 935 – 960 | B |
| GSM1800 | 1710 – 1785 / 1805 -1880 | C |
| UMTS(WCDMA)2100 | 1920 – 1980 / 2110 -2170 | E |

| Malaysia |  |  |
| --- | --- | --- |
| EGSM900 | 880 – 915/ 925 – 960 | B2 |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| UMTS(WCDMA)2100 | 1920 – 1980/ 2110 -2170 | E |
| BWA | 2300-2400 | P2 |

| Mongolia |  |  |
| --- | --- | --- |
| CDMA800 | 824 – 849 / 869 – 894 | A |
| GSM900 | 890 – 915/ 935 – 960 | B |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| CDMA450 | 450 - 460 / 460 – 470 | F |

| Myanmar (The Union of) |  |  |
| --- | --- | --- |
| CDMA800 / TDMA800 | 824 – 849 / 869 – 894 | A |
| GSM900 | 890 – 915/ 935 – 960 | B |

| New Zealand |  |  |
| --- | --- | --- |
| W-CDMA850/CDMA-2000 | 825 – 840 / 870 – 885 | A |
| GSM900/W-CDMA 900 | 890 – 915/ 935 – 960 | B |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| UMTS(WCDMA)2100 | 1920 – 1980/ 2110 -2170 | E |
| BWA | 2300-2400 | P2 |

| Northern Mariana Islands |  |  |
| --- | --- | --- |
| CDMA800 / GSM 800 | 824 – 849 / 869 – 894 | A |
| GSM1900 | 1850-1910/ 1930-1990 | D |

| Pakistan (Islamic Republic of) |  |  |
| --- | --- | --- |
| TDMA800 | 824 – 849 / 869 – 894 | A |
| GSM900 | 890 – 915/ 935 – 960 | B |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |

| Papua New Guinea |  |  |
| --- | --- | --- |
| EGSM900 | 880 – 915/ 925 – 960 | B2 |

| Philippines (Republic of the) |  |  |
| --- | --- | --- |
| 3G(WCDMA) | 870-880/825-835 | A |
| GSM900 | 890 – 915/ 935 – 960 | B |
| EGSM | 880-890/925-935 | B2 |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| CDMA 1900 | 1900 – 1910 / 1980 – 1990 | D |
| UMTS(WCDMA)2100 | 1920 – 1980/ 2110 -2170 | E |

| Singapore |  |  |
| --- | --- | --- |
| EGSM900 | 880 – 915/ 925 – 960 | B2 |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| UMTS(WCDMA)2100 | 1920 – 1980/ 2110 -2170 | E |

| Sri Lanka |  |  |
| --- | --- | --- |
| GSM900 | 890 – 915/ 935 – 960 | B |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |

| Taiwan |  |  |
| --- | --- | --- |
| CDMA800 | 824 – 849 / 869 – 894 | A |
| GSM900 | 890 – 915/ 935 – 960 | B |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| UMTS(WCDMA)2100 | 1920 – 1980/ 2110 -2170 | E |

| Thailand |  |  |
| --- | --- | --- |
| CDMA800 / UMTS 800 | 824 – 849 / 869 – 894 | A |
| GSM 900/ UMTS 900 | 897.5 – 915/ 942.5 - 960 | B3 |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| UMTS(WCDMA)2100 | 1920 – 1980/ 2110 -2170 | E |

| Tonga |  |  |
| --- | --- | --- |
| GSM900 | 890 – 915/ 935 – 960 | B |

| Vietnam (Socialist Republic of) |  |  |
| --- | --- | --- |
| CDMA800 | 829 – 845 / 874 – 890 (824 – 829 / 869 – 894 as of 2010) | A |
| GSM900 | 890 – 915/ 935 – 960 | B |
| GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| IMT 2000 | 1920 – 1980/ 2110 -2170 | E |
| CDMA 450 | 453.08 – 457.37 / 465.08 – 467.37 | F |

The information contained in the tables above merely reflects the current deployment of terrestrial cellular networks for each nation. The inclusion of a country/region within the tables above does not presume that an onboard cellular system has been authorized by that administration.

2： Step two: Derivation of control and connectivity bands used in Asia Pacific Region

- 2.1 Cellular frequency bands and technologies used in the Asia Pacific region

Based on the analysis in Step 1 above the following cellular bands and technologies have been identified in the Asia Pacific region.

| Technology | Frequency (UL/DL) | Band |
| --- | --- | --- |
| CDMA800 / TDMA 800/ GSM 800/ UMTS 800 | 824 – 849 / 869 – 894 | A |
| GSM900 | 890 – 915/ 935 - 960 | B |
| EGSM 900 | 880 – 915/ 925 - 960 | B2 |
| GSM 900/ UMTS 900 | 897.5 – 915/ 942.5 - 960 | B3 |
| CDMA 1800 / GSM1800 | 1710 – 1785/ 1805 -1880 | C |
| CDMA 1900 / GSM1900 | 1850-1910/ 1930-1990 | D |
| UMTS (WCDMA)/ CDMA 2000 | 1920 – 1980/ 2110 -2170 | E |
| E-UTRA(LTE) | 1950 – 1960 / 2140 – 2150 | E2 |
| CDMA450 | 450-460/ 460-470 | F |
| PDC 800 | 940 – 948 / 810 – 818 930 – 940 / 875 – 885 | G |
| UMTS (WCDMA) 800/ CDMA 800 | 815 - 845 / 860 - 890 | I |
| CDMA800 | 898 - 901 / 843 - 846 915 – 925 / 860 – 870 | J |
| UMTS (WCDMA)1500/CDMA1500 | 1427.9 - 1437.9 / 1475.9 - 1485.9 | K |
| UMTS(WCDMA)1700 | 1754.9 - 1764.9 / 1849.9 - 1859.9 1769.9 – 1784.9 / 1864.9 – 1879.9 | L |
| PHS | 1884.5 - 1918.4 (TDD) | M |
| TD-SCDMA | 1880 – 1900 (TDD) | M2 |
| TDD mobile communication systems | 2010 – 2025 (TDD) | N |
| BWA systems (WiMAX,XGP) | 2545 – 2575 (TDD) 2595 – 2625 (TDD) | O |
| TD-SCDMA | 2300 – 2400 (TDD) | P |
| Mobile WiMAX (WiBro) | 2300 – 2400 (TDD) | P2 |

The system will operate only when the vessel is outside the territorial waters of the country unless the country authorizes the use in the territorial waters.   The system will take all necessary steps to avoid interference to land based systems.  It is strongly recommended that bands above 1 GHz are used and the vessels coordinate their use with APT member states or have a mechanism to avoid using frequencies already in use by terrestrial systems in that area when they are within 30 kilo meters off shore.  At the same time the operation of an onboard system must not prevent mobile phones onboard the vessel from roaming onto a terrestrial cellular network when the vessel is inside the exclusive economic zone of the country. i.e. the onboard cellular system will not need to control cellular frequency bands being used by the terrestrial cellular network and activate any necessary mitigation mechanisms when entering the exclusive economic zone of the country.

Consideration may also be required for non cellular wireless technologies when deployed in the same frequency band. In particular where there is a mix of both cellular wireless and non cellular wireless terrestrial deployed systems in the same band.

- 2.2 Mapping of cellular frequency bands and technologies to countries/regions

Based on the analysis in step 1 the following bands and technologies of public mobile communications systems per country/region have been identified in the Asia Pacific region.

| Country / Region | Band Used | Country / Region | Band Used |
| --- | --- | --- | --- |
| Australia | A, B, C, E | Bangladesh | A, B, C |
| Bhutan | B | Brunei Darussalam | B, E |
| Cambodia | B, C, E, F | China (People’s Republic of) | A, B, C, E,M2, N ,P |
| Guam | A, D | India (Republic of) | A, B, C |
| Indonesia (Republic of) | A, B, C ,D,E, F | Japan | E, E2,G, I, J, K, L, M, O |
| Korea (Democratic People’s Republic of) | B | Korea (Republic of) | A, C, E, P2 |
| Laos | B, C | Macau | A, B, C, E |
| Mongolia | A, B | Malaysia | B2, C, E, P2 |
| New Zealand | A, B, C, E, P2 | Myanmar (The Union of) | A, B |
| Pakistan (Islamic Republic of) | A, B, C | Northern Mariana Islands | A, D |
| Philippines (Republic of the) | A,B, B2,C, D, E | Papua New Guinea | B2 |
| Sri Lanka | B, C | Singapore | B2, C, E |
| Thailand | A,B3, C,E | Taiwan | A, B, C, E |
| Vietnam (Socialist Republic of) | A, B, C, E, F | Tonga | B |

The information contained in the tables above reflects the current deployment of terrestrial cellular networks for each nation. The inclusion of a country/region within the tables above does not presume that the onboard cellular system has been authorized by that administration.

2.3 possible connectivity frequency bands and technologies for offering connectivity

Based on the condition that connectivity onboard can only be achieved using a mobile system which has an established roaming functionality (i.e. MAP (e.g. GSM and /or UMTS) or IS 41 (e.g. cdma2000) based networks) then the following technologies and bands can be used in the Asia Pacific region

| Technology | Frequency (UL/DL) | Band |
| --- | --- | --- |
| CDMA800 / GSM 800 | 824 – 849 / 869 – 894 | A |
| GSM900 | 890 – 915/ 935 – 960 | B |
| CDMA 1800 / GSM1800 | 1710 – 1785/ 1805 – 1880 | C |
| CDMA 1900 / GSM1900 | 1850 – 1910/ 1930 – 1990 | D |
| UMTS (WCDMA) | 1920 – 1980/ 2110 – 2170 | E |

3： Step Three: The Guidelines on Technical Conditions for the Use of Mobile Phones Onboard Vessels

| Nr. | Parameter | Description | Comments | Status |
| --- | --- | --- | --- | --- |
| 1 | Frequency band | Band A: 824 – 849 / 869 – 894 Band B: 890 – 915/ 935 – 960 Band B2: 880 – 915/ 925 – 960 Band B3: 897.5 – 915/ 942.5 – 960 Band C: 1710 – 1785/ 1805 -1880 Band D: 1850-1910/ 1930-1990 Band E: 1920 – 1980/ 2110 -2170 Band E2:1950 – 1960 / 2140 – 2150 Band F: 450-460/ 460-470 Band G: 893 - 898 / 838 - 843 940 - 948 / 810 – 818 925 - 940 / 870 - 885 Band I: 815-845 / 860 890 Band J: 887 – 889 / 832 - 834 898 - 901 / 843 – 846 915 – 925 / 860 - 870 Band K: 1427.9 - 1452.9 / 1475.9 - 1500.9 Band L: 1749.9 - 1784.9 / 1844.9 - 1879.9 Band M: 18845.-1919.6 (TDD) Band M2: 1880 – 1920 (TDD) Band N: 2 010 – 2 025 (TDD) Band O: 2 545 - 2 625 (TDD) Band P: 2300 – 2400 (TDD) Band P2: 2300 – 2390 (TDD) | Band B, C and D are currently in use for MCV. Other bands might be applicable in the future (marked dependent) | C |
| 2 | Radio service | Mobile Service |  | M |
| 3 | Application | On-board cellular passenger communication system | GSM/GPRS standard | C |
|  |  |  | CDMA 2000 standard |  |
| 4 | BTS Transmit power limit | The vessel-BS emissions measured anywhere external to the vessel (i.e. at ship perimeter or on its open deck areas) shall not exceed -80 dBm/200 kHz (assuming a 0 dBi measurement antenna gain) | Authorization of the use of the system will depend on the fact that the installed system conforms to the requirements given in order to ensure no harmful interference with terrestrial mobile networks. | M |
| 5 | DTX (Discontinuous transmission) | GSM standard 3GPP TS 148.008 | The use of the DTX function provides a significant reduction of the interference potential from the v-BS | C |
| 6 | Timing advance | GSM standard 3GPP TS 144.018 | Timing advance value of v-BS might be set to minimum to avoid unwanted attach to the v-BS from a l-MS | C |
| 7 | ACCMIN | 3GPP TS 144.018 | AccMin can be used to control the receiver sensitivity and the disconnection threshold for a v-MS connected to the v-BS. | C |
| 8 | Additional Requirements to BTS | The vessel base station shall control the transmit power of all mobile terminals controlled by the onboard system. The maximum controlled nominal power value for mobiles phones shall be no greater than -53 dBm/ Hz inside the territorial border of a country/region. | For GSM mobiles this is equivalent to a maximum permitted nominal power limit of 0 dBm/ 200KHz For cdma based mobiles consideration will also be required on the number of mobiles transmitting simultaneously For CMDA 2000 mobiles this limit is equivalent to a maximum permitted nominal power limit of 0 dBm/1.25 MHz (assuming 8 dB multiple interference factor in the vessel) | M |

Table 1 Minimum requirements set description

C = Conditional requirement depending on territorial sea of the country/region the vessel is traversing through

M= Mandatory requirement